| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0047 |
| --- | --- |

| Title: | The coded region completion SEI message in multi-layer context |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

JCTVC-S0148 proposed the coded region completion SEI message that enables the detection of an end of a coded picture prior to receiving any NAL unit of the next picture and hence reportedly reduces delay. However, when decoding multi-layer bitstreams, the design of JCTVC-S1048 does not always enable the detection of the end of the access unit before the first NAL unit of the next access unit. Thus, it is asserted that the solution in JCTVC-S0148 is not able to reduce delay for decoders that operate on access unit basis, which is the issue addressed in this contribution. It is proposed to take advantage of the layers not present SEI message to conclude on which layers pictures may be present within an access unit. Consequently, it is asserted that no additional syntax is needed to enable concluding the end of an access unit. An informative note is proposed to be added into the semantics of the coded region completion SEI message to clarify how the coded region completion and layers not present SEI messages can be considered jointly to conclude an end of access unit for an output layer set of interest.

# Introduction

## Background: JCTVC-S0148

JCTVC-S0148 proposed the coded region completion SEI message that includes (in the syntax element next_segment_address) the slice segment address of the next slice header (when present) for decoding latency minimization. It was asserted in JCTVC-S0148 that it would be beneficial, especially in ultra-low latency use cases, to have an earlier indication of when the coded picture is complete, and particularly to have such an indication within the same access unit as that coded picture itself. Some decoders may be designed to wait for reception of the complete decoded picture before performing the parsing and decoding processes for the picture's VCL NAL units. Requiring such decoders to wait until the next access unit begins is asserted to sometimes add significant delay.

The following was noted in the JCTVC-S meeting notes on JCTVC-S0148:

It was questioned what the implications would be for scalability, or other multi-layer context. The proposal could apply to a single layer. It would allow detection of the end of picture, but not the end of the access unit. While we have an access unit delimeter, that is carried in the next access unit, not the current access unit.

Some systems environments have an indication for end of picture, such as the marker bit in RTP.

There was support for the proposal. Interest was expressed in fully considering the multi-layer context. There is an expectation to adopt at the next meeting (Feb, Geneva) to allow more time for study.

## Problem statement

It is assumed that generally four types of decoders or decoder interfaces exist for multi-layer HEVC decoding:

- Decoders that operate on NAL unit basis, i.e. input and decode one NAL unit at a time. In other words, these decoders are able to operate on slice segment basis.

- Decoder that operate on slice basis, i.e. input and decode one slice at a time. In other words, these decoders require all slice segments of a slice to be input at a time.

To support both type 1 and 2 decoders flexibly with the coded region completion SEI message, it was suggested to "add a syntax element (flag) to indicate whether or not a dependent slice segment follows" (as reported in the JCTVC-S meeting notes) in the syntax of the coded region completion SEI message.

- Decoders that operate on picture unit basis, i.e. input and decode one picture unit at a time. The SEI message proposed in JCTVC-S0148 is believed to be sufficient for such decoders to avoid the latency for concluding the start of the next picture unit from its first VCL NAL unit.

- Decoders that operate on access unit basis, i.e. input and decode one access unit at a time. JCTVC-S0148 did not enable the detection of the end of the access unit of a multi-layer bitstream before the first NAL unit of the next access unit, which is the problem addressed in this contribution.

In the multi-layer H.265/HEVC extensions, such as SHVC and MV-HEVC, it is not required to include a picture unit in each layer consistently in each access unit. In other words, there may be a picture unit at layer A in one access unit but in another access unit no picture unit at layer A may be present. It is therefore not possible to conclude from the layer identifier values of a picture unit whether it is the last picture unit of an access unit.

A multi-layer bitstream may be subject to layer extraction in the sender and/or in one or more gateways. An indication of an end of an access unit should be resilient to layer extractions so that decoders can conclude an end of access unit reliably even if the bitstream has been subject to layer extraction. Particularly, if the highest layer(s) of the bitstream are extracted out from the bitstream, decoders should still have means to conclude an end of an access unit. This implies that an end-of-AU indication at the highest layer, e.g. as a specific NAL unit or in an SEI message, is not a sufficient solution for the problem, as that indication would be extracted out from the bitstream together with other data on that layer.

Layer extraction may additionally or alternatively take place as a part of receiving a bitstream. For example, a receiver may determine that decoding and playback of a subset of the layers is desirable e.g. due to insufficient decoding capabilities to decode all layers in real-time. Layers may be sent through separate logical channels, such as different elementary streams in an MPEG-2 transport stream. A receiver may receive and/or make use of a subset of layers only, e.g. received in a corresponding set of logical channels. A receiver should be able to conclude the end of an access unit from the data it has received and/or is making use of.

It is desirable that the NAL unit order is such that picture units appear in increasing order of layer identifier value within an access unit. This enables sending picture units of different layers in different logical channels, while the re-aggregation of an access unit can simply be done by concatenating picture units in increasing layer identifier order. In other words, it is desirable to avoid NAL unit level processing in re-aggregating an access unit. Furthermore, it is not desirable to rely on sending an indication of the end of the access unit as part of the highest layer and/or the highest logical channel, because the highest layer and/or the highest logical channel might not be received or made use of by the receiver.

## Background: layers not present SEI message

The semantics of layers not present SEI message are specified as follows in clause F.14.3.3:

The layers not present SEI message provides a mechanism for signalling that VCL NAL units of particular layers indicated by the VPS are not present in a particular set of access units.

The target access units are defined as the set of access units starting from the access unit containing the layers not present SEI message up to but not including the next access unit, in decoding order, that contains a layers not present change SEI message or the end of the CVS, whichever is earlier in decoding order.

When present, the layers not present SEI message applies to the target access units.

A layers not present SEI message shall not be included in an SEI NAL unit with TemporalId greater than 0.

lnp_sei_active_vps_id identifies the active VPS of the CVS containing the layers not present SEI message. The value of lnp_sei_active_vps_id shall be equal to the value of vps_video_parameter_set_id of the active VPS for the VCL NAL units of the access unit containing the SEI message.

layer_not_present_flag[ i ] equal to 1 indicates that there are no VCL NAL units with nuh_layer_id equal to layer_id_in_nuh[ i ] present in the target access units. layer_not_present_flag[ i ] equal to 0 indicates that there may or may not be VCL NAL units with nuh_layer_id equal to layer_id_in_nuh[ i ] present in the target access units.

When layer_not_present_flag[ i ] is equal to 1 and i is less than MaxLayersMinus1, layer_not_present_flag[ LayerIdxInVps[ IdPredictedLayer[ layer_id_in_nuh[ i ] ][ j ] ] ] shall be equal to 1 for all values of j in the range of 0 to NumPredictedLayers[ layer_id_in_nuh[ i ] ] − 1, inclusive.

# Proposal

It is proposed to take advantage of the layers not present SEI message to conclude on which layers pictures may be present within an access unit. Consequently, it is asserted that no additional syntax is needed to enable concluding the end of an access unit. Instead it is suggested that the coded region completion and layers not present SEI messages can be considered jointly to conclude an end of access unit for an output layer set of interest. The following informative note is proposed to be added into the semantics of the coded region completion SEI message to clarify this:

NOTE – The specifications given in clause F.7.4.2.4.4 for associating NAL units to access units facilitate determining of an end of an access unit when the first NAL unit of the next access unit is available. Some implementations may operate on access unit basis, e.g. when inputting NAL units to a decoder. The latency in such implementations is potentially reduced, when a bitstream includes multiple layer and an end of an access unit for the OLS with index trgtOlsIdx is concluded as follows in response to decoding a coded region completion SEI message with next_segment_address equal to 0:

–	The variable layerMayBePresent[ layerIdx ] is derived as follows:

–	If a layers not present SEI message is present in the current CVS and precedes the coded region completion SEI message in decoding order, layerMayBePresentFlag[ layerIdx ] is set equal to !layer_not_present_flag[ layerIdx ] of the previous layers not present SEI message, in decoding order, for each value of layerIdx from 0 to MaxLayersMinus1, inclusive.

–	Otherwise, layerMayBePresentFlag[ layerIdx ] is set equal to 1 for each value of layerIdx from 0 to MaxLayersMinus1, inclusive.

–	Let a set of nuh_layer_id values trgtLayerIdList of the necessary layers for an OLS with index trgtOlsIdx be equal to TargetDecLayerIdList derived using Equation F-56 by setting TargetOlsIdx equal to trgtOlsIdx.

–	Let numTrgtLayerIds be equal to the number of nuh_layer_id values in trgtLayerIdList.

–	Let currLayerId be equal to the nuh_layer_id value of the SEI NAL unit containing the coded region completion SEI message.

–	Let prevNecessaryLayerTrgtIdx be the index of the greatest value within trgtLayerIdList that is less than or equal to currLayerId.

–	When one of the following is true, there are no VCL NAL units of any necessary layer of the OLS with index trgtOlsIdx following, in decoding order, the coded region completion SEI message contained in a suffix SEI NAL unit within the current access unit:

–	currLayerId is greater than or equal to any value in trgtLayerIdList. [Ed. (MH): If coded region completion SEI message is allowed in a prefix SEI NAL unit and currLayerId is equal to the greatest value in trgtLayerIdList, the next picture, in decoding order, is the last picture of the OLS within this access unit. To be reflected in the text, when a decision whether coded region completion SEI message is allowed in prefix SEI NAL unit has been made.]

–	layerMayBePresentFlag[ LayerIdxInVps[ layerId ] ] is equal to 0 for all the values of layerId equal to trgtLayerIdList[ trgtIdx ] for each trgtIdx in the range of prevNecessaryLayerTrgtIdx + 1 to numTrgtLayerIds − 1, inclusive.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).