| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0048 |
| --- | --- |

| Title: | Non-CE1: On palette prediction for slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Gisquet, T. Poirier, G. Laroche, P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

The present contribution first reports issues found in the SCM3.0 encoder when various encoding features such as tiles and slices are enabled. It is asserted that when those features are enabled, the Palette prediction is interrupted or reset and this leads to a significant loss of the coding efficiency. This contribution thus proposes a modification of the Palette prediction mechanism to overcome this issue, where a PPS-level table is used to initialize the palette predictor. It is reported that the proposed normative change provides a BDR gain of 1.7% to 2.7% over SCM3.0 (when using slices of 1500B) for the screen content classes, using the All-Intra scenario.

# Issues and related bugfixes with IBC in the encoder

The encoder, basically, does not respect any constraint deriving from the use of tiles or slices. In the following, a block and the relevant position are said to be invalid if the block has pixels outside of the current non-dependent slice/tile.

## List of issues

Firstly, when defining a search range, either the 2D one (in TEncSearch::xSetIntraSearchRange) or the 1D one (defined and immediately put to use in TEncSearch::xIntraPatternSearch, the boundaries of both the current tile and the current slice are never checked.

Secondly, when filling the hash table used in the IBC hash search, whether an 8x8 block (for which the hash is generated) belongs to the current tile and the current slice is never checked. Indeed, in TEncSearch::xIntraBCHashTableUpdate, when updating the hash with the results of encoding the current CTB, the blocks used belong to an area that is actually offset by 8 horizontally and vertically compared to current CTB. This is required, as otherwise blocks would use pixels from the CTBs on the right or below, which are not yet encoded.

## Proposed fixes

The rectangular area defined by TEncSearch::xSetIntraSearchRange, as well as the 1D searches from TEncSearch::xIntraPatternSearch must account for both tiles and slices boundaries. Ideally, the hash table must be reset on a tile or a slice start.

However, the above may not guarantee that all block evaluations use valid pixels from the current slice or tile:

- Testing predictors or elements in the BV cache may lead to invalid positions;

- The shape of the boundaries of a slice may not match the rectangular 2D search area.

As a result, it is quite likely that the encoder still evaluates invalid positions, so it is useful to add such checks to TEncSearch::isValidIntraBCSearchArea. As a last validation, it is suggested to assert at the decoder that the decoded BVs are valid ones.

The above changes, as well as test scenarios for slices and tiles, are present in the patch that will be attached to the contribution at a later date.

## Consequences

It is asserted that the above changes, in a typical scenario using slices of 1500B, greatly impact the coding efficiency of SCM3.0. In particular, in that scenario, the Palette mode is often causing more losses when deactivated, compared to Intra Block Copy.

# Proposed normative changes to palette mode

Screen Content Coding offer important coding efficiency gains thanks in part to the dedicated modes such as the Palette or Intra Block Copy modes. Such gains mainly come from very high redundancy in Screen-Content images. When slices are used, the Palette prediction mechanism is broken and this significantly affects the Palette mode coding efficiency.

On every non-dependent slice start, the palette predictor must be reset. This brings up a slew of potential issues:

- The coding efficiency overall is reduced;

- Error resilience techniques such as Intra Refresh or Unequal Error Protection may become difficult to use.

To this end, it is proposed to allow initialising the palette prediction on a slice start.

## Basic idea

Instead of resetting the palette prediction state (which includes PredictorPaletteSize, PreviousPaletteSize and PredictorPaletteEntries) to all 0, values obtained from the PPS are instead used. This is similar to a global palette, which is what a lot of formats such as GIF or PNG use.

## Proposed PPS syntax

| pps_scc_extensions( ) { | Descriptor |
| --- | --- |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| palette_predictor_initializer_present_flag | u(1) |
| if( palette_predictor_initializer_present_flag ) { |  |
| luma_bit_depth_entry_minus8 | ue(v) |
| chroma_bit_depth_entry_minus8 | ue(v) |
| palette_predictor_initializer_size_minus1 | ue(v) |
| for( i = 0; i <= size_palette_predictor_initializer_minus1; i++ ) |  |
| for( comp = 0; comp < 3; comp++ ) |  |
| palette_predictor_initializer[ i ][ comp ] | u(v) |
| } |  |
| } |  |

7.4.3.3.3	Picture parameter set screen content coding extensions semantics

residual_adaptive_colour_transform_enabled_flag equal to 1 specifies that an adaptive colour transform may be applied to the residual in the decoding process. residual_adaptive_colour_trans-form_enabled_flag equal to 0 specifies that adaptive colour transform is not applied to the residual. When not present, the value of residual_adaptive_colour_transform_enabled_flag is inferred to be equal to 0.

palette_predictor_initializer_size_minus1 specifies the number of entries in the palette predictor initializer. The variable NumPalettePredictorEntries is derived as follows:

NumPalettePredictorEntries = palette_predictor_initializer_size_minus1 + 1

It is a requirement of bitstream conformance that the value of NumPalettePredictorEntries shall be less than or equal to the maximum predictor size of any picture that refers to this PPS.

luma_bit_depth_entry_minus8 specifies the sample bit depth of the luma component of the entries of the palette predictor initializer. The variable BitDepthEntryY is derived as follows:

BitDepthEntryY = 8 + luma_bit_depth_entry_minus8

It is a requirement of bitstream conformance that the value of BitDepthEntryY shall be equal to the bit depth of the luma components of any picture that refers to this PPS.

chroma_bit_depth_entry_minus8 specifies the sample bit depth of the chroma components of the entries of the palette predictor initializer. The variable BitDepthEntryC is derived as follows:

BitDepthEntryC = 8 + chroma_bit_depth_entry_minus8

It is a requirement of bitstream conformance that the value of BitDepthEntryC shall be equal to the bit depth of the chroma components of any picture that refers to this PPS.

palette_predictor_initializer contains palette entries. BitDepthEntryY specifies the number of bits of each of its luma entries. BitDepthEntryC specifies the number of bits of each of its chroma entries.

  - Proposed decoding process

In section 9.3.2.1, “General”, all instances stating:

  - The variables PredictorPaletteSize and PreviousPaletteSize are set to 0.

is modified to read:

  - The variable PredictorPaletteSize is set to minialettePredictorEntries, as defined by clause 7.4.3.3.3, and palette_max_predictor_size;

  - PredictorPaletteEntries is initialised using palette_predictor_initializer, as defined by clause 7.4.3.3.3, per the following:

- for( i = 0; i < PredictorPaletteSize; i++ )

- for ( comp = 0; comp < 3; comp++ )

- PredictorPaletteEntries[ i ][ comp ] = palette_predictor_initializer[ i ][ comp ]

  - Encoder algorithm

To generate the initializer, a pre-pass is performed on a subset of blocks to generate the palette predictor using the normal palette encoder module. However, as this is an analysis and not an encoding pass, speedups have been introduced in the encoder module.

# Results

The results were obtained on a cluster where runtimes are not trustworthy. Further updates were attached after the request to see results on a low-delay encoder.

## Intended use

The Common Test Conditions do not define slice constraints. In the results below, the common configuration is:

- An encoder respecting the slices boundaries is used as anchor;

- It is configured to produce slices of 1500 bytes;

- The test implements the proposals of section 2 on top of the above.

## Low-delay version

The current algorithm works on the current frame. It was requested that a low-delay version be provided. This version achieves it by storing the previous image in coding order and analyses it instead of the current frame.

## Common Test Conditions

# Conclusion

In a first aspect, this contribution provides some bugfixes when the slices and tiles features are enabled.

In a second aspect, this contribution shows the significant loss when slices of 1500 Bytes are enabled and provides a solution to improve the Palette prediction to recover part of the losses.

The results even show that, under the Common Test Conditions, the proposed method brings some gains.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).