| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0049r1 |
| --- | --- |

| Title: | On intra mode MPM derivation for SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Gisquet, G. Laroche, P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

The Screen Content Coding extension introduces new coding modes beside the classical Intra and Inter ones which are the Intra Block Copy and the Palette modes. This contribution proposes to set the Intra candidate prediction mode (also called most probable mode) to an angular mode when its neighbour CU is neither Intra nor Inter. It is asserted, when dealing with Screen Content video sequences, that this modification better takes into account the content of the Left and Top CU used for the Intra Mode prediction. It is reported that the BDR gains are from 0.2% to 0.3% on SCC classes, and without losses on other ones.

# New coding modes and intra mode coding

## Basic idea

When deriving the MPM list, the neighbour CUs are checked:

Top CU

Left CU

Current CU

When a neighbour CU is not intra, i.e. inter, its corresponding intra prediction mode is set to DC. This is very well suited for natural content, but Screen Content has strong directional features, e.g. due to GUI elements. In such case, the DC mode is an ill-suited choice.

However, the case of mixed mode makes obvious that the candidate modes cannot be forced to take an angular value. Instead, the fact that the neighbour CU is coded using Intra Block Copy or Palette mode, i.e. is not inter, is a strong indication that the current CU may be of an SCC type.

In order to reduce the need for unavailable pixels, the angular modes are selected to point to the pixels of the available CUs, so:

- If the Left CU is neither Intra nor Inter (i.e. Intra Block Copy or IBC), then its candidate mode is set to HOR;

- If the Top CU is neither Intra nor Inter (i.e. Intra Block Copy or IBC), then its candidate mode is set to VER.

## Current draft text

The current HEVC Screen Content Coding Draft Text 2 does not account that palette and IBC are marked as intra, and CU actual coding mode must also check intra_bc_flag and palette_mode_flag. This can be seen in such sections as 7.3.8.5, “Coding unit syntax”: reading of palette_mode_flag[ x0 ][ y0 ] depends on CuPredMode[ x0 ][ y0 ]  = =  MODE_INTRA && !intra_bc_flag[ x0 ][ y0 ].

In that case, the specifications in section 8.4.2, “Derivation process for luma intra prediction mode” could read as:

Otherwise, if CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA, or intra_bc_flag[ xNbX ][ yNbX ] is equal to 1, or palette_mode_flag[ xNbX ][ yNbX ] is equal to 1, or pcm_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeX is set equal to INTRA_DC.

## Proposed normative changes

Still in section 8.4.2, “Derivation process for luma intra prediction mode”:

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If availableX is equal to FALSE, candIntraPredModeX is set equal to INTRA_DC.

- Otherwise, if CuPredMode[ xNbX ][ yNbX ] is equal to MODE_INTER or pcm_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeX is set equal to INTRA_DC,

- Otherwise, if X is equal to B and yPb − 1 is less than ( ( yPb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ), candIntraPredModeB is set equal to INTRA_DC.

- Otherwise, if intra_bc_flag[ xNbX ][ yNbX ] or palette_mode_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeA and candIntraPredModeB are respectively set to INTRA_ANGULAR10 and INTRA_ANGULAR26;

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

# Results

Thanks ITRI for cross-checking (JCTVC-T0207).

## Lossy scenarios, 4:4:4

## Lossless configuration, 4:4:4

## Lossy scenarios, 4:2:0

## Lossless scenarios, 4:2:0

# Conclusion

The proposal offers reasonable gains for a limited modification at the CU-level, so it is recommended to adopt it.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).