| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0050 |
| --- | --- |

| Title: | Non-CE2: IBC encoder improvements and CE2 combinations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Gilles Malard Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes several encoder improvements to the Intra Block Copy in the SCM3.0. In addition, these encoder changes are combined with several CE2 test experiments to show some additional coding gains compared to the current IBC design. The proposed encoding choice reportedly gives an average BDR of -0.7% for the AI configuration, -1.4% for RA and LDB configurations compared to the current SCM3.0. When this encoding choice is combined with some of the CE2 tests, the average BDR gain reaches -2.3% for LDB configuration.

# Introduction

The core experiment 2 is dedicated to the Intra block copy. Some of the tested methods improve the coding efficiency while introducing a noticeable encoding time increase. This encoding time increase comes from additional RD evaluations of the IBC mode. In this contribution, we propose to modify the mode selection process in order to find a better compromise between coding efficiency and complexity compared to CE2 proposal.

# Proposed Encoder changes

The proposed encoder changes consist in disabling some early terminations to increase the amount of IBC tested. Figure 1 shows a flow chart of mode selection process of the SCM3.0 and the proposed modifications. As it is depicted, we propose to remove the early termination after the Inter evaluation so that if the Inter mode is evaluated, the Fast IBC is also evaluated. (We didn’t change the Early Skip termination or the Hash Inter early termination). In the same way, if the Intra mode is evaluated IBC is also evaluated.

In addition, we remove one early termination in IBC Hash.

Figure 1 Flow chart of the mode selection process of the SCM3.0 and the proposed method.

# Experimental results

In this section, several experiments are reported. The first section is dedicated to the encoder only change and the second one to the combination of the encoder change with some CE2 proposal.

## Encoder only changes

Table 1 gives the results of the proposed encoder changes compared to the SCM3.0 for 4:4:4 configuration.

Table 1 Lossy results of the proposed encoder changes compared to SCM3.0 Anchor for 4:4:4 configuration.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.9% | -0.9% | -0.9% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.2% | -1.0% | -1.0% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.4% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.0% | -1.6% | -1.6% |
| RGB, mixed content, 1440p & 1080p | -0.6% | -0.5% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.5% | -2.3% | -2.4% |
| YUV, mixed content, 1440p & 1080p | -0.8% | -0.6% | -0.6% |
| YUV, Animation, 720p | -0.1% | -0.4% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.9% | -1.3% | -1.3% |
| RGB, mixed content, 1440p & 1080p | -0.8% | -0.4% | -0.7% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.3% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.4% | -1.9% | -1.9% |
| YUV, mixed content, 1440p & 1080p | -1.0% | -0.8% | -0.5% |
| YUV, Animation, 720p | -0.1% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

## Combination with CE2 proposals

In this section, we present the encoder only changes combined with the following CE2 Tests:

1/ CE2 Test 4.1 on deblocking filter

2/ CE2 Test 5.3 on BV coding

3/CE2 Test 2 on IBC PU

Table 2 gives the results of combination of the proposed encoding choice with some CE2 proposals. The proposed combination achieves 3.6% gain for YUV Text and graphics class for LDB configuration.

Table 2 Lossy results of the proposed encoder changes with CE2 proposals compared to SCM3.0 Anchor for 4:4:4 configuration.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.0% | -2.7% | -2.6% |
| RGB, mixed content, 1440p & 1080p | -0.5% | -1.1% | -1.3% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -1.3% | -2.8% | -2.8% |
| YUV, mixed content, 1440p & 1080p | -0.7% | -1.5% | -1.4% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.2% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.0% | -5.3% | -5.2% |
| RGB, mixed content, 1440p & 1080p | -1.1% | -2.6% | -2.8% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -3.3% | -5.5% | -5.7% |
| YUV, mixed content, 1440p & 1080p | -1.2% | -3.3% | -3.3% |
| YUV, Animation, 720p | -0.1% | -0.6% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.5% | -5.1% | -5.1% |
| RGB, mixed content, 1440p & 1080p | -1.6% | -3.0% | -3.3% |
| RGB, Animation, 720p | -0.2% | -0.4% | -0.5% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -3.6% | -5.6% | -5.9% |
| YUV, mixed content, 1440p & 1080p | -1.6% | -4.5% | -4.3% |
| YUV, Animation, 720p | -0.1% | -0.4% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

In this contribution we propose some encoder modifications to the Intra Block Copy search. With these modifications IBC is tested systematically when the Inter and the Intra modes are tested. These modifications increase the coding efficiency of the current IBC design. When these encoder changes are combined with some CE2 Tests the efficiency is increased.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).