| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0051 |
| --- | --- |

| Title: | AHG10: On IBC memory reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Gilles Malard Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution is related to the Intra block copy memory reduction. In the current SCM3.0, an IBC PU predictor comes from reconstructed non-filtered blocks. This increases the memory needed for some implementations, an example being the case where loop filtered blocks need to be stored in addition to the reconstructed blocks because of IBC. To avoid this additional storage, this contribution proposes to signal the CTBs which are available for IBC prediction. For these CTBs, the DBF and SAO are disabled. The method was initially introduced in JCTVC-S0068. Some adaptations have been made to the initial implementation to reduce the losses.  The proposed method reportedly gives an average BDR for all classes of 0.7%, 0.4%, 0.4% for the AI configuration, 0.3%, -0.6%, -0.6% for RA configuration and 0.3%, -0.5%, -0.5% for LDB configuration, compared to the current SCM3.0.

The proposed method is combined with an encoder modification and the results obtained give a global coding efficiency gain for all configurations.

# Introduction

For the Intra Block Copy method, the PU predictors come from reconstructed data of the current frame and as all other Intra modes this data is non-loop filtered.

In the HEVC specification, the DBF and SAO loop filtering are defined at picture level. But these filterings can be fully parallelized at CU level if all neighboring data are available. So for some implementations, the DBF and SAO are proceeded on the fly and/or in parallel to the other processes of block reconstruction. To keep this ability to apply the SAO and DBF at lower level in parallel, when Intra Block Copy is enabled, it is needed to store both loop filtered data and non-filtered data. This increases the memory needed by one frame for the worst case because a BV in the last CU can point at the very beginning of the image.

The alternative is to delay DBF and SAO until the frame is reconstructed, but this then requires saving all DBF and SAO syntax for delayed application.

In this contribution, the available CTBs for Intra Block Copy predictor are signaled in the bitstream. When a CTB is signaled as available for IBC, it is not loop filtered.

# Proposed method

At the beginning of each CTB, a flag, available_for_IBC_flag, is inserted in the bitstream. This flag indicates that the CTB can be used as reference for IBC prediction. Moreover when this flag is true, the deblocking filter and SAO are disabled. Compared to our initial proposal JCTVC-S0068, this flag is now a CABAC context coded bin.

In addition, the current CTB and the previous one are always considered as available for IBC prediction.

As a consequence, the memory increase for the implementation processing DBF and SAO on the fly is only 2 CTBs. Figure 1 illustrates the proposed method.

Figure 1 Example of the proposed method.

This method was implemented on top of SCM3.0 where DBF and SAO are processed at picture level.  In our implementation, before using DBF and SAO, the encoder sets the value of the flag available_for_IBC_flag to one if at least one block vector points to this CTB and if the IBC PU is not in the current CTB or following CTB.

In addition to the JCTVC-S0068, we adapted the frame level SAO early termination of the HM software (JCTVC-I0184) to the proposed method. The frame level SAO early termination consists in enabling/disabling SAO for frames of level 1 and level 2 according to the SAO selection of frame of level 0. With the proposed method, the SAO statistics also depend on the use of IBC and not only on the coding efficiency of SAO. So, we modified the computed statistics so that the encoder takes into account only the SAO selection of the CTBs where SAO can be applied (available_for_IBC_flag=0) and not for the whole frame.

# Experimental results

In this section, 2 experiments are reported. The first one is the results of the method on top of SCM3.0 and for the second one the encoder only modification presented in our contribution JCTVC-T0050 and 2 modifications proposed in JCTVC-T0116 are combined to the proposed method to show a global coding efficiency improvement.

## Proposed method

Table 1 gives the results of the proposed method. The method has a small impact on coding efficiency. In average for all classes 0.6%, 0.2%, 0.2% is obtained for the AI configuration. For inter configurations these average are respectively 0.3%, -0.5%, -0.5% for RA configuration and 0.3%, -0.4%, -0.4%. Moreover a significant decoding time reduction is observed for Intra configuration.

Table 1: Lossy results of the proposed method compared to SCM3.0 Anchor for 4:4:4 configuration.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.5% | -1.0% | -0.8% |
| RGB, mixed content, 1440p & 1080p | 0.9% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.2% | 0.6% | 0.2% |
| RGB, camera captured, 1080p | 0.3% | 0.2% | 0.7% |
| YUV, text & graphics with motion, 1080p & 720p | 0.9% | 0.0% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 1.2% | 1.1% | 1.1% |
| YUV, Animation, 720p | 0.6% | 1.2% | 0.7% |
| YUV, camera captured, 1080p | 0.7% | 0.9% | 0.7% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 87% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.4% | -1.4% | -1.3% |
| RGB, mixed content, 1440p & 1080p | 0.3% | -1.3% | -1.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.8% | -0.9% | -1.2% |
| YUV, mixed content, 1440p & 1080p | 0.7% | -1.1% | -1.0% |
| YUV, Animation, 720p | 0.2% | 0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.2% | 0.3% | 0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.6% | -0.7% | -0.6% |
| RGB, mixed content, 1440p & 1080p | 0.3% | -1.0% | -1.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.9% | -0.6% | -0.9% |
| YUV, mixed content, 1440p & 1080p | 0.7% | -1.7% | -1.7% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |

## Proposed method + Encoder IBC modifications

Table 2 gives the results of the proposed method combined with IBC encoder modifications. These encoder modifications are our proposed changes of JCTVC-T0050, the IBC temporal candidate and the change of color space for hash based IBC search proposed in T0116.

Table 2: Lossy results of the proposed method combined with encoder modifications compared to SCM3.0 Anchor for 4:4:4 configuration.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.9% | -3.4% | -3.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.8% | -0.9% |
| RGB, Animation, 720p | 0.2% | 0.6% | 0.1% |
| RGB, camera captured, 1080p | 0.4% | 0.2% | 0.8% |
| YUV, text & graphics with motion, 1080p & 720p | -1.6% | -2.4% | -2.4% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.5% | 1.1% | 0.6% |
| YUV, camera captured, 1080p | 0.7% | 0.9% | 0.7% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 86% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.9% | -3.8% | -3.5% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -1.8% | -1.8% |
| RGB, Animation, 720p | 0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.7% | -3.4% | -3.8% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -1.9% | -1.7% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.2% | 0.3% | 0.1% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.9% | -2.9% | -2.7% |
| RGB, mixed content, 1440p & 1080p | -0.4% | -1.6% | -1.9% |
| RGB, Animation, 720p | 0.0% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -1.6% | -3.4% | -3.5% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -2.6% | -2.3% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 101% |  |  |

# Conclusion

In order to solve the memory increase due to the use of non-loop filtered block as reference for Intra block copy, this contribution proposes to signal the available CTB for IBC, and to disable the deblocking filter and SAO for these available CTBs.

The proposed method reduces the memory impact for IBC to only 2 CTBs. The proposed method reportedly gives an average BDR for all classes of 0.6%, 0.2%, 0.2% for the AI configuration, 0.3%, -0.5%, -0.5% for RA configuration and 0.3%, -0.4%, -0.4% for LDB configuration, compared to the current SCM3.0. The small losses can be compensated with an encoder modification.

According to the significant advantage of the proposed method and the small impact on coding efficiency, we propose to adopt the proposed method.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).