| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0052 |
| --- | --- |

| Title: | Non-CE1: Escape coded pixel prediction for palette based coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Jiangli Ye , Jianqing Zhu 30F,Kerry Parkside,NO.1155 Fangdian Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5811 threerabbit@cn.fujitsu.com zhujianqing@cn.fujitsu.com |
| Source: | Fujitsu R&D Center Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution document, escape coded pixels are predicted by palette predictor or escape predictor which is constructed by the escaped pixels occurred in previous palette-coded CUs. There is a CU-level flag to indicate existence of escape coded pixel prediction mode in current CU. If the prediction exists, for each escape pixel, use a flag to indicate current escape coded pixel is predicted or not. For those escape coded pixels which are predicted, use an index to indicate which element of predictor is its prediction.

3 schemes were tested. 1) Escape coded pixels are predicted by palette predictor. Reported test results show gain of 0.1% and 0.2% on TGM RGB and YUV (AI-lossless) on top of SCM3.0 anchor when maximum palette predictor size is 64; 2) Escape coded pixels are predicted by escape predictor. Reported test results show gain of 1.0% and 1.4% on TGM RGB and YUV (AI-lossless) on top of SCM3.0 anchor when maximum escape predictor size is 64 and gain of 1.5% and 2.0% on TGM RGB and YUV (AI-lossless) on top of SCM3.0 anchor when maximum escape predictor size is 128; 3) Escape coded pixels are predicted by palette predictor and escape predictor. Reported test results show gain of 1.5% and 2.0% on TGM RGB and YUV (AI-lossless) on top of SCM3.0 anchor when maximum palette predictor size is 64 and maximum escape predictor size is 64;

# Introduction

CE1[1] was established for study on improvements of palette mod in the 19th JCT-VC meetings in Strasbourg. And base software,SCM3.0[2] was suggested. This contribution is an improvement on top of the base software.

Escape coded pixel prediction schemes for palette based coding [3] [4] [5] was proposed in Strasbourg meeting.

This contribution improved the escape coded pixel prediction and evaluated them on top of SCM3.0.

# Proposed methods

## Escape coded pixel prediction by palette predictor

- Escape coded pixel prediction

If escape coded pixel exists, a CU level flag is used to indicate whether prediction is used for escape coded pixels in current CU.

If the prediction is used in current CU, a flag is used for each escape coded pixel (containing 3 components) to indicate whether it is predicted or not.

If the escape coded pixel is predicted, an index is coded to indicate which element in palette predictor is its prediction. Otherwise (it is not predicted), the escape pixel is coded same as in base software.

- Predictor

The predictor is constructed from palette predictor by excluding the reused elements for palette prediction.

## Escape coded pixel prediction by escape predictor

- Escape coded pixel prediction

If escape coded pixel exists, a CU level flag is used to indicate whether prediction is used for escape coded pixels in current CU.

If the prediction is used in current CU, a flag is used for each escape coded pixel (containing 3 components) to indicate whether it is predicted or not.

If the escape coded pixel is predicted, an index is coded to indicate which element in escape predictor is its prediction. Otherwise (it is not predicted), the escape pixel is coded same as in base software.

- Escape predictor construction

Firstly, the decoded escape pixels are stuffed to form a list of escape pixels (each escape pixel in the list is unique and in decoding order), maximum number of escape pixels(MAX_ESC_SIZE) in the list is 64;

After decoding of each palette-coded CU, the list of escape pixels is stuffed into escape predictor in similar way as stuffing process of palette predictor.

There is MAX_ESC_PRED_SIZE to indicate maximum size of escape predictor.

## Escape coded pixel prediction by palette predictor and escape predictor

- Escape coded pixel prediction

If escape coded pixel exists, a CU level flag is used to indicate whether prediction is used for escape coded pixels in current CU.

If the prediction is used in current CU, a flag is used for each escape coded pixel (containing 3 components) to indicate whether it is predicted by an element of escape predictor or not.

If it is predicted by an element of escape predictor, an index is coded to indicate which element in escape predictor is its prediction. Otherwise, another flag is used to indicate whether it is predicted by an element of palette predictor or not.

If it is predicted by an element of palette predictor, an index is coded to indicate which element in palette predictor is its prediction. Otherwise, the escape pixel is coded same as in base software.

# Experimental results

The anchor is the base software of SCM3 [2]. Test condition is common conditions for screen content coding test in S1015[6]. In following test results, only all intra configuration is tested,

## Result of method 2.1

Table 1. Simulation results in lossless coding mode MAX_PLT_PRED_SIZE = 64

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.4% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.2% | -0.5% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

Table 2. Simulation results in lossy coding mode MAX_PLT_PRED_SIZE = 64

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 103% |  |  |

## Result of method 2.2

Table 3. Simulation results in lossless coding mode MAX_ESC_PRED_SIZE = 64

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -1.0% | -1.0% | -1.8% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.4% | -1.5% | -4.5% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 130% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

Table 4. Simulation results in lossy coding mode MAX_ESC_PRED_SIZE = 64

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 106% |  |  |

Table 5. Simulation results in lossless coding mode MAX_ESC_PRED_SIZE = 128

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -1.5% | -1.5% | -3.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.0% | -2.1% | -7.0% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.3% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 145% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

Table 6. Simulation results in lossy coding mode MAX_ESC_PRED_SIZE = 128

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  |
| Dec Time[%] | 107% |  |  |

## Result of method 2.3

Table 1. Simulation results in lossless coding mode MAX_PLT_PRED_SIZE = 64, MAX_ESC_PRED_SIZE = 64

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -1.5% | -1.5% | -3.1% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.0% | -2.1% | -7.0% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.3% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 120% |  |  |  |
| Dec Time[%] | 110% |  |  |  |

Table 2. Simulation results in lossy coding mode MAX_PLT_PRED_SIZE = 64, MAX_ESC_PRED_SIZE = 64

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 113% |  |  |

# References

- P. Lai, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma, “Description of Core Experiment 1 (CE1): Improvements of palette mode,” JCTVC-S1101, Strasbourg, FR, Oct. 2014.

- R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye, “Screen Content Coding Test Model 3 Encoder Description (SCM 3),” JCTV-S1014, Strasbourg, FR, Oct. 2014.

- J. Ye, J. Zhu, “Non-CE6: Escape coded pixel prediction using previous escape coded pixels for palette based coding,” JCTV-S0052, Strasbourg, FR, Oct. 2014.

- J. Ye, J. Zhu, “Non-CE6: Non-CE6: Escape pixel prediction using previous escape coded pixels and palette for palette based coding,” JCTV-S0053, Strasbourg, FR, Oct. 2014.

- J. Ye, J. Zhu, “Non-CE6: Escape coded pixel prediction using previous palette for palette based coding,” JCTV-S0054, Strasbourg, FR, Oct. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu“Common conditions for screen content coding tests,” JCTVC-R1015, Sapporo, JP, July. 2014.

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).