| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0053 |
| --- | --- |

| Title: | Non-CE1: Palette based coding for non-4:4:4 format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Jianqing Zhu Zheng Wang Jiangli Ye 30F,Kerry Parkside,NO.1155 Fangdian Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5811 zhujianqing@cn.fujitsu.com cheng@cn.fujitsu.com threerabbit@cn.fujitsu.com |
| Source: | Fujitsu R&D Center Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution document, a scheme of palette mode for YUV4:2:2 and YUV4:2:0. There are 2 parts in palette. First part is a triplet palette. Second part is a luma palette. Index map coding is same as that of RGB/YUV4:4:4. Escape coded pixel is coded in 3-components or 1-component depends on a flag or the condition whether its corresponding chroma components have been coded or not. A simplified implementation is also proposed. Reported test results of simplified implementation show average gain of 0.5% on AI-lossless and 1.6%,4% and 2% on AI-lossy,Y,U,V respectively on sequences of YUV4:2:0 on top of SCM3.0 anchor.

# Introduction

CE1[1] was established for study on improvements of palette mod in the 19th JCT-VC meetings in Strasbourg. And base software,SCM3.0[2] was suggested. In SCM3.0, palette mode is only applied to RGB/YUV4:4:4. This contribution presents a scheme of palette mode for YUV4:2:2 and YUV4:2:0.

# Proposed method

## Palette mode for non-4:4:4

A CU in non-4:4:4 format is coded as follows.

- Firstly, palette is coded. There are 2 parts in palette. Each entry in first part of palette includes 3 components, i.e.(Y,U,V). Each entry in second part of palette is only luma component, i.e. Y. Put the second part of palette behind the first part. Given the number of entries in 1st part and 2nd part are Part1PaletteNum and Part2PaletteNum respectively. Part1PaletteNum is larger than 0. Part2PaletteNum is larger or equal to 0. The palette prediction is same as that in SCM3.0 for RGB/YUV 4:4:4.

- Secondly, index map is coded. The coding method is same as that in SCM3.0 for RGB/YUV 444.

- If the pixel is in escape mode, luma and chroma components (Y,U,V) or only luma (Y) component of this pixel is coded. A flag might be coded to indicate it is 3 components’ escape mode or just 1 compenent’s escape mode.

For format 4:2:0, pixel(2x,2y), pixel(2x+1,2y),pixel(2x,2y+1) and pixel(2x+1,2y+1) share same chroma value as shown in Fig-1(1).The chroma information, U(x,y) and V(x,y), can be coded in any one of these 4 pixels, either in palette mode or in escape mode.

For format 4:2:2, pixel(2x,y) and pixel(2x+1,y) share same chroma value as shown in fig1(2). The chroma information, U(x,y) and V(x,y), can be coded in any one of these 2 pixels, either in palette mode or in escape mode.

Fig-1

For pixel(x,y), the following rules are obeyed.

-If its index is in range of (0, Part1PaletteNum-1) and chroma(x/ SubWidthC, y/ SubHeightC) has not been coded, the corresponding (Y,U,V) in palette are valid for this pixel.

-If its index is in range of (0, Part1PaletteNum-1) and chroma(x/ SubWidthC, y/ SubHeightC) has been coded, only Y component in palette is valid for this pixel.

-If it is in escape mode and chroma(x/ SubWidthC, y/ SubHeightC) has been coded, only Y component is coded.

-If it is in escape mode and chroma(x/ SubWidthC, y/ SubHeightC) has been not coded, whether (Y,U,V) or just Y component is coded depends on a flag. The flag is coded firstly.

## A simplified implementation of 2.1for 4:2:0

Pixel (2x,2y) is coded in 3 components, either in palette or in escape mode.

For escaped coded pixel, no need flag to indicate 3 components or 1 component coded because only location (2x,2y) has 3 components.

# Experimental results of simplified implementation

The anchor is the base software of SCM3 [2]. Test condition is common conditions for screen content coding test in S1015[3]. In following test results, only all intra configuration is tested.

Table 1. Simulation results in lossless coding mode

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | -0.2% | -0.2% | -0.2% | -0.1% |
| Mixed content, 480p | -0.3% | -0.3% | -0.3% | -0.3% |
| Animation, 768p | -1.0% | -1.0% | -1.0% | -1.0% |
| Average of all sequences | -0.5% | -0.4% | -1.0% | -0.1% |
| Enc Time[%] | 127% |  |  |  |
| Dec Time[%] | 104% |  |  |  |

Table 2. Simulation results in lossy coding mode

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | -0.9% | -1.1% | -1.2% |
| Mixed content, 480p | -0.7% | -2.9% | -3.1% |
| Animation, 768p | -4.0% | -11.0% | -2.5% |
| Average of all sequences | -1.6% | -4.0% | -2.0% |
| Enc Time[%] | 119% |  |  |
| Dec Time[%] | 104% |  |  |

# References

- P. Lai, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma, “Description of Core Experiment 1 (CE1): Improvements of palette mode,” JCTVC-S1101, Strasbourg, FR, Oct. 2014.

- R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye, “Screen Content Coding Test Model 3 Encoder Description (SCM 3),” JCTV-S1014, Strasbourg, FR, Oct. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu“Common conditions for screen content coding tests,” JCTVC-R1015, Sapporo, JP, July. 2014.

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).