| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0054 |
| --- | --- |

| Title: | Non-CE1: On copy above mode for palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Jianqing Zhu , Zhanglei Xu 30F,Kerry Parkside,NO.1155 Fangdian Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5811 zhujianqing@cn.fujitsu.com xuzhanglei@cn.fujitsu.com |
| Source: | Fujitsu R&D Center Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution document, it is proposed to use pixel value instead of index for copy above mode in palette mode. And copy above mode is applied for the 1st line of CU. Reported test results show gain of 0.3% and 0.4% on AI-lossless of TGM RGB and YUV and 0.7% and 0.8% on AI-lossy TGM RGB and YUV contents on top of SCM3.0.

# Introduction

CE1[1] was established for study on improvements of palette mod in the 19th JCT-VC meetings in Strasbourg. And base software,SCM3.0[2] was suggested. This contribution is an modification on top of the base software.

In current test model of palette mode, index is used for copy above mode. However, there are some problems brought by this practice. A modified copy above mode for palette based coding [3] was proposed in Valencia meeting. It suggested using pixel value instead of index for copy above mode in palette mode. By using this scheme, these problems can be overcome.

This contribution evaluates above method on top of SCM3.0.

# Problems

## Copy above mode for the 1st line.

In current test model, copy above mode is forbidden for the 1st line. It will lose some gain.

If it is allowed for the 1st line, the index of above line cannot be used directly. It should be converted into index by searching current palette. The complexity is introduced in both encoding side and decoding side.

## Escape coded pixel

There are 2 problems for escape coded pixel.

Problem1: coding gain

For escape coded pixels in a same run, the pixel value of each escaped coded pixel will be coded even if they have same value.

Problem2: throughput

Pixels cannot be constructed immediately after index decoded for copy above mode if there are escape coded pixels existed. Pixel value will not be constructed until escaped coded pixels are parsed and decoded. It is not good for pipeline implementation.

# Proposal

To overcome above problems, it is proposed to use pixel value instead of index for copy above mode in palette mode. And copy above mode is applied for the 1st line of CU.

# Experimental results

The anchor is the base software of SCM3 [2]. Test condition is common conditions for screen content coding test in S1015[4]. In following test results, only all intra configuration is tested,

Table 1. Simulation results in lossless coding

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.5% | -1.6% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -0.5% | -1.6% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 97% |  |  |  |

Table 2. Simulation results in lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.7% | -0.7% | -0.7% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.8% | -0.8% | -0.7% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 98% |  |  |

# References

- P. Lai, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma, “Description of Core Experiment 1 (CE1): Improvements of palette mode,” JCTVC-S1101, Strasbourg, FR, Oct. 2014.

- R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye, “Screen Content Coding Test Model 3 Encoder Description (SCM 3),” JCTV-S1014, Strasbourg, FR, Oct. 2014.

- J. Zhu, J. Ye, K. Kazui, “AHG10: Modified copy above mode for palette based coding,” JCTVC-Q0174, Valencia, ES, March. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu“Common conditions for screen content coding tests,” JCTVC-R1015, Sapporo, JP, July. 2014.

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).