| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0055 |
| --- | --- |

| Title: | CE2-related: IntraBC constraint for multiple slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang1, Xiaozhong Xu2, Yu-Wen Huang1, Shan Liu2, Shawmin Lei1 1 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 2 MediaTek San Jose: 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | Tzu-Der Chuang +886-3-5670766 ext. 24602 {peter.chuang, xiaozhong.xu, yuwen.huang, shan.liu, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, an IntraBC reference area constraint is presented for multi-slice configuration. When a setting of multiple slices per frame is applied, IntraBC block can only refer to reconstructed samples within the same slice. In this way, as in HEVC and all other HEVC extensions, each slice can be reconstructed independently (and even decoded independently with certain loop filtering settings). Experimental results show that, under a configuration with maximum of 150 CTUs per slice, the proposed constraint causes 1.1 % and 1.2% BD-rate increases for RGB and YUV text & graphics with motion sequences under AI configuration.

# Introduction

In SCM-3.0 IntraBC search, a ladder shape search area constraint is applied as shown in Figure 1. For an IntraBC block in current CTU, its available search area is constrained to the blue CTUs and the reconstructed blocks in the current CTU.

Figure 1.  Ladder shape search area constraint for IntraBC

Figure 2 shows an example of a multi-slice configuration. A picture is divided into several slices. The maximum number of CTUs per slice is 19. In HEVC and all HEVC extensions except the HEVC SCC extensions, each slice can be reconstructed independently and even decoded independently with certain loop filtering settings.

In SCM-3.0, when the multi-slice configuration is applied, for an IntraBC block in CTU-35, its available search area is still the blue CTUs and the reconstructed blocks in CTU-35, as shown in Figure 2, which includes the CTUs of a non-current slice (e.g. CTU-0 to CTU-14, and CTU-16 to CTU-18). It violates the principle of reconstructing each slice independently.

Figure 2. Illustration of a multi-slice configuration

# Proposed constraint for multi-slice configuration

In this contribution, an IntraBC reference area constraint is proposed to limit the reference area of an IntraBC block to reconstructed samples within the same slice. In each slice, the ladder shape search area constraint is still applied. For example, in Figure 3, for an IntraBC block in CTU-35, its available search area is the CTU-19 to CTU-21, CTU-24 to CTU-28, CTU-32 to CTU-34, and the reconstructed blocks in CTU-35. In this way, each slice can be reconstructed independently.

Figure 3. Proposed constraint in multi-tiles coding

# Experimental results

The proposed method is integrated on SCM-3.0 [1]. All tests were conducted under the common test condition (CTC) [2] for SCC with the full picture IBC search range, except the multi-slice setting. For the anchor and the test, a multi-slice setting is applied. Each picture is divided into several slices. The maximum number of CTUs per slice is 150. Table 1 and Table 2 show the results of the proposed multiple-slices constraint for lossy and lossless coding respectively. The runtimes are not accurate.

# Conclusions

This contribution proposes to limit the reference area of an IntraBC block to reconstructed samples within the same slice. With the proposed method, each slice can be reconstructed independently.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- SCM-3.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2+SCM-3.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1015, Nov. 2014.

Table 1. Lossy results of the proposed constraint under a 150-CTU slice setting

Table 2. Lossless results of the proposed constraint under a 150-CTU slice setting