| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0056 |
| --- | --- |

| Title: | CE2-related: IntraBC constraint for multiple tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang1, Xiaozhong Xu2, Yu-Wen Huang1, Shan Liu2, Shawmin Lei1 1 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 2 MediaTek San Jose: 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | Tzu-Der Chuang +886-3-5670766 ext. 24602 {peter.chuang, xiaozhong.xu, yuwen.huang, shan.liu, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, an IntraBC reference area constraint is presented for multi-tile configuration. When a setting multiple tiles per frame is applied, IntraBC block can only refer to reconstructed samples within the same tile. In this way, as in HEVC and all other HEVC extensions, each tile can be reconstructed independently (and even decoded independently with certain loop filtering settings). Experimental results show that, under a configuration of 1x2 tiles per frame, the proposed constraint causes 3.0 % and 3.2% BD-rate increases RGB and YUV text & graphics with motion sequences under AI configuration.

# Introduction

In SCM-3.0 IntraBC search, a ladder shape search area constraint is applied as sown in Figure 1. For an IntraBC block in current CTU, its available search area is constrained to the blue CTUs and the reconstructed blocks in the current CTU.

Figure 1.  Ladder shape search area constraint for IntraBC

Figure 2 shows an example of a 2x2-tile per frame configuration. A picture is divided into four equal size tiles. In HEVC and all HEVC extensions except the HEVC SCC extensions, each tile can be reconstructed independently and even decoded independently with certain loop filtering settings. The numbers are the CTU indices. In a single thread decoder, the decoding order follows the CTU indices.

In SCM-3.0, when the multi-tile configuration is applied, for an IntraBC block in CTU-31, its available search area is still the blue CTUs and the reconstructed blocks in CTU-31, as shown in Figure 2, which includes the CTUs of non-current tiles (e.g. CTU-0 to CTU-18, CTU-20, CTU-21, and CTU-36). It violates the principle of reconstructing each tile independently. Furthermore, in current SCM-3.0, in which uses single-thread processing, CTU-36 is not yet available when CTU-31 is processed, and mismatch between encoder and decoder occurs.

Figure 2. Illustration of a 2x2-tile per frame configuration

# Proposed constraint for multi-tile configuration

In this contribution, an IntraBC reference area constraint is proposed to limit the reference area of an IntraBC block to reconstructed samples within the same tile. For example, in Figure 3, for an IntraBC block in CTU-31, its available search area is the CTU-24 to CTU-30 and the reconstructed blocks in CTU-31. In each tile, the ladder shape search area constraint is still applied. For example, in Figure 3, for an IntraBC block in CTU-21, its available search area is the CTU-12 to CTU-18, CTU-20, and the reconstructed blocks in CTU-21. In this way, each tile can be reconstructed independently.

Figure 3. Proposed constraint in multi-tiles coding

# Experimental results

The proposed method is integrated on SCM-3.0 [1]. All tests were conducted under the common test condition (CTC) [2] for SCC with the full picture IBC search range, except the multi-tile setting. For the anchor and the test, a 1x2 equal size tile setting is applied. Each picture is divided into two top/bottom equal size tiles, as shown in Figure 4. Table 1 and Table 2 show the results of the proposed multiple-tiles constraint for lossy and lossless coding respectively. Table 3 and Table 4 show the results of the proposed multiple-tiles constraint for lossy and lossless coding in 4:2:0 format respectively. The runtimes are not accurate.

Figure 4. Illustration of a 1x2-tile per frame configuration

# Conclusions

This contribution proposes to limit the reference area of an IntraBC block to reconstructed samples within the same tile. With the proposed method, each tile can be reconstructed independently.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- SCM-3.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2+SCM-3.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1015, Nov. 2014.

Table 1. Lossy results of the proposed constraint under a 1x2–tile per frame setting

Table 2. Lossless results of the proposed constraint under a 1x2–tile per frame setting

Table 3. Lossy results of the proposed constraint under a 1x2–tile per frame setting in 4:2:0 format

Table 4. Lossless results of the proposed constraint under a 1x2–tile per frame setting in 4:2:0 format