| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0057 |
| --- | --- |

| Title: | CE2-related: Temporal BV merge mode on CE2 Test-1 and Test-7 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Tzu-Der Chuang +886-3-5670766 ext. 24602 {peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In CE2 Test-1 and Test-7, the IntraBC mode is signalled without intra_bc_flag by reusing Inter signalling by adding the reconstructed current picture as the last reference picture in reference picture List_0. The inserted reconstructed current picture is marked as long-term reference picture. Therefore, the BV of the IntraBC coded block cannot be used to derive the temporal merge candidates. This contribution proposes to utilize the BVs in temporal merge candidate derivation if the co-located block is coded in IntaBC mode. Experimental results reportedly show that 0.6-0.7% gains are achieved on top of CE2 Test-1 and Test-7 for RGB and YUV text & graphics with motion sequences in lossy condition under inter configuration.

# Introduction

In HEVC temporal merge mode derivation, a scaled motion vector is derived based on co-located PU belonging to the picture which has the smallest POC difference with current picture within the given reference picture list. The reference picture list to be used for derivation of the co-located PU is explicitly signalled in the slice header. The scaled motion vector for temporal merge candidate is obtained as illustrated by the dotted line in Figure 1, which is scaled from the motion vector of the co-located PU using the POC distances, tb and td, where tb is defined to be the POC difference between the reference picture of the current picture and the current picture and td is defined to be the POC difference between the reference picture of the co-located picture and the co-located picture. The reference picture of the current picture is the picture with reference picture index (ref_Idx) equal to 0 in List_0. The ref_Idx of temporal merge candidate is set equal to zero. However, in temporal merge candidate derivation, if the reference picture of the co-located picture is long-term reference picture and the reference picture of the current picture is not long-term reference picture, the temporal merge candidate is not available.

In CE2 Test-1 [1] and Test-7 [2], the IntraBC mode is signalled without intra_bc_flag by reusing Inter signalling by adding the reconstructed current picture as the last reference picture in reference picture list 0 (List_0). With the shared signalling, IntraBC can be differentiated from the conventional Inter prediction by checking the selected reference picture. If the picture order count (POC) of the selected reference picture is the same as the POC of the current picture, the prediction block is coded in IntraBC mode. In CE2 Test-1 and Test-7, the inserted reconstructed current picture is marked as long-term reference picture. Since the reconstructed current picture is marked as long-term reference picture, the block vector (BV, or said the MV that points to the reconstructed picture of the co-located picture) of the co-located IntraBC coded PU cannot be used to derive the temporal merge candidate.

Figure 1.  Illustration of motion vector scaling for temporal merge candidate

# Proposed method

In this contribution, the BVs are proposed to be utilized in temporal merge candidate derivation. In temporal merge candidate derivation for List_0 MV, if the MV of the co-located block is BV (points to the reconstructed picture of the co-located picture), the BV is used as the temporal merge candidate. The ref_Idx of temporal merge candidate is set equal to reference picture index that points to the current reconstructed picture, which is the last ref_Idx in List_0.

In the proposed method, it might generate the temporal merge candidates that List_0 MV is a BV, and List_1 MV is a normal MV. This kind of merge candidates is disabled in CE2 Test-1 and Test-7 encoder searching. In this contribution, the encoder modified to be able to search this kind of merge candidates.

# Experimental results

The proposed method is integrated on SCM-3.0 [3]. All tests were conducted under the common test condition (CTC) [4] for SCC with the full picture IBC search range. Table 1 and Table 2 show the results of the proposed method on top of CE2 Test-1 for lossy and lossless coding respectively. Table 3 and Table 4 show the results of the proposed method on top of CE2 Test-7 for lossy and lossless coding respectively. In Table 1 to Table 4, the anchors are SCM-3.0. The runtime are not accurate.

Table 5 and Table 6 show the results of the proposed method on top of CE2 Test-1 and CE2 Test-7 for lossy coding respectively. The anchor in Table 5 is CE2 Test-1, and the anchor in Table 6 is CE2 Test-7. For RGB and YUV text & graphics with motion sequences 0.6-0.7% BD-rate gains are shown under inter configuration. The runtimes are not accurate.

The proposed encoder modification can be applied on CE2 Test-1 and Test-7 as well. Table 7 and Table 8 show the results of the proposed method on top of CE2 Test-1 and CE2 Test-7 for lossy coding respectively. The anchor in Table 5 is CE2 Test-1 with encoder modification, and the anchor in Table 6 is CE2 Test-7 with encoder modification. For RGB and YUV text & graphics with motion sequences 0.4-0.5% BD-rate gains are shown under inter configuration. The runtimes are not accurate.

# Conclusions

In CE2 Test-1 and Test-7, the reconstructed current picture is inserted to the reference frame list and marked as long-term picture. Therefore, the BV of the IntraBC coded block cannot be used to derive the temporal merge candidates. This contribution proposes to utilize the BVs in temporal merge candidate derivation if the co-located block is coded in IntaBC mode. Experimental results reportedly show 0.6-0.7% gains on top of CE2 Test-1 and Test-7, for RGB and YUV text & graphics with motion sequences in lossy condition under inter configuration.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C. Pang, Y.-K. Wang, V. Seregin, K. Rapaka, M. Karczewicz, X. Xu, S. Liu, S. Lei, B. Li, J. Xu, “CE2 Test1: Intra block copy and inter signalling unification,” Document of Joint Collaborative Team on Video Coding, JCTVC-T0094, Feb. 2015.

- X. Xu, T.-D. Chuang, S. Liu, S. Lei, “CE2 Test 7: Combination tests of Test 1 and Test 3.2,” Document of Joint Collaborative Team on Video Coding, JCTVC-T0075, Feb. 2015.

- SCM-3.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2+SCM-3.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1015, Nov. 2014.

Table 1.  Lossy results of the proposed method. 
Anchor: SCM-3.0. Test: CE2 Test-1 + proposed method.

Table 2.  Lossless results of the proposed method. 
Anchor: SCM-3.0. Test: CE2 Test-1 + proposed method.

Table 3.  Lossy results of the proposed method. 
Anchor: SCM-3.0. Test: CE2 Test-7 + proposed method.

Table 4.  Lossless results of the proposed method. 
Anchor: SCM-3.0. Test: CE2 Test-7 + proposed method.

Table 5.  Lossy results of the proposed method. 
Anchor: CE2 Test-1. Test: CE2 Test-1 + proposed method.

Table 6.  Lossy results of the proposed method. 
Anchor: CE2 Test-7. Test: CE2 Test-7 + proposed method.

Table 7.  Lossy results of the proposed method. 
Anchor: CE2 Test-1 + encoder modification. Test: CE2 Test-1 + proposed method.

Table 8.  Lossy results of the proposed method. 
Anchor: CE2 Test-7+ encoder modification. Test: CE2 Test-7 + proposed method.

Table 9.  Lossy results of the proposed method in 4:2:0 format. 
Anchor: SCM-3.0. Test: CE2 Test-1 + proposed method.

Table 10.  Lossless results of the proposed method in 4:2:0 format. 
Anchor: SCM-3.0. Test: CE2 Test-1 + proposed method.

Table 11.  Lossy results of the proposed method in 4:2:0 format. 
Anchor: SCM-3.0. Test: CE2 Test-7 + proposed method.

Table 12.  Lossless results of the proposed method in 4:2:0 format. 
Anchor: SCM-3.0. Test: CE2 Test-7 + proposed method.