| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0058 |
| --- | --- |

| Title: | CE1-related: Index map scan for 64x64 palette coding block |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Chen Sun, Yu-Wen Huang, Shawmin Lei MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Tzu-Der Chuang +886-3-5670766 ext. 24602 {peter.chuang, chingyeh.chen, yc.sun, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HEVC and its non-SCC extensions, hardware decoding is often pipelined with 32x32 processing units due to the maximum transform block size equal to 32x32. In SCM-3.0, a 64x64 traverse scan is utilized for 64x64 palette coded block. A different block pipelining scheme (e.g., 64x64, which results in significantly larger silicon area) is needed. In this contribution, the 64x64 traverse scan is divided into four 32x32 traverse scans to accommodate to the 32x32 block pipelining. It is asserted that the implementation cost can be reduced. It is reported the BD-rate increases are 0-0.2% with average smaller than 0.1%.

# Introduction

For hardware design of video encoding or decoding, a pipelined architecture is usually utilized to improve the processing throughput. In a pipelined architecture, the processing unit of each pipeline stage is the key to achieve a high processing throughput with a low hardware cost. The processing unit of each pipeline stage should be similar and as small as possible in order to reduce the required buffers between different pipeline stages. For example, a pipelined HEVC decoder architecture is shown in Fig. 1. In HEVC decoding processing, the entropy decoder parses the bitstream one bit by one bit. The IQ/IT should be able to perform a 32x32 TU, which aligns with the maximum TB size in the HEVC standard. For intra prediction, the processing unit is decided by TB partitions, so the processing unit of intra prediction and reconstruction is also one 32x32 block. As for inter prediction, the maximum size of supported PUs is 64x64. In deblocking stage, the processing unit is one 8x8 block based on the filtering operations. For SAO, its processing unit is one 64x64 block. According to the above, it seems that the processing unit should be one 64x64 block in one HEVC decoder. However, for inter prediction and SAO processing, although their processing units are 64x64 blocks, the operations of inter prediction and SAO can be performed independently for each pixels. That is to say, we can divide one 64x64 block into four 32x32 blocks and process four blocks in sequential. Then, one 32x32 block is enough to be the processing unit of each pipeline stage in one HEVC decoder.

In HEVC Screen Content Coding Extensions, a palette mode is introduced. For a 64x64 block coded in palette mode, a 64x64 traverse scan is utilized, as shown in Figure 2. In this scan order, a 64x64 block cannot be divided into four 32x32 sub-blocks. Therefore, in a hardware decoder implementation, a larger buffer (e.g. 64x64 buffer) is required to store these palette related information, and the processing unit of the palette reconstruction module is forced to be a 64x64 block. It increases the implementation cost and complexity.

Figure 1.  An example of pipelined HEVC video decoder architecture

Figure 2. Traverse scan of a 64x64 block

# Proposed index map scan for 64x64 palette CU

## Method-1: divided into four 32x32 traverse scans

In this contribution, an alternative scan order is proposed for 64x64 palette coded block, as shown in Figure 3. A 64x64 traverse scan is divided into four 32x32 traverse scans. The scan order for the four 32x32 sub-blocks is Z order, which fits into the HEVC residue-quad-tree scan. With the proposed scan order, the processing unit of the palette reconstruction module can be a 32x32 block, which is unified with the processing unit of the conventional HEVC decoder design, and a 32x32 pipeline buffer is enough. The implementation cost and complexity for a 64x64 palette coded block decoding can be reduced significantly.

Figure 3. Proposed scan order for a 64x64 palette coded CU

## Method-2 (requested during the meeting): disable palette mode for 64x64 block

Another alternative method is to disable palette mode for 64x64 blocks. The text modification is listed as below.

coding_unit( x0, y0, log2CbSize ) {

...

nCbS = ( 1 << log2CbSize )

...

if( palette_mode_enabled_flag && ChromaArrayType = = 3 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && !intra_bc_flag[ x0 ][ y0 ] && nCbS != 64 )

palette_mode_flag[ x0 ][ y0 ]

if( palette_mode_flag[ x0 ][ y0 ] )

palette_coding( x0, y0, nCbS )

else {

...

# Experimental results

The proposed method is integrated on SCM-3.0 [1]. All tests were conducted under the common test condition (CTC) [2] for SCC with the full picture IBC search range. Table 1 and Table 2 show the results of the proposed alternative scan order for 64x64 palette coded block for lossy and lossless coding respectively. For RGB and YUV text & graphics with motion sequences in lossy condition, 0.1% and 0.1% BD-rate increases are shown under AI configuration, 0.1% and 0.2% BD-rate increases are shown under RA configuration, and 0.1% and 0.1% BD-rate increases are shown under LB configuration. The runtimes are not accurate.

The results of method-1 and method-2 on top of CE1 Test A.1.5 are shown in Table 3 and Table 4 respectively. The anchors of Table 3 and Table 4 are CE1 Test A.1.5.

# Conclusions

This contribution proposes an alternative scan order for 64x64 palette coded block. With proposed scan order, the implementation cost and complexity for a 64x64 palette coded block decoding can be reduced. Experimental results reportedly show 0.0-0.2% losses for RGB and YUV text & graphics with motion sequences in lossy condition.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- SCM-3.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2+SCM-3.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1015, Nov. 2014.

Table 1.  Lossy results of the proposed index map scan for 64x64 palette CU

Table 2.  Lossless results of the proposed index map scan for 64x64 palette CU

Table 3.  Lossy results of the proposed index map scan for 64x64 palette CU on top of CE1 Test A.1.5

Table 4.  Lossy results of disabling palette mode for 64x64 block on top of CE1 Test A.1.5