| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0059 |
| --- | --- |

| Title: | On adaptive motion vector resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Xianguo Zhang1, Jicheng An1, Han Huang1, Shawmin Lei2 1 MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Kai Zhang +86-10-5690-0888 ext. 83890 {kai.zhang, xianguo.zhang, jicheng.an, h.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current HEVC Screen Content Coding (SCC) Extensions, a slice-level adaptive motion vector resolution (AMVR) approach is adopted to improve coding efficiency. However, AMVR may incur inconsistence between the resolution of the temporal motion vector prediction (TMVP) and the resolution of the MV. Moreover, it may violate the MV range constraint in HEVC specification. In this contribution, two modifications are proposed. First, the resolution of TMVP and the resolution of the MV are set to be consistent. Second, MVs are constrained differently when different MV resolutions are applied. Experimental results reportedly show that the proposed methods almost do not change the coding performance.

# Introduction

In HEVC SCC Extensions [1], a slice-level adaptive motion resolution (AMVR) method is adopted to improve coding efficiency. In this approach, a flag (use_integer_mv_flag) is signaled in slice header to indicate whether luma MVs in the current slice use integer-sample or quarter-sample resolution. For simplification, MVs are parsed and reconstructed in the same way in both integer-sample and quarter-sample cases. The motion vector should be shifted left by 2 (MV=MV<<2) before the interpolation process when integer-sample resolution is selected. The current design of AMVR incurs two problems.

First, the resolution of the temporal motion vector prediction (TMVP) and the resolution of the MV may be mismatched. For example, if MVs of the current slice use integer-sample resolution but MVs of the collocated picture use quarter-sample resolution, TMVP will become inaccurate.

Second, the MV range constraint may be violated by AMVR. In HEVC, it is constrained that

The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −215 to 215 − 1, inclusive.

With this constraint, we can guarantee two properties:

- MV can always be represented and operated within 16 bits (a short type variable);

- The accessing range in the reference picture will not exceed [-213, 213-1] x [-213, 213-1] for each luma sample.

However, when use_integer_mv_flag is equal to 1, MV cannot be handled within 16 bits in the interpolation process. Besides, the accessing range in the reference picture will become [-215, 215-1] x [-215, 215-1], which is four times of [-213, 213-1] x [-213, 213-1].

# Proposed methods

In this contribution, two modifications for AMVR are proposed.

First, it is proposed to make the resolution of TMVP consistent with the resolution of MVs in the current slice.

When use_integer_mv_flag = 1,

–	TMVP = TMVP >> 2 before it is used as MVP.

–	MV = MV << 2 before it is stored into the MV buffer, which will be used as TMVP by following pictures.

Second, it is proposed to set different constraint for different MV resolution.

If use_integer_mv_flag is equal to 0,

Otherwise (use_integer_mv_flag is equal to 1),
. In this way, we can guarantee the two properties no matter use_integer_mv_flag is equal to 0 or 1.

The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −215 to 215 − 1, inclusive.

The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −213 to 213 − 1, inclusive.

# Experimental results

The proposed method is integrated on SCM-3.0 [2]. All tests were conducted under the common test condition (CTC) [3] for SCC. Table 1 and Table 2 show the results of the proposed methods for lossy and lossless coding respectively. The results reportedly show almost no coding performance change.

Table 1. Lossy results of the proposed method.

Table 2. Lossless results of the proposed method.

# Conclusions

This contribution proposes two modifications on AMVR. First, the resolution of TMVP should be consistent with the resolution of MVs in the current slice. Second, the MV range is constrained based on MV resolution, so that the all operations on MVs can be done within 16 bits.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- R. Joshi, J. Xu, “HEVC Screen Content Coding Draft Text 2,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1005, Oct. 2014.

- SCM-3.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2+SCM-3.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1015, Oct. 2014.