| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0060 |
| --- | --- |

| Title: | CE1-related: Table based binarization for palette_escape_val |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Han Huang1, Shawmin Lei2 1 MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Kai Zhang +86-10-5690-0888 ext. 83890 {kai.zhang, jicheng.an, xianguo.zhang, h.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HEVC Screen Content Coding (SCC) Extensions, palette_escape_val is binarized using truncated binary (TB) code with cMax calculated by a complicated procedure. This contribution proposes to fetch cMax from a predefined table directly with QP as the table input. Experimental results reportedly show that coding performance is not changed by the proposed method.

# Introduction

In HEVC SCC Extensions [1], palette_escape_val is binarized using truncated binary (TB) code. In TB code, the maximum possible code value, cMax, should be specified. To code palette_escape_val, cMax is calculated as below.

- The quantization parameter qP is derived as follows:

qP = ( cIdx  = =  0 ) ? Qp′Y : ( ( cIdx  = =  1 ) Qp′Cb ? Qp′Cr )

- A quantization step size parameter qStep is derived as follows:

qStep = ( qP= =0 ) ? Round( 2(qP − 4) / 6 ) : 1

- A maximum possible quantized value maxValue is derived as follows:

maxValue = ( ( 1<<bitDepth ) − 1 ) / Qstep

- The number of bins numBins of the fixed length binarization codeword is derived as follows

while( maxValue ) {
			maxValue = maxValue >> 1
			numBins++
		}

- The maximum parameter cMax for the fixed length binarization is derived as follows

cMax = ( 1 << numBins ) − 1

The derivation of cMax is quite complicated with some division operations. In the worst case, this procedure is invoked for each coding unit (CU). Furthermore, these complicated calculations are applied in the parsing process and may degrade the parsing throughput. It should be noted that cMax cannot be calculated once at the beginning of a slice since QP for each CU may be different.

# Proposed method

It is proposed to fetch cMax from a look-up table indexed by QP directly to avoid the complex calculations. Tables are predefined with fixed entries for different bit-depth value. Table 1 demonstrates the look-up table when bit-depth is equal to 8.

Table 1. cMax lookup table for bit-depth = 8

| QP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cMax | 255 | 255 | 255 | 255 | 255 | 227 | 204 | 181 | 127 | 127 | 127 | 113 | 102 |
| QP | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| cMax | 91 | 80 | 63 | 63 | 57 | 51 | 45 | 40 | 36 | 31 | 28 | 25 | 23 |
| QP | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| cMax | 20 | 18 | 15 | 14 | 13 | 11 | 10 | 9 | 7 | 7 | 6 | 6 | 5 |
| QP | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| cMax | 4 | 3 | 3 | 3 | 3 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |

# Experimental results

The proposed method is integrated on SCM-3.0 [2]. All tests were conducted under the common test condition (CTC) [3] for SCC. Table 2 and Table 3 show the results of the table based binarization of palette_escape_val for lossy and lossless coding respectively. The results reportedly show no coding performance change.

Table 2. Lossy results of the proposed method.

# Table 3. Lossless results of the proposed method.

# Conclusions

This contribution proposes to fetch cMax for palette_escape_val from a look-up table directly instead of calculating it. No coding performance change is reported by applying the proposed method.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- R. Joshi, J. Xu, “HEVC Screen Content Coding Draft Text 2,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1005, Oct. 2014.

- SCM-3.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2+SCM-3.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1015, Oct. 2014.