| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0061 |
| --- | --- |

| Title: | Non-CE: Efficient Coding for Static CTUs of Screen Content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weinong Huang David Wang Sean Chen Yong Yan | Email: Email:Email:Email: | Weinong.Huang@polycom.comDavid.Wang@polycom.com Sean.Chen@polycom.com Yong.Yan@polycom.com |
| Source: | Polycom, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a new coding method for the static coding tree units (CTUs) located in the inter-prediction frames that show no change from its co-located area in reference frames. The proposed method represents one static CTU with one flag, and code the flag in one bin in CABAC process. Further, if all CTUs in one slice are static CTUs with same reference frame, such a slice is defined as a static slice and is coded as 1-bit flag in the slice segment header. One static slice won’t carry any payload except the slice segment header.

It is reported that the proposed approach achieves solid B-D rate reduction for lossy low-delay coding of scenarios related with text and graphic content. For example, for YUV 4:4:4 cases, it achieves -0.5% B-D rate reductions for the mixed YUV content at 1080p & 1440p, and -0.4% rate reduction for the text & graphic content with motion at 720p & 1080p. For 4:2:0 cases, it achieves -0.6% B-D rate reduction for the text & graphic with motion at 720p. For the worst case, the proposed method could cause 0.1% B-D rate increase for lossy LB and RA coding of YUV 4:4:4 animation and camera capture sequences. Also, as expected, the B-D rate for all-intra tests was not changed.

It should also be noted that the proposed approach demonstrates a better performance in low bit-rate scenarios which are typical in video conferencing applications or other real-time video communications. For example, in some Non-Common conditions tests, the proposed approach achieves around -1.5% B-D rate reductions for lossy LB coding of the text & graphic content with motion at 720p & 1080p.

# Introduction

If a CTU has no change from its reference frame and the reconstruction of that CTU is just a copy from its co-located area in reference frames, such a CTU is a static CTU. Most screen contents contain a lot of static CTUs. In H.265 SCC, one static CTU is always coded as the skip mode, which needs one split flag, one skip flag and one merge index. So one static CTU needs to code at least 3 bins in CABAC process, and it could code up to 6 bins if multiple merge candidates are available. If one CTU is on the edge of the picture and only part of CTU is within that picture, that CTU is called an edge CTU. One edge CTU has a minimum split depth of 1. The encoder needs more bins to code one static edge CTU. Further, if all CTUs in one slice are static CTUs and all CTUs have same reference frame, that slice is a static slice and it needs to repeatedly code similar bin-pattern for all CTUs. There is redundancy in these CABAC bins for static CTU and static slice.

Since one static CTU can be reconstructed from its co-located area in reference frame directly and all coding information of that CTU has been self-implied, it could be more efficient to represent the static CTU with one flag and use 1 bin to code the flag. Similarly, if one slice is a static slice and all CTUs have same bin-pattern, that slice can be coded as 1-bit flag in the slice segment header. One static slice doesn’t contain any payload for CTUs except the slice segment header.

Based on this finding, it is proposed to define a new coding method for static CTUs of screen content. The new coding method defines 2 new flags: static_ctu_flag in the syntax of CTU, and static_slice_flag in the syntax of slice segment header. The static_ctu_flag indicates if current CTU is a static CTU and the static_slice_flag indicates if current slice is a static slice.

# Static CTU and Static Slice

As defined, one static CTU is one copy of the co-located CTU in the reference frame. That implies that the motion vector and coding residual of one static CTU are 0. It’s actually a constrained skip CTU. When multiple reference frames are present, different static CTUs could have different reference frames. In order to code these static CTUs efficiently in H.265 SCC, more restrictions are applied to static CTU.

In the proposed method, one CTU can be coded as a Static CTU when that CTU meet all the following conditions:

- For non-edge CTUs, the split depth of CTU is 0.  For edge CTUs, no further split for all CUs within edge CTUs.

- The prediction mode is MODE_INTER.

- The prediction residual is 0 for both Luma and Chroma.

- All motion vectors are 0.

- The prediction partition size is 2Nx2N.

- The reference frame index is always 0 for both P slice and B slice.

- The prediction direction is always 0 (Uni_Direction_0) for P slice and 2 (Bi_Direction) for B slice.

If all CTUs in one slice can be coded as static CTUs, that slice is one static slice. One static slice will disable the de-blocking filter and SAO filter within the slice because these operations have been done for each CTU in the reference frame. It’s not necessary to repeat these operations in static slice.

# Proposed encoder operations

The proposed coding method requires detecting static CTUs and setting the static CTU flag properly before the CABAC process of each CTU in the encoder. The detection of static CTU can be done either in the mode-decision process or after the mode-decision process. Since one static CTU is also a skipped CTU with restrictions, the detection of static CTU has been done after the mode-decision process of each CTU in the modified SCM3.0 encoder for the proposed coding method. The detection function is lightweight and it contains only some logic comparison operations for one static CTU. When starting the CABAC process of one CTU, the static_ctu_flag is coded first. If the flag is true, the coding of current CTU is done.

If one static slice is detected, the CABAC process of each CTU is skipped and the encoder only needs to code the slice segment header with some extra information: one flag to indicate that the current slice is a static slice, and a variable to show the size of the slice.  If one frame has only one slice, that variable of slice size could be just one flag. It should also be noted that one static slice will disable the de-blocking filter and SAO filter by default and it doesn’t need to transmit these parameters related to de-blocking filter and SAO filter in the slice segment header.

Since the detection and encoding of one static CTU don’t require reloading or accessing the pixel data of one CTU, the proposed coding method won’t introduce any extra memory bandwidth. What’s more, since the proposed method is not involved in the mode-decision process, it won’t affect the PSNR for each static CTU if the slice is not a static slice.

# Proposed syntax changes

The proposed coding method requires some syntax changes in H.265 SCC. It adds several flags to SPS SCC extension section, slice segmentation header, and CTU syntax.

In SPS SCC extension section, one flag, static_ctu_coding_enabled_flag, is added to indicate if Static CTU coding is enabled for current sequence. The syntax change is as follows:

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| static_ctu_coding_enabled_flag | u(1) |
| } |  |

In CTU syntax, one extra flag, static_ctu_flag, is added to indicate if current CTU is one static CTU. If it’s, coding of current CTU ends. If not, go to normal coding of one CTU. The syntax changes are as follows:

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> CtbLog2SizeY, yCtb >> CtbLog2SizeY ) |  |
| if(static_ctu_coding_enabled_flag && slice_type != I ) |  |
| static_ctu_flag | ae(v) |
| if( !static_ctu_flag ) |  |
| coding_quadtree( xCtb, yCtb, CtbLog2SizeY, 0 ) |  |
| } |  |

In slice segment header, a few more flags are added in order to represent one static slice properly.  One flag (static_slice_flag) is used to indicate if current slice is a static slice. If that flag is true, another flag, slice_is_full_frame_flag, is used to indicate if current slice is also a full frame.  If that flag is false, one variable is used to indicate the size of current slice. The syntax changes are as follows:

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … | u(1) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( static_ctu_coding_enabled_flag ) { |  |
| static_slice_flag | u(1) |
| if( static_slice_flag ) { |  |
| slice_is_full_frame_flag | u(1) |
| if( ! slice_is_full_frame_flag ) |  |
| count_of_ctus_in_slice | u(v) |
| } |  |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| if( !static_slice_flag ) { |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| … |  |
| if( pps_loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_deblocking_filter_disabled_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } else { |  |
| if( lists_modification_present_flag && NumPicTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( chroma_qp_offset_list_enabled_flag ) |  |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| } |  |
| … |  |

# Test Results

## Results for lossy coding of YUV4:4:4 and RGB contents

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.4% | -0.4% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.5% | -0.5% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 98% |  |  |

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 98% |  |  |

## Results for lossless coding of YUV4:4:4 and RGB contents

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.4% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.5% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 96% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |  |
| Dec Time[%] | 96% |  |  |  |

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  |  |
| Dec Time[%] | 98% |  |  |  |

## Results for lossy coding of YUV4:2:0 contents

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | -0.6% | -0.5% | -1.2% |
| Mixed content, 480p | 0.1% | 0.4% | 0.0% |
| Animation, 768p | 0.1% | 0.1% | 0.0% |
| Average of all sequences | -0.2% | -0.1% | -0.6% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 95% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% |
| Mixed content, 480p | 0.1% | 0.1% | 0.1% |
| Animation, 768p | 0.1% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |

## Results for lossless coding of YUV4:2:0 contents

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 96% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 98% |  |  |  |

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

## Results for lossy low-delay low bit-rate coding of contents with Non-Common Conditions (QP Set: 34, 37, 40 and 43)

|  | Low Delay B |  |  |
| --- | --- | --- | --- |
| YUV Sequences | G/Y | B/U | R/V |
| sc_web_browsing_1280x720_30_8bit_300_444 | -1.6% | -1.6% | -1.5% |
| sc_video_conferencing_doc_sharing_1280x720_30_8bit_300_444 | -1.4% | -1.2% | -1.3% |
| sc_SlideShow_1280x720_20_8bit_500_444 | -0.6% | -1.0% | -1.8% |
| sc_cad_waveform_1920x1080_20_8bit_200_444 | -2.2% | -2.2% | -2.2% |
| sc_desktop_1920x1080_60_8bit_444 | -0.4% | -0.4% | -0.4% |
| sc_ppt_doc_xls_1920x1080_20_8bit_200_444 | -2.5% | -1.9% | -3.3% |
| MissionControlClip3_1920x1080_60p_8b_444 | -0.9% | -0.9% | -0.8% |
| MissionControlClip2_2560x1440_60p_8b_444 | -0.3% | -0.7% | -0.1% |
|  | Low Delay B |  |  |
| RGB Sequences | G/Y | B/U | R/V |
| sc_web_browsing_1280x720_30_8bit_300_rgb | -1.0% | -1.1% | -1.0% |
| sc_video_conferencing_doc_sharing_1280x720_30_8bit_300_rgb | -1.1% | -0.9% | -1.0% |
| sc_SlideShow_1280x720_20_8bit_500_rgb | -0.5% | -0.7% | -0.4% |
| sc_cad_waveform_1920x1080_20_8bit_200_rgb | -2.0% | -2.1% | -2.1% |
| sc_desktop_1920x1080_60_8bit_rgb | -0.5% | -0.5% | -0.5% |
| sc_ppt_doc_xls_1920x1080_20_8bit_200_rgb | -2.5% | -2.3% | -2.3% |
| MissionControlClip3_1920x1080_60p_8b_rgb | -0.7% | -0.6% | -0.6% |
| MissionControlClip2_2560x1440_60p_8b_rgb | -0.1% | -0.1% | -0.2% |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.3% | -1.3% | -1.2% |
| RGB, mixed content, 1440p & 1080p | -0.4% | -0.4% | -0.4% |
| YUV, text & graphics with motion, 1080p & 720p | -1.5% | -1.4% | -1.8% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -0.8% | -0.4% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |

# Patent rights declaration(s)

Polycom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).