| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0065r2 |
| --- | --- |

| Title: | Non CE1: Grouping Palette Indices At Front |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Wei Pu, Rajan Joshi, Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-7749 martak@qti.qualcomm.com Wei.Pu@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes to signal the palette mode syntax element palette_index_idc in the CU all together before run length coding. Compared with SCM3.0 anchor under SCC common test condition, the proposed method does not affect coding efficiency while improving CABAC throughput.

# Introduction

In context-based adaptive binary arithmetic coding (CABAC) [6], which is used in H.264 and H.265, there are two modes. One of them is named as bypass mode, and the other one is named as context mode. In bypass mode, there is no context update process. Therefore, it can achieve higher data throughput than context mode by exploiting hardware or ISA level parallelism. Such benefit grows if a bunch of bypass bins can be processed together.

In the current palette mode design Error: Reference source not found in screen content coding standard, the syntax elements of palette_index_idc which are CABAC bypass coded are interleaved with other syntax elements such palette_run_msb_id_plus1 which are CABAC context coded. This contribution proposes a method of grouping all of the occurrences of palette_index_idc at front.

# Technical Description

With the proposed method of grouping palette indices at the front, the palette mode syntax table is updated as:

| ... |  |
| --- | --- |
| scanPos = 0 |  |
| indices_idc_coding() (Relocate all occurrence of palette_index_idc to here) |  |
| while( scanPos < nCbS * nCbS ) { |  |
| … |  |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE && adjustedIndexMax > 0 ) |  |
| palette_index_idc | ae(v) |
| … |  |

The function of indices_idc_coding() can be specified as:

| indices_idc_coding() { |  |
| --- | --- |
| num_palette_index | ae(v) |
| for (i = 0; i < num_palette_index; i++) |  |
| palette_index_idc | ae(v) |
| palette_last_run_type_flag | ae(v) |
| } |  |

num_palette_index specifies the number of palette index signalled in the current block. The difference between this value and indexMax [1] is coded into the bitstream using HEVC coefficient coding method, i.e. by calling the function of xWriteCoefRemainExGolomb() in HM with parameter equal to 2+indexMax/6. If negative value occures, they are mapped to positive values with equal spacing of 32 in non palette share mode, and 8 in palette share mode.

palette_last_run_type_flag specifies the value of the last occurence of palette_last_run_type_flag.

The proposed method is only enabled when the variable indexMax is greater than 0 because otherwise there is no necessarity of grouping. The coding of last paletteRun can be bypassed when the run type equals to palette_last_run_type_flag and the number of occurred palette_index_idc equals to num_palette_index.

Also, the coding of palette_run_type_flag can be bypassed when the number of occurred palette_index_idc equals to num_palette_index or the current position is the last one in the block.

# Simulation Results

Simulations are carried out under the test condition specified in CE1 description [3] and SCC common test condition [2]. Only AI lossy results are presented in this section. The complete test results are attached in the proposal package. In the test, the encoder fix method in [4] and the palette sharing removal method M1 in [5] are enabled.

Table 1. Proposed Method v.s. SCM3.0 Anchor

# Conclusion

In this document, a method of grouping the CABAC bypass coded syntax element palette_index_idc is proposed. Simulation results indicate that this method does not affect coding efficiency while  improving throughput.

# References

- R. Joshi and J. Xu, “HEVC screen content coding draft text 2,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting, JCTVC-S1005, Strasbourg, FR, 17–24 Oct. 2014.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting, JCTVC-S1015, Strasbourg, FR, 17–24 Oct. 2014.

- P. Lai, P. Onno, R. Cohen, V. Seregin, X. Xiu, and Z. Ma, “Description of core experiment 1 (CE1): Improvements of palette mode,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting, JCTVC-S1101, Strasbourg, FR, 17–24 Oct. 2014.

- W. Pu, R. Joshi, M. Karczewicz, V. Seregin, and F. Zou, “Non CE1: Palette mode syntax, codeword, and encoder fixes,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting, JCTVC-T0063, Geneva, CH, 10–18 Feb. 2015.

- M. Karczewicz, W. Pu, R. Joshi, V. Seregin, and F. Zou, “Non CE1: Comments on palette sharing,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting, JCTVC-T0064, Geneva, CH, 10–18 Feb. 2015.

- D. Marpe, H. Schwarz, and T. Wiegand, “Context-based adaptive binary arithmetic coding in the H.264/AVC video compression standard,” in IEEE Trans. Cir. & Sys. Video Tech., vol. 13, no. 7, July 2003.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).