| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0066r1 |
| --- | --- |

| Title: | CE1-related: Bypassing the context-based coding for CU-level transpose flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Chun-Lung Lin, 3Ching-Chieh Lin, Jih-Sheng Tu, and Chao-Hsiung Hung 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2Chunlung@itri.org.tw 3JackLin@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

A transpose flag is proposed in SCM 3.0 to enable the horizontal scan mode or the vertical scan mode. For the transpose flag, a context-based coding is used for further compression. This contribution proposes to bypass the context-based coding for the transpose flag. It is able to reduce one context bin. When the bypassing method is implemented on SCM 3.0 reference software, the results show that there is no coding loss under the conditions of AI-lossy coding.

# Introduction

Within the palette mode, a context-based coding method for the transpose flag is used as follows:

| Table 9-1 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| palette_transpose_flag | 0, 1 (subclause Error: Reference source not found) | na | na | na | na | na |

It would require one context bin for the coding. This contribution proposes to bypass the context-based coding of the CU-level transpose flag for the improvement on the complexity.

# Proposed coding

To bypass the context-based coding of the CU-level transpose flag, the table for the context bin in the working draft is modified as follows:

| Table 9-2 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| palette_transpose_flag | bypass | na | na | na | na | na |

# Simulation results

The proposed method is simulated on top of SCM 3.0 reference software under the common test condition [1]. Table 1 shows the lossy results, where the anchor is SCM 3.0. The results show that there is no coding loss with the reduction of context bin. Note that the coding time is not reliable for RA and LB results. Coding time for RA and LB is not reliable due to running on different clusters.

Figure 1. Lossy results

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.0% |
| YUV, Animation, 720p | -0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.2% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

# Conclusion

This contribution proposes to bypass the context-based coding for the transpose flag. The results show that there is no coding loss with one-context-bin reduction. It is recommended to adopt this contribution for the simplicity of the CU-level transpose flag coding.

# Reference

- P. Lai, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma, “Description of Core Experiment 1 (CE1): Improvements of palette mode,” 19th JCT-VC meeting, JCTVC-S1101, Strasbourg, France, Oct. 2014.

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)