| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0067 |
| --- | --- |

| Title: | CE1-related: Bypassing the context-based coding for CU-level share flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Chun-Lung Lin, 3Ching-Chieh Lin, Chao-Hsiung Hung and Jih-Sheng Tu 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2Chunlung@itri.org.tw 3JackLin@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

A CU-level share flag is proposed in SCM 3.0 to enable the sharing mode. For the CU-level share flag, a context-based coding is used for further compression. This contribution proposes to bypass the context-based coding for the CU-level share flag. It is able to reduce one context coded bin. When the bypassing method is implemented on SCM 3.0 reference software, the results show that there is no coding loss under the conditions of AI-lossy coding.

# Introduction

Within the palette mode, there are three main CU-level flags including the sharing flag, escape-value flag and transpose flag. The context-based coding for three flags in SCM 3.0 can be as described as follows:

| Table 9-1 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| palette_share_flag | 0 | na | na | na | na | na |
| palette_escape_val_present_flag | bypass | na | na | na | na | na |
| palette_transpose_flag | 0, 1 (subclause Error: Reference source not found) | na | na | na | na | na |

To verify the necessity of the context bin for the share flag, this contribution bypasses the context-based coding for the share flag.

# Proposed coding

To bypass the context-based coding of the CU-level share flag, the table for the context bin in the working draft is modified as follows:

| Table 9-2 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| palette_share_flag | bypass | na | na | na | na | na |
| palette_escape_val_present_flag | bypass | na | na | na | na | na |
| palette_transpose_flag | 0, 1 (subclause Error: Reference source not found) | na | na | na | na | na |

In JCTVC-T0066, a bypassing of the context based coding for the CU-level transpose flag is proposed. Here we further combine the proposed bypassing with the bypassing in JCTVC-T0066. The working draft is modified as follows:

| Table 9-3 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| palette_share_flag | bypass | na | na | na | na | na |
| palette_escape_val_present_flag | bypass | na | na | na | na | na |
| palette_transpose_flag | bypass | na | na | na | na | na |

# Simulation results

The bypassing of the context-based coding for the CU-level share flag is simulated on top of SCM 3.0 reference software. Table 1 shows the AI lossy results under the common test condition [1]. The results show that there is no coding loss with the reduction of one context coded bin. Table 2 further shows the AI lossy results of the combined bypassing proposed in this contribution and JCTVC-T0066. The results show no coding loss with the reduction of two context coded bins.

Table 1. AI lossy results for the bypassing of the context coding for the share flag

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2. AI lossy results for the combined bypassing

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

This contribution proposes to bypass the context-based coding for the sharing flag. The results show that there is no coding loss with one-context-bin reduction. It also shows the results of the combined bypassing of the context coding for the share flag and the transpose flag also have no coding loss with two-context-bin reduction. It is recommended to adopt this contribution for the improvement on the CABAC throughput.

# Reference

- P. Lai, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma, “Description of Core Experiment 1 (CE1): Improvements of palette mode,” 19th JCT-VC meeting, JCTVC-S1101, Strasbourg, France, Oct. 2014.

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)