| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0069 |
| --- | --- |

| Title: | High-level syntax refinement for adaptive motion vector resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan | Tel: Email: | +1-425-703-5308 garysull@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution proposes a modification of the SPS-level syntax for the slice-level syntax of adaptive motion vector resolution control (as recently adopted from JCTVC-S0085). The lower-level functionality is not proposed to be modified. The change is asserted to be a small correction for a forgotten aspect of what was previously proposed in JCTVC-P0277 – specifically, using a three-valued syntax element at the SPS level rather than just a flag at that level, so that the slice-level control flag can be omitted when all slices in the CVS will operate in the same way – regardless of whether that is to use quarter-pel MVs or full-pel MVs.

A two-bit mode indicator is proposed for the SPS level, namely motion_vector_resolution_control_idc, in place of the current adaptive_mv_resolution_enabled_flag. When the mode is 0 or 1, the slice-level use_integer_mv_flag is inferred to be equal to motion_vector_resolution_control_idc, and when the mode is 2, the use_integer_mv_flag is sent explicitly in the slice header. This saves one bit per slice header in the case where MVs are always in full-pel precision.

# Discussion

The abstract above seems like an adequate discussion of this proposal; nothing further seems necessary. The functionality is unaffected, and the text impact is trivial. This would save one bit per slice header when operating in the third mode, which is deemed mode 1 here to make the text description simple.

The proposal is suggested to be a high-level syntax cleanup/bug-fix rather than really being a coding efficiency proposal, to avoid sending a redundant flag in every slice header when the flag value will never change (either because the encoder is aware that the type of content it is encoding is unlikely to benefit from quarter-pel motion, or because the MV search is simpler for not using quarter-pel motion). However, the net effect is to use one more bit in the SPS to enable saving one bit per slice header in the all-integer mode. Experiment results are not provided, as the characteristics seem obvious.

# Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).