| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0075 |
| --- | --- |

| Title: | CE2 Test 7: Combination of CE2 Test 1 and Test 3.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Tzu-Der Chuang Shan Liu Shawmin Lei | Email: | xiaozhong.xu@mediatek.com peter.chuang@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document proposes to add default BVs to the Imerge candidate list. In this proposal, the syntax structure is the same as in CE2 test 1, the changes made is to add some preset values as additional Intra BC merge candidates after the 1st zero merge candidate. The experimental results show that the proposed method brings on average 1.2%, 2.5% and 2.4% bitrate saving as against SCM3.0 anchor for RGB TGM 1080p & 720p in AI, RA and LB lossy coding, respectively; the proposed method brings on average 1.5%, 2.8% and 2.6% bitrate saving as against SCM3.0 anchor for YUV TGM 1080p & 720p in AI, RA and LB lossy coding, respectively. Performance gain is also observed for the other cases.

# Introduction

In CE 2 [1] test 1, the merge candidate is derived using spatial/temporal neighboring coded blocks. If merge mode is used for the current prediction block and the referred candidate block (by merge index) is an Intra BC coded one, the current prediction block is also coded in Intra BC mode. The BV from the referred Intra BC coded block will also be used for the current prediction block. If the referred block is an inter coded block, the current prediction block will also be an inter coded block, as described in HEVC v1.

# Proposed methods

It is proposed in this contribution that default BVs are inserted in to the following position of the merge list construction:

- Merge candidate list construction:

  - Spatial and temporal merge candidates

  - Bi-candidates, e.g. (L0_of_cand_0, L1_of_cand_1)

  - Zero-candidate with refIdx = 0, e.g. (0,0) refIdx = 0

  - Default BVs of (-w,0), (-2w,0), (0,-h), (0,-2h), (-w,-h)

  - Zero-candidates with different refIdx, e.g. (0,0) refIdx = 1, 2, …

  - Zero-candidates with refIdx = 0, e.g. (0,0) refIdx = 0

In the above, w and h are the current CU width and height.

For the BV = (BV_x, BV_y) from each Intra BC merge candidate, if it points to some overlapped area with the current CU, it is clipped as follows:

diff_x = CU_x – PU_x – BV_x – PU_Width;

diff_y = CU_y – PU_y – BV_y – PU_Height;

if( diff_x < 0 && diff_y < 0 )

{

if( diff_x >= diff_y )

{

BV_x = CU_x - PU_x - PU_Width;

}

else

{

BV_y = CU_y - PU_y - PU_Height;

}

}

Where in the above, CU_x and CU_y are the top left pixel position of the current CU; PU_x and PU_y are the top left pixel position of the current PU; PU_Width and PU_Height are the width and height of the current PU.

# Experimental results

HM-16.2+ SCM-3.0 default coding structure is used as the anchor. The common test conditions specified in Error: Reference source not found are followed. The summarized experimental results of test 7 for 444 lossy and lossless coding are shown in Table 1 and 2, respectively. The summarized experimental results of test 7 for 420 lossy and lossless coding are shown in Table 3 and 4, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for 444 lossy coding in test 7

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.2% | -2.7% | -2.6% |
| RGB, mixed content, 1440p & 1080p | -0.8% | -1.4% | -1.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.5% | -2.9% | -3.0% |
| YUV, mixed content, 1440p & 1080p | -1.2% | -2.1% | -2.2% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.5% | -4.4% | -4.4% |
| RGB, mixed content, 1440p & 1080p | -1.0% | -2.4% | -2.6% |
| RGB, Animation, 720p | -0.1% | 0.0% | -0.2% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.2% |
| YUV, text & graphics with motion, 1080p & 720p | -2.8% | -4.6% | -4.7% |
| YUV, mixed content, 1440p & 1080p | -1.4% | -3.3% | -3.4% |
| YUV, Animation, 720p | -0.2% | -0.5% | -0.3% |
| YUV, camera captured, 1080p | -0.2% | 0.0% | -0.4% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 93% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.4% | -4.1% | -3.9% |
| RGB, mixed content, 1440p & 1080p | -1.3% | -2.7% | -2.9% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.6% | -4.1% | -4.3% |
| YUV, mixed content, 1440p & 1080p | -1.6% | -4.0% | -4.3% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 92% |  |  |

Table 2 Experimental results for 444 lossless coding in test 7

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.2% | 0.0% | -1.5% | 1.9% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.4% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.9% | 0.8% | -0.8% | 2.6% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 0.2% | -0.2% | 0.5% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.4% | -0.8% | -2.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.3% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.3% | -1.7% | 0.3% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 94% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.4% | -0.9% | -3.5% | 0.0% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.2% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.5% | -3.1% | 0.2% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 97% |  |  |  |

Table 3 Experimental results for 420 lossy coding in test 7

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | -3.0% | -3.5% | -3.5% |
| Mixed content, 480p | -1.3% | -1.2% | -1.5% |
| Animation, 768p | -1.0% | -1.8% | -2.2% |
| Average of all sequences | -2.1% | -2.5% | -2.7% |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | -2.7% | -3.8% | -3.8% |
| Mixed content, 480p | -0.5% | -1.2% | -1.5% |
| Animation, 768p | -0.8% | -2.7% | -2.7% |
| Average of all sequences | -1.7% | -2.9% | -2.9% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | -2.0% | -2.3% | -4.2% |
| Mixed content, 480p | 0.3% | 0.4% | -0.4% |
| Animation, 768p | -0.2% | -0.4% | -1.7% |
| Average of all sequences | -1.0% | -1.1% | -2.7% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 93% |  |  |

Table 4 Experimental results for 420 lossless coding in test 7

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | -0.7% | -0.7% | -0.8% | -0.6% |
| Mixed content, 480p | -0.1% | -0.1% | -0.1% | -0.1% |
| Animation, 768p | 0.1% | 0.1% | 0.1% | 0.1% |
| Average of all sequences | -0.2% | -0.3% | -0.8% | 0.1% |
| Enc Time[%] | 80% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | -0.5% | -0.8% | -1.2% | -0.4% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | -0.1% | -0.4% | -1.2% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | -0.2% | -0.4% | -0.7% | -0.1% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | -0.2% | -0.7% | 0.0% |
| Enc Time[%] | 96% |  |  |  |
| Dec Time[%] | 97% |  |  |  |

# Conclusions

This contribution proposes to add some default BVs as additional Intra BC merge candidates for HEVC merge mode. Additional BD rate reduction on top of Intra BC merge mode in CE2 Test 3.1 is observed.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCTVC-S1102, “Description of Core Experiment 2 (CE2): Intra block copy relationship to inter coding,” Strasbourg, FR, 17–24 Oct. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1015, Strasbourg, FR, 17–24 Oct. 2014.