| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0077 |
| --- | --- |

| Title: | CE1-related: Improvements of Draft Text on palette index |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai, Jungsun Kim, Shan Liu, and Shawmin Lei 2860 Junction Ave, San Jose, CA, USA | Email: | { polin.lai, jungsun.kim, shan.liu, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents Draft Text modifications to consolidate redundant condition checks in the palette index part. Modified syntax table, and the patch to align the software with the modified syntax table, are both provided. This modification does not change the bitstream.

This document also presents naming unifications related to palette run copy modes.

# Introduction

In the current HEVC Screen Content Coding Draft Text 2 [1], for parsing each palette index run, there exist redundant condition checks. The syntax is provided in the next page.

As can be seen, for each run of palette index:

- First, check indexMax to determine whether to parse palette_run_type_flag.

- Then, determine whether to parse palette_index_idc using a derived variable adjustedIndexMax, for which the derivation requires its own condition check, see the excerpted text below.

- Finally, whether to parse palette run is determined again by checking indexMax.

From HEVC Screen Content Coding Draft Text 2 [1]:

The variable indexMax specifies the maximum possible value for a palette index for the current coding unit. If palette_escape_val_present_flag is equal to 1, indexMax is set equal to the currentPaletteSize. If palette_escape_val_present_flag is equal to 0, indexMax is set equal to the (currentPaletteSize – 1).

The variable adjustedIndexMax is derived as follows:

adjustedIndexMax = indexMax
if( scanPos > 0  ) 
	adjustedIndexMax − = 1

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| … |  |
| … |  |
| scanPos = 0 |  |
| while( scanPos < nCbS * nCbS ) { |  |
| xC = x0 + travScan[ scanPos ][ 0 ] |  |
| yC = y0 + travScan[ scanPos ][ 1 ] |  |
| if( scanPos > 0) { |  |
| xcPrev = x0 + travScan[ scanPos − 1 ][ 0 ] |  |
| ycPrev = y0 + travScan[ scanPos − 1 ][ 1 ] |  |
| } |  |
| if( indexMax > 0 && scanPos > = nCbS && palette_run_type_flag[ xcPrev ][ ycPrev ] ! = COPY_ABOVE_MODE ) { |  |
| palette_run_type_flag[ xC ][ yC ] | ae(v) |
| } |  |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE && adjustedIndexMax > 0) |  |
| palette_index_idc | ae(v) |
| if( indexMax > 0 ) { |  |
| maxPaletteRun = nCbS * nCbS – scanPos – 1 |  |
| if( maxPaletteRun > 0 ) { |  |
| palette_run_msb_id_plus1 | ae(v) |
| if( palette_run_msb_id_plus1 > 1 ) |  |
| palette_run_refinement_bits | ae(v) |
| } |  |
| } else |  |
| paletteRun = nCbS * nCbS − 1 |  |
| runPos = 0 |  |
| while ( runPos < = paletteRun ) { |  |
| xR = x0 + travScan[ scanPos ][ 0 ] |  |
| yR = y0 + travScan[ scanPos ][ 1 ] |  |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE && paletteIndex = = indexMax ) { |  |
| PaletteSampleMode[ xR ][ yR ] = ESCAPE_MODE |  |
| PaletteIndexMap[ xR ][ yR ] = paletteIndex |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| palette_escape_val | ae(v) |
| PaletteEscapeVal[ cIdx ][ xR ][ yR ] = palette_escape_val |  |
| } |  |
| } else if(palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE ) { |  |
| PaletteSampleMode[ xR ][ yR ] = COPY_INDEX_MODE |  |
| PaletteIndexMap[ xR ][ yR ] = paletteIndex |  |
| } else { |  |
| PaletteSampleMode[ xR ][ yR ] = COPY_ABOVE_MODE |  |
| PaletteIndexMap[ xR ][ yR ] = PaletteIndexMap[ xR ][ yR − 1 ] |  |
| } |  |
| runPos++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |

# Proposed methods

## Consolidated conditional checks

It is proposed to condition the avoidance of signalling palette run type, palette index, and palette run, with simply one conditional check. The proposed syntax table is as below.

- Only a single conditional check on indexMax will determine whether to omit palette_run_type_flag, palette_index_idc, and palette run.

- If indexMax is 0, there is no need to even derive adjustedIndexMax.

## Naming unifications related to copy modes of palette run

In the Draft Text 2 [1], the syntax element palette_run_type_flag is used to indicate the subsequent palette run to be COPY_ABOVE_MODE or COPY_INDEX_MODE. This contribution proposes to unify the naming between “type” and “mode” for consistency.

- Suggestion 1: palette_run_mode_flag, COPY_ABOVE_MODE, and COPY_INDEX_MODE

# Experimental results

(For Section 2.1) Currently in SCM-3.0 [2], the parsing and decoding follows the multiple condition checks as specified in the current HEVC SCC Draft Text 2 [1]. A software patch is provided in this contribution to align the software with the proposed Draft Text change. The bitstream remains unchanged with the proposed modification.

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| … |  |
| … |  |
| scanPos = 0 |  |
| while( scanPos < nCbS * nCbS ) { |  |
| xC = x0 + travScan[ scanPos ][ 0 ] |  |
| yC = y0 + travScan[ scanPos ][ 1 ] |  |
| if( scanPos > 0) { |  |
| xcPrev = x0 + travScan[ scanPos − 1 ][ 0 ] |  |
| ycPrev = y0 + travScan[ scanPos − 1 ][ 1 ] |  |
| } |  |
| if( indexMax > 0 ) { |  |
| if( indexMax > 0 && scanPos > = nCbS && palette_run_type_flag[ xcPrev ][ ycPrev ] ! = COPY_ABOVE_MODE ) { |  |
| palette_run_type_flag[ xC ][ yC ] | ae(v) |
| } |  |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE && adjustedIndexMax > 0) |  |
| palette_index_idc | ae(v) |
| if( indexMax > 0 ) { |  |
| maxPaletteRun = nCbS * nCbS – scanPos – 1 |  |
| if( maxPaletteRun > 0 ) { |  |
| palette_run_msb_id_plus1 | ae(v) |
| if( palette_run_msb_id_plus1 > 1 ) |  |
| palette_run_refinement_bits | ae(v) |
| } |  |
| } else |  |
| paletteRun = nCbS * nCbS − 1 |  |
| runPos = 0 |  |
| while ( runPos < = paletteRun ) { |  |
| xR = x0 + travScan[ scanPos ][ 0 ] |  |
| yR = y0 + travScan[ scanPos ][ 1 ] |  |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE && paletteIndex = = indexMax ) { |  |
| PaletteSampleMode[ xR ][ yR ] = ESCAPE_MODE |  |
| PaletteIndexMap[ xR ][ yR ] = paletteIndex |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| palette_escape_val | ae(v) |
| PaletteEscapeVal[ cIdx ][ xR ][ yR ] = palette_escape_val |  |
| } |  |
| } else if(palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE ) { |  |
| PaletteSampleMode[ xR ][ yR ] = COPY_INDEX_MODE |  |
| PaletteIndexMap[ xR ][ yR ] = paletteIndex |  |
| } else { |  |
| PaletteSampleMode[ xR ][ yR ] = COPY_ABOVE_MODE |  |
| PaletteIndexMap[ xR ][ yR ] = PaletteIndexMap[ xR ][ yR − 1 ] |  |
| } |  |
| runPos++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- R. Joshi, J. Xu, “HEVC Screen Content Coding Draft Text 2,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1005, Oct. 2014.

- SCM-3.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-14.0+RExt-7.0+SCM-3.0/