| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0078 |
| --- | --- |

| Title: | CE1-related : simplification for index map coding in palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, PoLin Lai, Shan Liu, Shawmin Lei 2860 Junction Ave, San Jose, CA, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { jungsun.kim,polin.lai, shan.liu, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposed to remove one context for run_type coding and one condition check for index redundancy removal. Therefore, the context of run_type coding is fixed as 0 always and index redundancy removal is applied only to the pixels whose previous pixel has copy index mode. Experiment results show average 0.0%, 0.0%, 0.0% loss under AI, RA and LB configuration respectively for first change, 0.0%, 0.0%, 0.1% loss under AI, RA and LB configuration respectively for the second change and 0.1%, 0.1%, 0.0% loss under AI, RA and LB configuration respectively for both changes.

# Problem statement

The palette mode in SCM-3.0 signalls run_type, palette index, number of run and escape values if necessary for index map coding. For run_type indicates current pixel and consecutive ‘number of run’ pixels are predicted copy above or copy index, 2 contexts are used as follows:

uiCtx=(yC==0)?0:runtype[xC][yC−1]

Where xC and yC indicate the location of current pixel.

For palette index which indicates the index of current pixel and consecutive ‘number of run’ pixels, index redundancy removal is applied. The index of current pixel(red pixel in Figure 1) cannot be same as previous pixel’s(grey pixel in Figure 1) if the previous pixel has copy index mode (Figure 1 (a)). Also the index of current pixel(red pixel in Figure 1) cannot be same as above pixel’s(grey pixel in Figure 1) if the previous pixel has copy above mode (Figure 1 (a)).

| (a) previous pixel has copy index mode | (b) previous pixel has copy above mode |
| --- | --- |

Figure 1 index redundancy removal

With this property the number of index candidate can be reduced by one if reference pixel is not ESCAPE.

# Proposed methods

## Fixed Context for run_type coding

It is proposed to fix the context of run_type to 0 to avoid referring above line’s run_type every time. I.e., the context for run_type is as follows :

uiCtx=0

WD is changed as follows:

| palette_run_type_flag | 0, 1 (subclause Error: Reference source not found) | na | Na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |

| Syntax element | condL | condA | ctxInc |
| --- | --- | --- | --- |
| split_cu_flag[ x0 ][ y0 ] | CtDepth[ xNbL ][ yNbL ] > cqtDepth | CtDepth[ xNbA ][ yNbA ] > cqtDepth | ( condL && availableL ) + ( condA && availableA ) |
| cu_skip_flag[ x0 ][ y0 ] | cu_skip_flag[ xNbL ][ yNbL ] | cu_skip_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| palette_transpose_flag [ x0 ][ y0 ] | palette_transpose_flag [ xNbL ][ yNbL ] | na | ( condL && availableL ) |
| palette_run_type_flag [ x0 ][ y0 ] | na | palette_run_type_flag [ xNbA ][ yNbA ] | ( condA && availableA ) |

## Limited index redundancy removal

It is proposed to apply index redundancy removal only to the pixels whose previous pixel has copy index mode (Figure 1 (a)) to avoid referring above lines index.

WD is changed as follows:

palette_index_idc is an indication of an index to the array represented by currentPaletteEntries. The value of palette_index_idc shall be in the range of 0 to adjustedIndexMax, inclusive. When palette_index_idc is not present, it is inferred to be equal to 0. The variable PaletteIndexMap[ xC ][ yC ] specifes a palette index, which is an index to the array represented by CurrentPaletteEntries. The array indices xC, yC specify the location ( xC , yC ) of the sample relative to the top-left luma sample of the picture. The value of PaletteIndexMap[ xC ][ yC ] shall be in the range of 0 to indexMax, inclusive. The variable adjustedRefIndexMax is derived as follows: adjustedRefIndex = indexMax + 1 if( scanPos > 0 ) { if( palette_run_type_flag[xcPrev][ycPrev] != COPY_ABOVE_MODE ) adjustedRefIndex = PaletteIndexMap[xcPrev][ycPrev] (7-1) else adjustedRefIndex = PaletteIndexMap[xC][yC − 1] When palette_run_type_flag[ xC ][ yC ] is equal to COPY_INDEX_MODE, the variable paletteIndex is derived as follows: paletteIndex = palette_index_idc if( palette_index_idc >= adjustedRefIndex ) paletteIndex = palette_index_idc + 1

# Experimental results

The proposed method is integrated on SCM-3.0 [1]. All tests were conducted under the common test condition (CTC) [2] for SCC with full frame IBC search range. Table 1 shows the results of 2.1 fixed context for run_type coding. It shows average 0.0%, 0.0%, 0.0% loss under AI, RA and LB configuration respectively. Table 2 shows the results of 2.2 limited index redundancy removal. It shows 0.0%, 0.0%, 0.1% loss under AI, RA and LB configuration respectively. Table 3 shows the results of 2.1+2.2. It shows 0.1%, 0.1%, 0.0% loss under AI, RA and LB configuration respectively.

Table 1 2.1 fixed context for run_type coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.2% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.3% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | -0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2 2.2 limitted index redundancy removal

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.2% | 0.2% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.2% | 0.2% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.2% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.2% |
| YUV, Animation, 720p | 0.1% | 0.1% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

Table 3 2.1 + 2.2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.2% | 0.2% | 0.2% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.3% | 0.4% | 0.4% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.2% | 0.2% | 0.2% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.2% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.2% | 0.2% |
| RGB, mixed content, 1440p & 1080p | -0.1% | 0.2% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.1% | 0.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- SCM-3.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-14.0+RExt-7.0+SCM-3.0/

- P. Lai, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma, “Description of Core Experiment 1 (CE1): Improvements of palette mode,” JCTVC-S1101, Oct. 2014.