| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0021 |
| --- | --- |

| Title: | CE1: Coding performance of SCC tools and suggested common test conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | PoLin Lai, Shan Liu, and Shawmin Lei 2860 Junction Ave, San Jose, CA, USA | Email: | { polin.lai, shan.liu, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Under common test conditions, this contribution presents coding performance measurements of Intra block copy (IBC) and adaptive colour transform (ACT).

For 4:4:4, Lossy, All Intra (AI), text and graphics with motion class (TGM, 7 HD sequences):

- Turn off IBC:   YUV: +62.9%, enc/dec time 83%/111%;	RGB: +59.2%, enc/dec time 90%/115%

- Turn off ACT:  YUV:  +0.4%, enc/dec time 79%/99%;	RGB: +10.9%, enc/dec time 74%/100%

From these measurements, it is asserted that turning off ACT for YUV sequences significantly speeds up runtime for simulations, with relatively minor impact on coding performance. It is suggested to modify the common test condition such that ACT is turned off for YUV sequences at sequence level.

# Introduction

Under common test conditions [1], intra block copy (IBC), and adaptive colour transform (ACT) have relatively larger runtime measurements. Following the mandates of AHG6 “Analyze coding performance of draft and proposed SCC coding features”, this contribution presents BD-rate and runtime measurements of these tools, and proposes modifications to the common test conditions.

# Experimental results

The tests were conducted under common test condition [1], by turning off each individual tool.

- Turning off ACT (Currently short, FULL Table to be added!!!)

- Turning off ACT:

# Summary

BD-rate and runtime measurements are provided, under common test conditions, for IBC and ACT. From the results, it is asserted that turning off ACT for YUV sequences significantly speeds up runtime for simulations, with relatively small impact on coding performance. It is suggested to modify the common test condition such that ACT is turned off at sequence level.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1015, Oct. 2014.