| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0081 |
| --- | --- |

| Title: | CE2-related: on the initialization of block vector prediction for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Tzu-Der Chuang Shan Liu Shawmin Lei | Email: | xiaozhong.xu@mediatek.com peter.chuang@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document provides two methods for the initialization of block vector prediction at the beginning of each CTU. In method 1, a fix is proposed to solve the mismatch between WD specification and SCM implementation. In method 2, a simplified initialization is proposed, using only constant values relative to the minimum CU size. The experimental results show that:

- the proposed method 1 brings on average 0.0%, -0.2% and -0.1% bitrate change as against SCM3.0 anchor for RGB TGM 1080p & 720p class in AI, RA and LB lossy coding, respectively;

- the proposed method 1 brings on average 0.0%, 0.0% and -0.1% bitrate change as against SCM3.0 anchor for YUV TGM 1080p & 720p class in AI, RA and LB lossy coding, respectively.

- the proposed method 2 brings on average -0.1%, 0.0% and -0.1% bitrate change as against SCM3.0 anchor for RGB TGM 1080p & 720p class in AI, RA and LB lossy coding, respectively;

- the proposed method 2 brings on average 0.0%, 0.0% and 0.1% bitrate change as against SCM3.0 anchor for YUV TGM 1080p & 720p class in AI, RA and LB lossy coding, respectively.

# Introduction

In the current WD [1], in block vector prediction for IBC, 2 spatial neighboring blocks’ BVs are used as the primary predictors. At the meantime, the last 2 coded BVs may be used in case the spatial candidates are not available. These two coded BVs are reset at the beginning of each CTU to be (-2W, 0) and (-W, 0), with W being the CU width of the first decoded IBC CU within the CTU.

However, in the software implementation, this initialization process is not done exactly the way as described in the document.

For the first decoded IBC PU (with CU width W1), the last 2 BVs are set to be (-2W1, 0) and (-W1, 0), and the decoded BV for this PU is BV1 = ( BV1_x, BV1_y).

For the second decoded IBC PU (with CU width W2), according to the text, the last coded 2 BVs should be (BV1_x, BV1_y) and (-2W1, 0), respectively (assuming no duplication in BV values here). In current SCM software, the last 2 coded BVs for the second decoded IBC PU are set to be (BV1_x, BV1_y) and (-2W2, 0). There is a mismatch in this initialization process for the last 2 coded BVs in block vector prediction.

# Proposed methods

It is proposed in method 1 in this contribution to modify the software such that the SCM implementation matches what is described in the text.

It is observed that the two initialization BV values are depending on the size of the first decoded IBC CU in the CTU. They can be different from one CTU to another. Further, they can be set only when the first IBC coded CU occurs in the CTU. Therefore, it is also proposed in method 2 in this contribution that, instead of using CU size dependent variables, two constant values (-2*min_CU_width, 0), and (-4*min_CU_width, 0) are used for the initialization of the last 2 coded BVs. In this approach, the initialization can be done at the beginning of each CTU, regardless where the 1st IBC coded CU is in a CTU.

# Experimental results

HM-16.2+ SCM-3.0 default coding structure is used as the anchor. The common test conditions specified in Error: Reference source not found are followed. The summarized experimental results of the proposed method 1 for 444 lossy and lossless coding are shown in Table 1 and 2, respectively. The summarized experimental results of the proposed method 2 for 444 lossy and lossless coding are shown in Table 3 and 4, respectively. The summarized experimental results of the proposed method 1 for 420 lossy and lossless coding are shown in Table 5 and 6, respectively. The summarized experimental results of the proposed method 2 for 420 lossy and lossless coding are shown in Table 7 and 8, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for 444 lossy coding in method 1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.1% | -0.2% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.4% | -0.1% |
| RGB, Animation, 720p | 0.1% | 0.3% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.2% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.2% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.7% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | -0.2% | -0.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.2% | 0.0% | -0.4% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | -0.1% | 0.0% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2 Experimental results for 444 lossless coding in method 1

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% |  |  |  |
| Dec Time[%] | 118% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |  |
| Dec Time[%] | 129% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 112% |  |  |  |
| Dec Time[%] | 123% |  |  |  |

Table 3 Experimental results for 444 lossy coding in method 2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 112% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | -0.1% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 111% |  |  |
| Dec Time[%] | 120% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.3% | -0.3% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 111% |  |  |
| Dec Time[%] | 124% |  |  |

Table 4 Experimental results for 444 lossless coding in method 2

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |  |
| Dec Time[%] | 109% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |  |
| Dec Time[%] | 122% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 111% |  |  |  |
| Dec Time[%] | 116% |  |  |  |

Table 5 Experimental results for 420 lossy coding in method 1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | 0.1% | 0.0% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.1% |
| Average of all sequences | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | 0.0% | 0.2% | -0.6% |
| Mixed content, 480p | 0.1% | 0.2% | 0.7% |
| Animation, 768p | 0.1% | 0.4% | -0.4% |
| Average of all sequences | 0.0% | 0.2% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |

Table 6 Experimental results for 420 lossless coding in method 1

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | -0.1% | -0.2% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | -0.2% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

Table 7 Experimental results for 420 lossy coding in method 2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | 0.1% | 0.1% | 0.2% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.1% | 0.0% | 0.1% |
| Average of all sequences | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | 0.1% | 0.0% | 0.2% |
| Mixed content, 480p | 0.0% | -0.1% | 0.2% |
| Animation, 768p | 0.0% | -0.1% | 0.2% |
| Average of all sequences | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | -0.1% | 0.2% | -0.4% |
| Mixed content, 480p | 0.1% | 0.4% | 0.1% |
| Animation, 768p | 0.2% | 0.9% | 0.0% |
| Average of all sequences | 0.0% | 0.4% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

Table 8 Experimental results for 420 lossless coding in method 2

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% | 0.1% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 96% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | -0.1% | -0.1% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

# Conclusions

This contribution proposes 2 methods to change the initialization of last 2 coded BVs for block vector prediction in the SCM software. In method 1, the SCM is modified to match what is described in the specification; in method 2, the CU size related initialization values are simplified to constant values. Simulation results show that the impact of these changes to the coding efficiency is minimal.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- R. Joshi, J. Xu, “HEVC Screen Content Coding Draft Text 2,” JCTVC-S1005, Strasbourg, FR, 17–24 Oct. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1015, Strasbourg, FR, 17–24 Oct. 2014.