| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0082r1 |
| --- | --- |

| Title: | CE1-related: Syntax fixes for the palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Chun-Lung Lin, 3Ching-Chieh Lin, Jih-Sheng Tu, Chao-Hsiung Hung 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2Chunlung@itri.org.tw 3JackLin@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

A syntax modification is presented in this contribution. The modification is to simplify the syntax by inferring the palette_sharing_flag as 0 when the stuffing buffer is empty. This fix can also solve the ambiguous problem of the signalling on the condition of empty stuffing palette buffer and zero palette size. The results reportedly show negligible coding performance change.

# Problem statement

In the current syntax of the palette coding, a palette_sharing_flag is always parsed to indicate whether to enable the sharing mode. If the palette_sharing_flag is 1, the palette table of the current CU is copied from the palette table of the previously decoded palette CU, and the palette size of the current CU is equal to that of the previously decoded palette CU. If the palette_sharing_flag is 0 and the stuffing palette entries exist, a palette_predictor_run is first parsed to indicate which stuffing palette entries are reused in the palette table of the current CU, a palette_num_signalled_entries is further parsed to indicate the number of the palettes signalled, lastly a palette_entry is parsed to get the values of the palettes signalled, and the palette size of the current CU is the addition of the number of the stuffing palette entries reused and the number of the palettes signalled. If the palette_sharing_flag is 0 and the stuffing palette entries are empty, the palette_predictor_run is skipped, the palette_num_signalled_entries is parsed to indicate the palette size of the current CU, and the palette_entry is further parsed to get the values of all palettes.

This contribution proposes to fix the syntax for the case that the stuffing buffer is empty. This case is occurred when all of the previously decoded palette CUs are zero palette size. Note that the case is absolutely occurred in the first decoded palette CU. For this case, the empty stuffing buffer means there is no previously palettes shared for the current CU, and it is redundancy to parse the palette_sharing_flag. Note that the palette_predictor_run flag related to stuffing scheme is also skipped for this case in the current reference software SCM 3.0. Besides, there is an ambiguous issue if an empty stuffing buffer and a zero palette size for the current CU are concurrently occurred. On the condition of the empty stuffing buffer, the following two parsing processes have the same implication that the palette size is zero:

- Parse palette_sharing_flag as 1. The palette size must be zero.

- Parse palette_sharing_flag as 0. Parse palette_num_signalled_entries as 0, which means the palette size must be zero.

Therefore, the syntax on the condition of the empty stuffing buffer must be fixed to avoid the ambiguous of the decoding process for the implication that the palette size is zero.

# Proposed method

A syntax fix is proposed to figure out the issue discussed above.

When the stuffing palette entries are empty, the palette_sharing_flag is proposed to be inferred as 0, as demonstrated in the syntax table below.

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| If(PredictorPaletteSize != 0) |  |
| palette_share_flag[ x0 ][ y0 ] | ae(v) |
| … |  |

The fix simplifies the syntax for the case of the empty stuffing buffer. Besides, for the condition of the empty stuffing buffer, there is only one parsing process to indicate if the current palette size is zero by parsing palette_num_signalled_entries binarized as 1.

# Simulation results

The fix of the syntax for the palette mode are simulated on top of SCM 3.0 [1]. The results are shown in Table 1. It shows that there is negligible coding performance change for the proposed modification.

Table 1. Lossy results.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | -0.1% | -0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.2% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.1% | 0.0% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.4% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

This contribution proposes a syntax modification for palette coding. By constraining palette_share_flag, the syntax redundancy can be removed, and ambiguous of the parsing process for the case of the zero palette size can also be solved. Negligible coding performance change is reported by applying the proposed fixe. It is recommended to adopt this contribution for the next version of SCM.

# Reference

- P. Lai, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma, “Description of Core Experiment 1 (CE1): Improvements of palette mode,” 19th JCT-VC meeting, JCTVC-S1101, Strasbourg, France, Oct. 2014.

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)