| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0087r3 |
| --- | --- |

| Title: | CE1-Related: Improved Palette Table Generation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Chao-Hsiung Hung 2Chun-Lung Lin 3Yao-Jen Chang 4Ching-Chieh Lin 5Jih-Sheng Tu 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | 1+886-3-5917964 1chhung@itri.org.tw 2Chunlung@itri.org.tw 3britpablo@itri.org.tw 4JackLin@itri.org.tw 5SunriseJSTu@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

Palette coding provides good coding performance for SCC (screen content coding) test sequences. It analyzes major colors of a CU. Then it creates a palette table and presents each pixel by corresponding color index. The K-means method is adopted for generating major colors. We improve this method by selecting significant peaks from the histogram as initial color groups. Under full frame, all intra, and lossy test condition, the proposed method provides -0.2% and -0.5% BD-rate saving for the classes “RGB, text & graphics with motion, 1080p & 720p” and “YUV, text & graphics with motion, 1080p & 720p.” Combing the proposed method with the fixed CE1 test D1 in JCTVC-T0169 improves the coding gain to -0.6%.

# Introduction

SCC test sequences are usually composed of a few major colors. Palette coding adopts this characteristic and compresses them efficiently. It represents a CU by a palette table and a color index map. The different major color sets provide much different coding performances.

The K-means method classifies pixels into different color groups [1]. The initial color groups all have null pixel number and zero average. For the current processing pixel, the difference between this pixel and each color group average is calculated. If the minimum difference is less than the error limit based on the quantization step size, the current pixel will be classified into the corresponding color group. For this corresponding color group, its pixel number will be added by 1 and the average will be re-calculated. Otherwise, the current processing pixel forms a new color group. For this new color group, its pixel number is 1 and its average is the current processing pixel value. All the color groups will be used for next classification. After finishing processing all pixels of the CU, the generated major colors are chosen from color group averages.

The new-added pixel affects average of selected color group. If the color group contains a few pixels, the re-calculated average may be shifted far away. The different scan order may also result in different major colors. The generated major colors may be also far away from the original significant colors. In this proposal, we select significant peaks from the histogram as initial color groups. The propose method generates major colors near original ones and provides better coding performance.

# Proposed algorithm

Figure 1. The histogram of the current CU pixels.

We select the significant peaks from the histogram as the initial color groups. We implement the proposed method in following steps.

1. We generate the histogram for the current CU pixels in Figure 1. The pixel C1 has the maximum pixel number P1 in the histogram. The current CU size is W. The error limit E is based on the quantization step size.

2. If P1 ≥ T1, we then find the pixel C2 with the maximum pixel number P2 in the range E1 around C1. We set T1 = max(W × E / 4, W / 2) and E1 = E / 4.

3. If P2 ≥ T2 = P1 / 2, we just remove C1 from the histogram. The removed C1 will be not selected as an initial color group.

4. Otherwise, we select C1 as an initial color group G1. The average and pixel number of G1 are C1 and P1. We put C1 pixel value into an initial pixel table. We then clean all pixels, including C1, in the range E2 = E / 4 around C1.

5. We process remaining pixels in the histogram in above steps until no pixel with pixel number larger than T1.

6. After initial groups selection above, we have a set of initial groups and an initial pixel table. We then raster scan the current CU pixels. If a new-scanned pixel finds no matched pixel in the initial pixel table, it will be classified by original K-means method. If the current processing pixel is classified to an initial color group, this initial color group will be updated similarly by original K-means method. Otherwise, this pixel also forms a new color group. All color groups will be used for future classification.

# Simulation results

The proposed method is simulated based on SCM 3.0 reference software under the common test condition [2]. Table 1 and Table 2 show the results of proposed method. We also combine it with fixed CE1 test D1 [3]. Table 3 to Table 6 show the results of combined method.

Table 1. lossless results.

Table 2. lossy results.

Table 3. Combing with T0169 version 1, lossy results

Table 4. Combing with T0169 version 1, lossless results

Table 5. Combing with T0169 version 2, lossy results

Table 6. Combing with T0169 version 2, lossless results

# Conclusion

We propose an improved palette table generation. We select significant pixels from the histogram as the initial color groups. We then classify un-selected pixels by original K-means method. Compared with SCM 3.0 reference software, the BD rate savings are -0.2% and -0.5% for “RGB, text & graphics with motion, 1080p & 720p” and “YUV, text & graphics with motion, 1080p & 720p” respectively. The coding gain will be improved to -0.6% if we combine it with the fixed encoder modification in CE1 test D1. We recommend to adopt this improved method.

# Reference

- C. Gisquet, G. Laroche, T. Poirier, P. Onno, “SCCE3: Test C.2 – Combination of palette coding tools,” 18th JCT-VC meeting, JCTVC-R0086, Sapporo, Japan, June 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding,” 19th JCT-VC meeting, JCTVC-S1015, Strasbourg, France, Oct. 2014.

- Y.-J. Chang, C.-H. Hung, C.-L. Lin, C.-C. Lin, J.-S. Tu, “CE1-related: Fix of CE1 Test D.1,” 20th JCT-VC meeting, JCTVC-T0169, Geneva, Switzerland, Feb. 2015.

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)