| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0088r1 |
| --- | --- |

| Title: | Non-CE 1: Modifications of copy-above mode in index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Jih-Sheng Tu, 2Chun-Lung Lin, Chao-Hsiung Hung, 3Ching-Chieh Lin, Yao-Jen Chang 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1sunrisejstu@itri.org.tw 2Chunlung@itri.org.tw 3JackLin@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

In this proposal, a copy mode modification method and a context increment extension for coding index/copy mode bin are proposed. The test results report that the proposed methods achieve 0.23% gain for 1080p & 720p text and graphics test sequences in the full frame intra BC test condition for All Intra lossy case.

# Introduction

In 19th JCT-VC meeting, a copy mode modification method was proposed in section 2.2 of JCTVC-S0047 [1]. It solved the problem which was described in section 1.2 of JCTVC-S0047 [1]. In this contribution, this method is integrated into SCM3.0 with a proposed context increment extension.

# The Proposed Methods

The proposed methods of copy mode modification and context increment extension are applied under the circumstance mentioned in section 1.2 of JCTVC-S0047 [1]. In this case, when coding the index/copy mode bin, the context increment would not be set as 0 or 1, but set as 2 in the proposed method. And a pseudo index value (i.e. index number 0) is used to replace the above-neighbouring index virtually when testing copy mode is valid or not.

When processing the first line of the CU, it is naturally to consider the above-neighbouring index as the pseudo index. Hence, the index/copy mode bin would be coded on the first line of CU.

# Simulation Results

The proposed scheme is implemented on SCM3.0. The modification improves 0.23% gain in average for 1080p & 720p text and graphics test sequences. Two test conditions are provided:

- Test 1: All of the proposed methods.

- Test 2: All of the proposed methods excepting the context increment extension.

Table 1 to Table 2 demonstrate the coding performance under full frame intra BC against SCM3.0 anchor. The encoding time may not be accurate.

Table 1. Proposed(Test 1) vs. Anchor  (AI-lossy)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.23% | -0.20% | -0.20% |
| RGB, mixed content, 1440p & 1080p | -0.03% | -0.03% | -0.04% |
| RGB, Animation, 720p | 0.00% | 0.01% | 0.00% |
| RGB, camera captured, 1080p | -0.01% | -0.01% | 0.01% |
| YUV, text & graphics with motion, 1080p & 720p | -0.23% | -0.26% | -0.23% |
| YUV, mixed content, 1440p & 1080p | -0.10% | -0.12% | -0.18% |
| YUV, Animation, 720p | 0.02% | 0.01% | -0.02% |
| YUV, camera captured, 1080p | 0.00% | -0.01% | 0.00% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2. Proposed(Test 2) vs. Anchor  (AI-lossy)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.14% | -0.11% | -0.12% |
| RGB, mixed content, 1440p & 1080p | 0.00% | 0.00% | -0.01% |
| RGB, Animation, 720p | 0.00% | 0.00% | 0.00% |
| RGB, camera captured, 1080p | 0.00% | 0.00% | 0.00% |
| YUV, text & graphics with motion, 1080p & 720p | -0.22% | -0.12% | -0.11% |
| YUV, mixed content, 1440p & 1080p | -0.06% | -0.05% | -0.10% |
| YUV, Animation, 720p | 0.02% | -0.01% | -0.01% |
| YUV, camera captured, 1080p | 0.00% | 0.00% | 0.04% |
| Enc Time[%] | NA |  |  |
| Dec Time[%] | NA |  |  |

# Reference

- J.-S. Tu, C.-L. Lin, C.-H. Hung, C.-C. Lin, Y.-J. Chang, “Non-CE6: Modifications of copy-left and copy-above modes in index coding,” 19th JCT-VC meeting, JCTVC-S0047, Strasbourg, FR, Oct. 2014.

# Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).