| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0089 |
| --- | --- |

| Title: | CE2: Test 5.1 and Test 8.1 on vector binarization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Marta Karczewicz Chao Pang Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 858-658-2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal reports results of CE2 Test 5.1 and CE2 Test 8.1 on vector binarization for block vector and motion vector coding. In this test a unified binarization scheme is proposed for block vector difference (BVD) and motion vector difference (MVD). In this test, two regular bins are coded to indicate if the BVD value is greater than 0 and 4 respectively. The remaining bins are bypass coded using a Exp-Golomb(EG) or fixed length code. The test 8.1 applies this method on top of unification framework as in CE2 Test 1. It is reported that Test 5.1 provides objective luma bit rate reductions of (0.2 %, 0.3%), (1.1%, 1.3%), (1.6%,1.8%) for All Intra, Random access and Low Delay-B text and graphics RGB and YUV categories, respectively over SCM-3.0 anchor. It is reported that Test 8.1 provides objective luma bit rate reductions of (0.9 %, 1.0%), (3.1%, 3.5%), (3.6%,3.9%) for All Intra, Random access and Low Delay-B text and graphics RGB and YUV categories, respectively over SCM-3.0 anchor

# Proposed Method

It is observed that the distributions of BVD’s and MVD’s for screen content sequences were different from that of camera captured sequences. It was asserted that the current binarization methods were inefficient in adapting to these characteristics and CE2 was organized to study binarization methods. It was also suggested in the 19th JCT-VC meeting to have a unified binarization scheme for BVD and MVD coding. The unified binarization test in CE2 5.1 is detailed below

In this test, first a context coded flag is coded to indicate vector is zero. When vector is not zero, another flag may be coded to indicate if absolute level of vector is greater than 4. If vector is greater than 4, the remaining absolute level is coded using bypass Exp-Golomb codes of order 4 else it is coded using a fixed length code with length equal to 2. One flag is coded to indicate sign of the vector.

.In CE2 Test 8.1, the same method of CE2 Test 5.1 is applied on top of unification framework of CE2 Test 1.

# Experimental results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM3.0, and the common test conditions specified in Error: Reference source not found are followed. The summarized experimental results of 4:4:4 format sequences for lossy coding for Test 5.1 and Test 8.1 are shown in Table 2 and Error: Reference source not found, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Test 5.1 - Experimental results for lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.2% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 88% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.1% | -1.1% | -1.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.2% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -1.3% | -1.3% | -1.4% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.2% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.6% | -1.4% | -1.3% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.4% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.4% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.8% | -1.6% | -1.8% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.6% | -0.2% |
| YUV, Animation, 720p | -0.1% | -0.2% | -0.4% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 91% |  |  |
| Dec Time[%] | 92% |  |  |

Table 2 Test 8.1 - Experimental results for lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.9% | -2.3% | -2.2% |
| RGB, mixed content, 1440p & 1080p | -0.8% | -1.6% | -1.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -1.0% | -2.3% | -2.4% |
| YUV, mixed content, 1440p & 1080p | -0.7% | -1.7% | -1.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 112% |  |  |
| Dec Time[%] | 85% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.1% | -4.9% | -4.8% |
| RGB, mixed content, 1440p & 1080p | -0.9% | -2.3% | -2.4% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -3.5% | -5.0% | -5.2% |
| YUV, mixed content, 1440p & 1080p | -1.0% | -3.0% | -3.0% |
| YUV, Animation, 720p | -0.1% | -0.4% | 0.0% |
| YUV, camera captured, 1080p | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 86% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.6% | -5.0% | -4.9% |
| RGB, mixed content, 1440p & 1080p | -1.2% | -2.6% | -2.8% |
| RGB, Animation, 720p | 0.0% | -0.2% | -0.3% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -3.9% | -5.3% | -5.6% |
| YUV, mixed content, 1440p & 1080p | -1.4% | -3.9% | -4.2% |
| YUV, Animation, 720p | -0.1% | -0.6% | -0.5% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 91% |  |  |
| Dec Time[%] | 86% |  |  |

# References

[1] S. Liu, C. Pang, and J. Xu, “Description of Core Experiment 2 (CE2): Intra block copy relationship to inter coding”, JCTVC-S1102, Strasbourg, FR, October 2014.

[2] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding”, JCTVC-S1015, Strasbourg, FR, October 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text modifications

The text changes for the proposed method are marked in yellow.

### Block/Motion vector difference syntax

### Block/Motion vector difference syntax

The text changes for the proposed MVD coding method are marked in yellow.

| mvd_coding ( x0, y0, refList ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater4_flag[ 0 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater4_flag[ 1 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 0 ] ) { |  |
| if(abs_mvd_greater4_flag[ 0 ] ) |  |
| abs_mvd_ minus5[ 0 ] | ae(v) |
| else |  |
| abs_mvd_minus1[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if(abs_mvd_greater0_flag[ 1 ] ) { |  |
| if(abs_mvd_greater4_flag[ 1 ] ) |  |
| abs_mvd_ minus5[ 1 ] | ae(v) |
| else |  |
| abs_mvd_minus1[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

Semantics

abs_mvd_greater0_flag[ compIdx ] specifies whether the absolute value of a block vector component difference is greater than 0.

abs_mvd_greater4_flag[ compIdx ] specifies whether the absolute value of a block vector component difference is greater than 4.

When abs_mvd_greater4_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_minus1[ compIdx ] plus 1 specifies the absolute value of a block vector component difference.

When abs_mvd_minus1[ compIdx ] is not present, it is inferred to be equal to −1.

abs_mvd_ minus5[ compIdx ] plus 5 specifies the absolute value of a block vector component difference.

When abs_mvd_minus5[ compIdx ] is not present, it is inferred to be equal to −5.

mvd_sign_flag[ compIdx ] specifies the sign of a block vector component difference as follows:

If mvd_sign_flag[ compIdx ] is equal to 0, the corresponding block vector component difference has a positive value.

Otherwise (mvd_sign_flag[ compIdx ] is equal to 1), the corresponding block vector component difference has a negative value.

When mvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

The block vector difference lMvd [ compIdx ] for compIdx = 0..1 is derived as follows:

lMvd [ compIdx ] = abs_mvd_greater0_flag [ compIdx ]  *  (  abs_mvd_greater4_flag? : (abs_mvd_minus5 [ compIdx ] + 5 )  :  ( abs_mvd_minus1 [ compIdx ] + 1 ) ) * ( 1 − 2 * mvd_sign_flag [ compIdx ] );

| Table 9-xx – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| mvd_coding ( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater4_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus1 [ ] | FL | cMax =4 |
|  | abs_mvd_minus5 | EG4 |  |
|  | mvd_sign_flag [ ] | FL | cMax = 1 |