| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0090 |
| --- | --- |

| Title: | CE2: Test 5.3 and Test 8.2 on block vector (BV) coding for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Marta Karczewicz Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121-1714 USA Kazuyuki Miyazawa Akira Minezawa Shun-ichi Sekiguchi | Tel: Email: | 858-658-2042 krapaka@qti.qualcomm.com +81 467 41 2463 Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp |
| Source: | Qualcomm Inc. Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This proposal reports results of CE2 Test 5.3 and CE2 Test 8.3 on block vector (BV) coding for IBC. This test aims to remove redundancies in the BV coding such as sign, value of the BVD. In addition, this test adds an additional BV predictor namely (0, 0). It is reported that Test 5.3 provides objective luma bit rate reductions of (0.7 %, 0.8%), (1.4%, 1.6%), (1.8%,1.9%) for All Intra, Random access and Low Delay-B text and graphics RGB and YUV categories, respectively over SCM-3.0 anchor. It is reported that Test 8.2 provides objective luma bit rate reductions of (1.1%, 1.2%), (3.4%, 3.7%), (3.8%, 4.1%) for All Intra, Random access and Low Delay-B text and graphics RGB and YUV categories, respectively over SCM-3.0 anchor.

# Introduction

Unlike the inter modes, the IBC mode uses already decoded samples within the same picture for its prediction. This present an inferred restriction that an IBC block vector cannot fall outside of the already decoded region corresponding to that block. This contribution proposes changes to remove these redundancies by inferring sign and block vectors in some scenarios such that resultant block vector falls in the already decodable region.

In the figure 1 shown below, for the current block (marked in green), the blocks marked with yellow are valid BVD’s and those marked with blue are invalid as they fall in the non-decoded region corresponding to the green block. It may be observed, for example, for predictor’s p2 when the BVDx (horizontal component of the block) is positive, then in order for the BV to be valid and not fall in the non-decoded region, the BVDy vertical component has to be negative. Additionally the BVDy components also need to be greater than height of the current block. Similar redundancies that exist are asserted to be removed by proposed method.

Figure 1: An example IBC BVD distribution for a CU in a picture

Note that the redundancy removal do not introduce any parsing dependency and they only infer the availability and mode information of the blocks and are similar in nature to the checks performed for the parsing of some syntax elements in HEVC.

# Proposed Solution

In CE2 Test 5.3 [1], a ‘bv_pred_flag’ is added to indicate whether the BV is predicted or not.

- When BV is not predicted and vertical component is zero, the sign and value of horizontal component are inferred.

- When BV is not predicted and horizontal component is greater than or equal to zero, the sign and value of vertical component are inferred.

In CE2 Test 8.3 [1], the same method is applied on top of unification frame of CE2 Test 1.

# Experimental results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM3.0, and the common test conditions specified in Error: Reference source not found are followed. The summarized experimental results of 4:4:4 format sequences for lossy coding for Test 5.3 and Test 8.3 are shown in Table 2 and Error: Reference source not found, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Test 5.3 - Experimental results for lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.7% | -0.6% | -0.6% |
| RGB, mixed content, 1440p & 1080p | -0.4% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.8% | -0.8% | -0.7% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.5% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 91% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.4% | -1.4% | -1.3% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.6% | -1.6% | -1.6% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.4% | -0.4% |
| YUV, Animation, 720p | -0.1% | -0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.8% | -1.6% | -1.5% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.2% | -0.5% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.4% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.9% | -1.9% | -1.8% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.7% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.5% | -0.2% |
| YUV, camera captured, 1080p | 0.1% | -0.1% | -0.1% |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 93% |  |  |

Table 2 Test 8.2 - Experimental results for lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.1% | -2.5% | -2.4% |
| RGB, mixed content, 1440p & 1080p | -1.0% | -1.7% | -1.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.2% | -2.4% | -2.6% |
| YUV, mixed content, 1440p & 1080p | -0.9% | -1.9% | -2.0% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 117% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.4% | -5.1% | -5.1% |
| RGB, mixed content, 1440p & 1080p | -0.9% | -2.5% | -2.6% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | -0.2% | -0.1% | -0.2% |
| YUV, text & graphics with motion, 1080p & 720p | -3.7% | -5.3% | -5.5% |
| YUV, mixed content, 1440p & 1080p | -1.2% | -3.1% | -3.0% |
| YUV, Animation, 720p | -0.2% | -0.4% | -0.2% |
| YUV, camera captured, 1080p | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 84% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.8% | -5.2% | -5.1% |
| RGB, mixed content, 1440p & 1080p | -1.2% | -2.8% | -3.2% |
| RGB, Animation, 720p | 0.0% | -0.3% | -0.4% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -4.1% | -5.5% | -5.5% |
| YUV, mixed content, 1440p & 1080p | -1.6% | -4.3% | -4.4% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | -0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 89% |  |  |

# References

[1] S. Liu, C. Pang, and J. Xu, “Description of Core Experiment 2 (CE2): Intra block copy relationship to inter coding”, JCTVC-S1102, Strasbourg, FR, October 2014.

[2] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding”, JCTVC-S1015, Strasbourg, FR, October 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text modifications

The text changes for the proposed method are marked in yellow.

### Block/Motion vector difference syntax

The text changes for the proposed MVD coding method are marked in yellow.

| mvd_coding ( x0, y0, refList ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if((abs_mvd_greater0_flag[ 0 ] ¦ ¦ abs_mvd_greater0_flag[ 1 ] ) && !codeMvdFlag ) |  |
| mv_pred_flag | ae(v) |
| if(abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater4_flag[ 0 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater4_flag[ 1 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 0 ] ) { |  |
| if(abs_mvd_greater4_flag[ 0 ] ) |  |
| abs_mvd_ minus5[ 0 ] | ae(v) |
| else |  |
| abs_mvd_minus1[ 0 ] | ae(v) |
| if!(((mv_pred_flag==0) && (abs_bvd_greater0_flag[ 1 ]==0))&& !codeMvdFlag) |  |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if(abs_mvd_greater0_flag[ 1 ] ) { |  |
| if(abs_mvd_greater4_flag[ 1 ] ) |  |
| abs_mvd_ minus5[ 1 ] | ae(v) |
| Else |  |
| abs_mvd_minus1[ 1 ] | ae(v) |
| if!(((mv_pred_flag==0) && (bvd_sign_flag[ 0 ] ==0 ¦¦ abs_bvd_greater0_flag[ 0 ] == 0)) && !codeMvdFlag ) |  |
| bvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

For unification framework based on test1, the variable codeMvdFlag is derived as follow:

codeMvdFlag = ( DiffPicOrderCnt( RefPicList0[ ref_idx_l0 ], currPic )  !=  0 ) ? 1 : 0

For non-unification framework based on sSCM3.0 , the variable codeMvdFlag is derived as follow:

codeMvdFlag = !intra_bc_flag[x0][y0]

mv_pred_flag specifies whether the block vector is predicted or not. When bv_pred_flag is equal to 0, the block vector is not predicted else it is predicted.

When bv_pred_flag is not present, it is inferred to be equal to 1.

bvd_sign_flag[ compIdx ] specifies the sign of a block vector component difference as follows:

If bvd_sign_flag[ compIdx ] is equal to 0, the corresponding block vector component difference has a positive value.

Otherwise (bvd_sign_flag[ compIdx ] is equal to 1), the corresponding block vector component difference has a negative value.

When bvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 1.

The block vector difference lBvdT [ compIdx ] for compIdx = 0..1 is derived as follows:

lBvdT[ compIdx ] = abs_bvd_greater0_flag [ compIdx ]  *  (  abs_bvd_greater4_flag? : (abs_bvd_minus5 [ compIdx ] + 5 )  :  ( abs_bvd_minus1 [ compIdx ] + 1 ) ) * ( 1 − 2 * bvd_sign_flag [ compIdx ] );

The block vector difference lBvd [ compIdx ] for compIdx = 0..1 is derived as follows:

If(((bv_pred_flag==0) && abs_bvd_greater0_flag[ 1 ]==0) &&  !codeMvdFlag)   
		lBvd [ 0 ] = lBvdT[ 0 ] + uiWidth-1;

If((bv_pred_flag==0) && lBvdT[ 0 ] >=0) &&  !codeMvdFlag)   
		lBvd [ 1 ] = lBvdT[ 1 ] + uiHeight-1;

where uiWidth and uiHeight are the width and height of the current PU.

| Table 9-xx – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| mvd_coding ( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater4_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus1 [ ] | FL | cMax =4 |
|  | abs_mvd_minus5 | EG4 |  |
|  | mvd_sign_flag [ ] | FL | cMax = 1 |