| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0091 |
| --- | --- |

| Title: | Non-CE3: Adaptive PU partitioning for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Marta Karczewicz Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 858-658-2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to enable adaptive PU partitioning for intra block copy mode (IBC). IBC mode based on current working draft (WD) enables only fixed partitions such as 2Nx2N, 2NxN, Nx2N and NxN. In this contribution an adaptive PU partitioning for IBC is proposed in the similar lines to AMP in the inter mode coding. The maximum number of PU partitions (hence the block vectors) within the CU are maintained to be four which is also maximum PU partitions (and block vectors) for a CU in current SCC WD. The block vectors difference (BVD) for each partition are coded using the unified binarization scheme as in CE2 Test 5.1. The worst case regular bins for CU does not exceed the HEVC v2 specifications. The experimental results show that for RGB TMG sequences, the proposed method improves the coding efficiency by over 1.1%.

# Introduction

In the current working draft, IBC mode enables fixed PU partitions 2Nx2N, 2NxN and Nx2N PU partition for all the CU sizes. In addition, when the CU is the smallest CU, it enables NxN PU partition. Figure 2 illustrates the fixed IBC PU partition sizes.

Fig 2: IBC fixed PU partitions

In general, PU partitioning improves the prediction quality thus minimizing the residues that are coded. It is asserted that for screen content sequences which exhibit rapid changes in the content, a finer adaptive PU partitioning further improves the prediction quality. Several methods have been proposed such as intra Line/string copy that provide prediction units as small as a sample to row/column length. However smaller the prediction units, the larger are the complexities and their bandwidth requirements. Some drawbacks of these methods include a) number of BV’s within the current PU can go upto number of rows/columns or samples thus severely affecting the cabac throughput b) Large number of motion vectors increase the worst case bandwidth due to increased fetches of smaller data from non-aligned regions.

# Proposed Solution

In this contribution, an adaptive PU partitioning for IBC is proposed in the similar lines to AMP in the inter mode coding. A PU partition is sub-divided into at most 2 sub-partitions (sub-PU) as shown in the figure 3.

Fig 3: Proposed adaptive Intra block copy sub-partitioning

The presence of sub-PU’s and the sub-PU split info within the current PU is signalled. Each Sub-PU has associated block vector in the similar lines to the PU. The sub-PU splitting is not enabled for NxN PU modes. Note that there is no change to signalling of the PU mode info or the block motion compensation. The maximum number of PU partitions and the block vectors within the CU are maintained to be four (which is also maximum PU partitions in SCC WD. The BVD for each partition are coded using the unified binarization scheme as in CE2 Test 5.1. The worst case regular bins for CU does not exceed the HEVC v2 specifications.

# Experimental results

HM-16.2+SCM-3.0 default coding structure is used as the anchor. The common test conditions specified in [1] are followed. The summarized experimental results are shown in the following tables.

Table 1 : - Experimental results for lossy coding All Intra configuration

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.1% | -1.0% | -1.0% |
| RGB, mixed content, 1440p & 1080p | -1.0% | -0.9% | -0.9% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.0% | -0.9% | -0.9% |
| YUV, mixed content, 1440p & 1080p | -1.0% | -0.9% | -0.9% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 120% |  |  |
| Dec Time[%] | 101% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.7% | -0.6% | -0.7% |
| RGB, mixed content, 1440p & 1080p | -0.4% | -0.4% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.6% | -0.7% | -0.6% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.3% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | #VALUE! |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.4% | -0.3% | -0.4% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.4% | -0.6% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 95% |  |  |

# References

- JCTVC-S1015, “Common conditions for screen content coding tests,” Strasbourg, FR, 17–24 Oct. 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).