| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0092 |
| --- | --- |

| Title: | Non-CE3: Vector coding for CE3 Test A on Intra line Copy mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Marta Karczewicz Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 858-658-2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution aims at improving the cabac throughput and reducing the worst case number of regular bins for CE3 Test A - intra line copy mode by unifying the intra line vectors binarization schemes with block/motion vector coding as in CE2 Test 5.1. The intra line vectors difference (ILVD) for each line are coded using the unified binarization scheme as in CE2 Test 5.1. Further to maintain the worst case regular bins for 8x8 CU within HEVC v2 specifications, except the first line vector, all the other line vectors are coded using bypass bins and ILC is not enabled for NxN. The experimental results show that for RGB and YUV TMG sequences, the proposed method applied to CE3 Test A improves the coding efficiency by over 1.9% and 1.7% respectively over SCM3.0 anchor.

# Introduction

CE3-Test A studies the intra line copy technique for screen content coding. The Intra line copy mode (ILC) partitions the prediction unit into multiple 1-sample-wide rows or columns. Each of these partitions have a corresponding line vectors that point to the prediction samples within the current picture. The number of line vectors for a PU depends on its number of rows or columns. CE3- Test A applies current IBC binarization for coding all line vectors thereby increasing the number of regular bins for that PU to factor of rows or columns compared to IBC mode (1-regular bin for each vector).Further, CE3 Test A: proposes to disable RDPCM when ILC mode is enabled in order to maintain the HEVC worst case regular bins limits for a CU. It is asserted that this restriction does not resolve the vector parsing throughput within the PU which can vary significantly based on the number of rows or columns in the PU.

# Proposed Solution

In this contribution, following changes are proposed to CE3 Test A.

- First intra line difference vector in the PU are coded using the unified binarization as tested in CE2 Test 5.1

- Rest of intra line difference vector in the PU are bypass coded using the above binarization.

- Remove the constraints on RDPCM for ILC mode.

- Intra line copy is not enabled for NxN.

# Experimental results

HM-16.2+SCM-3.0 default coding structure is used as the anchor. The common test conditions specified in [1] are followed. The summarized experimental results are shown in the following tables.

Table 1 : - Experimental results for lossy coding All Intra configuration

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.9% | -1.7% | -1.7% |
| RGB, mixed content, 1440p & 1080p | -2.0% | -1.7% | -1.7% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.7% | -1.6% | -1.5% |
| YUV, mixed content, 1440p & 1080p | -1.7% | -1.7% | -1.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 120% |  |  |
| Dec Time[%] | 98% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.2% | -0.9% | -1.0% |
| RGB, mixed content, 1440p & 1080p | -1.0% | -0.8% | -0.9% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.1% | -1.0% | -1.0% |
| YUV, mixed content, 1440p & 1080p | -1.0% | -0.9% | -0.9% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 90% |  |  |
| Dec Time[%] | 91% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.8% | -0.6% | -0.6% |
| RGB, mixed content, 1440p & 1080p | -0.5% | -0.3% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.6% | -0.5% | -0.7% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.6% | -0.6% |
| YUV, Animation, 720p | 0.0% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.1% | 0.1% |
| Enc Time[%] | 86% |  |  |
| Dec Time[%] | 89% |  |  |

# References

- C. C. Chen, Y. Chen, J. Z. Xu, T. Lin, and W. Wang, “Description of Core Experiment 3 (CE3): Intra Line Copy and Intra String Copy,” JCTVC-S1103, Strasbourg, Oct. 2014.

- JCTVC-S1015, “Common conditions for screen content coding tests,” Strasbourg, FR, 17–24 Oct. 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).