| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0094-r1 |
| --- | --- |

| Title: | CE2 Test1: Intra block copy and inter signalling unification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Ye-Kui Wang, Vadim Seregin, Krishna Rapaka, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA Xiaozhong Xu, Shan Liu, Shawmin Lei MediaTek Bin Li, Jizheng Xu Microsoft Corp. | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com xiaozhong.xu@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com libin@microsoft.com jzxu@microsoft.com |
| Source: | Qualcomm Inc., MediaTek, Microsoft Corp. |  |  |

_____________________________

# Abstract

This proposal reports results of CE2 Test 1 on intra block copy and inter signalling unification. In this test, the intra block copy mode is signalled reusing inter signalling by adding the current picture to the reference picture list. With the shared signalling, intra block copy can be differentiated from the conventional inter by checking the attributes of the reference picture.

# Introduction

Intra block copy (Intra BC) is an efficient technique to predict repetitive patterns in the frame and leads to significant bit-rate savings for screen content. Currently, the Intra BC mode is signalled at CU-level, and it has different partition modes, including 2Nx2N/2NxN/Nx2N/NxN. To save the memory bandwidth, the block vector (BV) for Intra BC is with integer-pixel precision.

# Proposed method

In CE2 Test 1 [1], the Intra BC mode is signalled reusing inter signalling by adding the current picture (without in-loop filtering) into the reference picture list. With the shared signalling, Intra BC can be differentiated from the conventional Inter mode by checking reference index of the reference picture used in the prediction. If the reference picture is the current picture, then the prediction mode is Intra BC mode. Otherwise, it is conventional Inter mode. The main modifications in this contribution lie in the following aspects:

- When Intra BC is enabled, there shall be no I-slice in CVS.

- The current picture is marked as long-term before decoding the current frame, and marked back to short-term after decoding the current frame.

- When the reference picture is the current picture, MVD is coded in integer-pixel precision with the current Intra BC BV prediction method and block vector difference (BVD) coding method.

- When the reference picture is the current picture, the MVs stored in DPB are with quarter-pixel precision. Thus, the motion compensation and deblocking modules for inter can be transparently used without any changes.

- It is restricted that the collocated picture cannot be the current picture.

- 4x4 Intra BC blocks are disabled, which is also aligned with the inter mode partitioning.

- When constrained Intra prediction is enabled, the prediction only uses decoded samples from neighbouring coding blocks coded with Intra mode or Intra BC mode.

- When the reference picture is the current picture and constrained Intra prediction is enabled, the TMVP shall be disallowed.

# Experimental results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM3.0, and the common test conditions specified in Error: Reference source not found are followed. The summarized experimental results of 4:4:4 format sequences for lossy and lossless coding are shown in Table 1 and Table 2, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for lossy coding

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.6% | -2.0% | -2.0% | -2.2% | -4.0% | -3.9% | -2.3% | -3.8% | -3.7% |
| RGB, mixed content, 1440p & 1080p | -0.7% | -1.5% | -1.6% | -0.7% | -2.2% | -2.4% | -0.9% | -2.5% | -2.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% | -0.2% | -0.1% | -0.2% | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.7% | -2.0% | -2.2% | -2.4% | -4.0% | -4.3% | -2.5% | -3.9% | -4.1% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -1.7% | -1.7% | -0.9% | -2.7% | -2.8% | -1.4% | -3.9% | -4.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% | -0.2% | -0.3% | 0.0% | 0.0% | 0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% | -0.2% | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 117% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 96% |  |  | 102% |  |  |

Table 2 Experimental results for lossless coding

|  | All Intra | Random Access | Low Delay B |
| --- | --- | --- | --- |
|  | Bit-rate change (Average) | Bit-rate change (Average) | Bit-rate change (Average) |
| RGB, text & graphics with motion, 1080p & 720p | 0.4% | -0.6% | -0.8% |
| RGB, mixed content, 1440p & 1080p | -0.8% | -0.2% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 1.2% | -0.2% | -0.5% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% | 97% | 96% |
| Dec Time[%] | 100% | 95% | 97% |

# References

[1] S. Liu, C. Pang, and J. Xu, “Description of Core Experiment 2 (CE2): Intra block copy relationship to inter coding”, JCTVC-S1102, Strasbourg, FR, October 2014.

[2] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding”, JCTVC-S1015, Strasbourg, FR, October 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).