| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0095 |
| --- | --- |

| Title: | CE2 Test 4.2: Inter deblocking for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | [Name of sourcing organization, AHG, etc.] |  |  |

_____________________________

# Abstract

This proposal reports results of CE2 Test 4.2 on intra block copy deblocking. In this test, inter deblocking is used for intra block copy.

# Introduction

In the current HEVC SCC Extensions specification, intra block copy (Intra BC) mode is treated as Intra in the deblocking process. However, this filter strength might be too strong especially for the screen contents.

# Proposed method

In the proposed method, Intra BC is treated as inter during the deblocking process instead. Moreover, in order to keep the deblocking module unchanged, after an Intra BC block is decoded and before the deblocking process, it is converted to be a uni-directionally predictive Inter block with the block vector being its motion vector and the current picture being the reference picture.

# Experimental results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM3.0, and the common test conditions specified in Error: Reference source not found are followed. The summarized experimental results of 4:4:4 and 4:2:0 sequences for lossy coding are shown in Error: Reference source not found and Error: Reference source not found, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for 4:4:4 lossy coding

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -1.3% | -1.2% | -0.2% | -1.8% | -1.7% | -0.2% | -1.5% | -1.5% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.5% | -0.6% | -0.1% | -1.1% | -1.2% | -0.1% | -1.2% | -1.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -1.2% | -1.3% | -0.1% | -1.7% | -1.9% | -0.1% | -1.6% | -1.9% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.9% | -0.9% | 0.0% | -1.9% | -1.8% | -0.2% | -2.8% | -2.9% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.3% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 97% |  |  | 96% |  |  | 97% |  |  |
| Dec Time[%] | 96% |  |  | 100% |  |  | 100% |  |  |

Table 2 Experimental results for 4:2:0 lossy coding

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.1% | 0.0% | -0.3% | -0.2% | -0.2% | -0.2% | -1.1% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | 0.1% | -0.1% | -0.3% |
| Animation, 768p | 0.0% | 0.3% | -0.5% | 0.0% | 0.1% | -1.1% | 0.0% | 0.0% | -0.5% |
| Average of all sequences | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% | -0.4% | -0.1% | -0.1% | -0.8% |
| Enc Time[%] | 97% |  |  | 97% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 101% |  |  | 100% |  |  |

# References

[1] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding”, JCTVC-S1015, Strasbourg, FR, October 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).