| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0096 |
| --- | --- |

| Title: | CE2 Test 6.2: Intra block copy block vector prediction with line buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal reports results of SCCE1 Test 3.1 on block vector prediction method for intra block copy. In the proposed method, the line buffer constraint is removed in the block vector prediction. The performance is reported to be an average BD-rate of -0.5% and -0.5%, respectively for 1080p & 720p RGB and YUV SC text & graphics with motion sequences under AI configuration.

# Introduction

In the current intra block copy (Intra BC) mode, the above spatial neighbor Intra BC blocks are considered as unavailable in the block vector (BV) predictor derivation process if these blocks are out of current CTU.

# Proposed method

In HEVC AMVP, the above spatial neighbors can be out of current CTU, and line buffer is needed to store the motion vector (MVs) of the above row. Thus, this line buffer can be shared with Intra BC mode such that the above spatial neighbors can be used for BV predictor as well with no additional buffer requirement.

# Experimental results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM3.0, and the common test conditions specified in Error: Reference source not found are followed. The summarized experimental results of 4:4:4 format sequences for lossy and lossless coding are shown in Error: Reference source not found and Error: Reference source not found, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for lossy coding

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.4% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.5% | -0.2% | -0.2% | -0.3% | -0.1% | -0.2% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.3% | -0.2% | 0.0% | -0.2% | -0.2% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 97% |  |  | 96% |  |  | 96% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  | 98% |  |  |

Table 2 Experimental results for lossless coding

|  | All Intra | Random Access | Low Delay B |
| --- | --- | --- | --- |
|  | Bit-rate change (Average) | Bit-rate change (Average) | Bit-rate change (Average) |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.1% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% | 96% | 95% |
| Dec Time[%] | 95% | 98% | 97% |

# References

[1] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding”, JCTVC-S1015, Strasbourg, FR, October 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).