| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0097-r1 |
| --- | --- |

| Title: | Non-CE2: Block vector prediction method for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution a block vector prediction method is proposed for Intra block copy. In the proposed method, the total number of prediction candidates has increased. The performance is reported to be an average BD-rate of -2.0% and -2.2%, respectively for 1080p & 720p RGB and YUV SC text & graphics with motion sequences under AI configuration.

# Introduction

For intra block copy (Intra BC) in the current SCC draft specification, the block vector (BV) predictor candidate list is constructed by first accessing in order the spatial neighboring blocks a1 and b1 as shown in Error: Reference source not found. If not both of the spatial neighbors contain BVs, the last N coded BVs, which are initialized with (-2*CUw, 0) and (-CUw, 0), are used to fill into the block vector candidate list to make it contain two different entries, where CUw is the CU width.

Figure 1 Spatial neighbors for BV prediction

# Proposed method

In the proposed method, the total number of BVs in the candidate list is increased to 5, and the candidate list is constructed by first accessing in order the spatial neighboring blocks {a1, b1, b0, a0, b2} as shown in Error: Reference source not found. If the list is not full, the temporal BV predictor (TBVP) and the last 2 coded BVs are added to the candidate list. At last, the following 5 predefined BVs in the checking order of {(-2*PUw, 0), (-PUw, 0), (0, -2*PUh), (0, -PUh), (-PUw, -PUh)} are added until the candidate list is full, where PUw and PUh are the width and height of an Intra BC block.

# Experimental results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM3.0, and the common test conditions specified in Error: Reference source not found are followed. To be consistent with CE2 tests, similar encoder optimization methods as in CE2 Test 3 [2] are employed. The summarized experimental results of 4:4:4 format sequences for lossy and lossless coding are shown in Error: Reference source not found-4, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for lossy coding without TBVP

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.0% | -2.0% | -2.0% | -1.8% | -1.7% | -1.7% | -1.4% | -1.1% | -1.0% |
| RGB, mixed content, 1440p & 1080p | -1.2% | -1.2% | -1.3% | -1.0% | -1.0% | -1.0% | -0.9% | -0.7% | -0.9% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.2% | -2.2% | -2.0% | -2.1% | -2.1% | -2.0% | -1.6% | -1.5% | -1.6% |
| YUV, mixed content, 1440p & 1080p | -1.4% | -1.3% | -1.4% | -1.4% | -1.3% | -1.4% | -1.5% | -1.0% | -1.1% |
| YUV, Animation, 720p | -0.3% | -0.4% | -0.4% | -0.2% | -0.5% | -0.1% | 0.0% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.1% | 0.1% | -0.1% |
| Enc Time[%] | 119% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |

Table 2 Experimental results for lossless coding without TBVP

|  | All Intra | Random Access | Low Delay B |
| --- | --- | --- | --- |
|  | Bit-rate change (Average) | Bit-rate change (Average) | Bit-rate change (Average) |
| RGB, text & graphics with motion, 1080p & 720p | -1.4% | -1.1% | -1.0% |
| RGB, mixed content, 1440p & 1080p | -0.7% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.9% | -0.9% | -0.9% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 120% | 105% | 104% |
| Dec Time[%] | 100% | 101% | 102% |

Table 3 Experimental results for lossy coding with TBVP

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.0% | -2.0% | -2.0% | -2.1% | -1.9% | -1.9% | -1.8% | -1.4% | -1.3% |
| RGB, mixed content, 1440p & 1080p | -1.2% | -1.2% | -1.3% | -1.0% | -1.0% | -1.1% | -1.1% | -0.8% | -1.0% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.2% | -2.2% | -2.0% | -2.4% | -2.3% | -2.2% | -2.0% | -1.8% | -1.9% |
| YUV, mixed content, 1440p & 1080p | -1.4% | -1.3% | -1.4% | -1.4% | -1.2% | -1.3% | -1.5% | -1.2% | -1.4% |
| YUV, Animation, 720p | -0.3% | -0.4% | -0.4% | -0.2% | -0.5% | -0.2% | -0.1% | 0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% | -0.2% |
| Enc Time[%] | 120% |  |  | 105% |  |  | 101% |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  | 106% |  |  |

Table 4 Experimental results for lossless coding with TBVP

|  | All Intra | Random Access | Low Delay B |
| --- | --- | --- | --- |
|  | Bit-rate change (Average) | Bit-rate change (Average) | Bit-rate change (Average) |
| RGB, text & graphics with motion, 1080p & 720p | -1.4% | -1.3% | -1.2% |
| RGB, mixed content, 1440p & 1080p | -0.7% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.9% | -1.2% | -1.1% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 121% | 105% | 104% |
| Dec Time[%] | 100% | 101% | 104% |

# References

[1] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding”, JCTVC-S1015, Strasbourg, FR, October 2014.

[2] S. Liu, C. Pang, and J. Xu, “Description of Core Experiment 2 (CE2): Intra block copy relationship to inter coding”, JCTVC-S1102, Strasbourg, FR, October 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).