| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0098-r1 |
| --- | --- |

| Title: | Non-CE2: Zero merging candidates derivation for Intra BC/Inter signalling unification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to change the zero merging candidates derivation for Intra BC/Inter signaling unification in CE2 Test 1. In the proposed method, when the reference picture is the current picture, pre-defined MVs instead of zero MVs are used for zero merging candidates. The performance is reported to be an average BD-rate of -1.2% and -1.6%, respectively for 1080p & 720p RGB and YUV SC text & graphics with motion sequences under AI configuration.

# Introduction

In CE2 Test 1 [1], the Intra BC mode is signalled reusing inter signalling by adding the current picture (without in-loop filtering) into the reference picture list. With the shared signaling, Intra BC can be differentiated from the conventional Inter mode by checking the reference index used in the prediction. If the corresponding reference picture is the current picture, the prediction mode is Intra BC mode. Otherwise, it is conventional Inter mode.

With the unified signaling, HEVC merge mode is supported for both Intra BC and Inter. In HEVC MV merging candidates derivation, when the candidate list is not full, zero MV merging candidates are used to fill the remaining entries in the candidate list. However, because zero MV is always invalid for Intra BC, the zero merging candidates will never be used for Intra BC.

# Proposed method

In this contribution, it is proposed to change the zero merging candidate derivation for Intra BC/Inter signaling unification. First, five predefined MVs (in quarter-pel precision), {(-4 * CUs, 0), (-8*CUs, 0), (0, -4 * CUs), (0, -8 * CUs), (-4 * CUs, -4 * CUs)}, are defined, where CUs is the CU size. The derivation for the reference indices are kept the same as in HEVC. However, when the corresponding reference picture is the current picture, the above predefined MVs are used instead of using zero MVs; otherwise, zero MVs are used as in HEVC.

# Discussion on Intra BC constraints

In the current SCC, to make sure that the reference block for Intra BC is valid, the following bitstream conformance constraints are exerted:

- The reference block should be within the same slice/tile.

- The reference block cannot overlap with current CU.

- All the pixels inside the reference block should be reconstructed.

- The reference block should be within a certain region due to the parallelization implementation consideration.

- When constrained intra prediction is enabled, the reference block cannot include any pixel coded with the conventional inter mode.

However, nonconforming bitstream is generated in case any of these constraints is violated. To avoid this, one solution is to convert the final Intra BC block vector (BV), either from BV prediction or merge, to be a valid BV at the decoder if possible. Another solution is to replace the unavailable pixels with predefined values, such as 0, or 1 << (B - 1), where B is the bitdepth.

# Experimental results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of CE2 Test 1 software, and the common test conditions specified in Error: Reference source not found are followed. The summarized experimental results for lossy and lossless coding are shown in Table 1 and Table 2. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for lossy coding

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.2% | -2.7% | -2.6% | -2.4% | -4.2% | -4.2% | -2.4% | -3.7% | -3.6% |
| RGB, mixed content, 1440p & 1080p | -0.8% | -1.4% | -1.5% | -0.9% | -2.3% | -2.4% | -1.1% | -2.3% | -2.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | -0.3% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.6% | -2.9% | -3.0% | -2.6% | -4.3% | -4.4% | -2.4% | -4.0% | -4.0% |
| YUV, mixed content, 1440p & 1080p | -1.1% | -2.0% | -2.1% | -1.2% | -3.0% | -3.3% | -1.4% | -4.0% | -4.2% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.3% | -0.2% | -0.7% | -0.3% | 0.0% | -0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.1% | 0.1% | -0.1% |
| Enc Time[%] | 109% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  | 100% |  |  |

Table 2 Experimental results for lossless coding

|  | All Intra | Random Access | Low Delay B |
| --- | --- | --- | --- |
|  | Bit-rate change (Average) | Bit-rate change (Average) | Bit-rate change (Average) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.7% | -0.8% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.8% | -0.3% | -0.5% |
| YUV, mixed content, 1440p & 1080p | 0.2% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% | 100% | 100% |
| Dec Time[%] | 98% | 99% | 102% |

# Draft Text Changes

The proposed text changes are based on the draft text of CE2 Test 1. The changes are marked in yellow.

- 8.5.3.2.5 Derivation process for zero motion vector merging candidates

Inputs to this process are:

- a variable nCbS specifying the size of the current luma coding block

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0zeroCandm and refIdxL10zeroCandm of every new candidate zeroCandm added into mergeCandList during the invocation of this process,

- the prediction list utilization flags predFlagL0zeroCandm and predFlagL10zeroCandm of every new candidate zeroCandm added into mergeCandList during the invocation of this process,

- the motion vectors mvL0zeroCandm and mvL10zeroCandm of every new candidate zeroCandm added into mergeCandList during the invocation of this process.

The variable numRefIdx is derived as follows:

- If slice_type is equal to P, numRefIdx is set equal to num_ref_idx_l0_active_minus1 + 1.

- Otherwise (slice_type is equal to B), numRefIdx is set equal to Min( num_ref_idx_l0_active_minus1 + 1, num_ref_idx_l1_active_minus1 + 1 ).

The variables bvIntraVirtual[ i ][ j ] (with i being 0, 1, 2, 3, 4, and j being equal to 0 or 1) specify five virtual motion vectors are set as follows:

bvIntraVirtual[ 0 ][ 0 ] = -4 *  nCbS,	bvIntraVirtual[ 0 ][ 1 ] = 0;

bvIntraVirtual[ 0 ][ 0 ] = -8 *  nCbS,	bvIntraVirtual[ 0 ][ 1 ] = 0;

bvIntraVirtual[ 0 ][ 0 ] = 0,		bvIntraVirtual[ 0 ][ 1 ] = -4 *  nCbS;

bvIntraVirtual[ 0 ][ 0 ] = 0,		bvIntraVirtual[ 0 ][ 1 ] = -8 *  nCbS;

bvIntraVirtual[ 0 ][ 0 ] = -4 *  nCbS,	bvIntraVirtual[ 0 ][ 1 ] = -4 *  nCbS;

When numCurrMergeCand is less than MaxNumMergeCand, the variable numInputMergeCand is set equal to numCurrMergeCand, the variable zeroIdx is set equal to 0, and the following steps are repeated until numCurrMergeCand is equal to MaxNumMergeCand:

- For the derivation of the reference indices, the prediction list utilization flags and the motion vectors of the zero motion vector merging candidate, the following applies:

    - If slice_type is equal to P, by the candidate zeroCandm with m equal to ( numCurrMergeCand − numInputMergeCand ) is added at the end of mergeCandList, i.e. mergeCandList[ numCurrMergeCand ] is set equal to zeroCandm, and the reference indices, the prediction list utilization flags, and the motion vectors of zeroCandm are derived as follows and numCurrMergeCand is incremented by 1:

refIdxL0zeroCandm = ( zeroIdx < numRefIdx ) ? zeroIdx : 0	(8-1)

refIdxL1zeroCandm = −1	(8-2)

predFlagL0zeroCandm = 1	(8-3)

predFlagL1zeroCandm = 0	(8-4)

Let is_curr_picture_flag_L0 be whether the reference picture in reference list 0 indicated by refIdxL0zeroCandm is the current picture, the motion vectors of zeroCandm are derived as follows and numCurrMergeCand is incremented by 1:

mvL0zeroCandm[ 0 ] = is_curr_picture_flag_L0 ? bvIntraVirtual[zeroIdx ][ 0 ] : 0	(8-5)

mvL0zeroCandm[ 1 ] = is_curr_picture_flag_L0 ? bvIntraVirtual[zeroIdx ][ 1 ] : 0	(8-6)

mvL1zeroCandm[ 0 ] = 0	(8-7)

mvL1zeroCandm[ 1 ] = 0	(8-8)

numCurrMergeCand = numCurrMergeCand + 1	(8-9)

    - Otherwise (slice_type is equal to B), the candidate zeroCandm with m equal to ( numCurrMergeCand − numInputMergeCand ) is added at the end of mergeCandList, i.e. mergeCandList[ numCurrMergeCand ] is set equal to zeroCandm, and the reference indices, the prediction list utilization flags, and the motion vectors of zeroCandm are derived as follows and numCurrMergeCand is incremented by 1:

refIdxL0zeroCandm = ( zeroIdx < numRefIdx ) ? zeroIdx : 0	(8-10)

refIdxL1zeroCandm = ( zeroIdx < numRefIdx ) ? zeroIdx : 0	(8-11)

predFlagL0zeroCandm = 1	(8-12)

predFlagL1zeroCandm = 1	(8-13)

Let is_curr_picture_flag_L0 and is_curr_picture_flag_L1 be whether the reference picture in reference list 0 and reference list 1 indicated by refIdxL0zeroCandm  and refIdxL1zeroCandm is the current picture, the motion vectors of zeroCandm are derived as follows and numCurrMergeCand is incremented by 1:

mvL0zeroCandm[ 0 ] = is_curr_picture_flag_L0 ? bvIntraVirtual[zeroIdx ][ 0 ] : 0	(8-14)

mvL0zeroCandm[ 1 ] = is_curr_picture_flag_L0 ? bvIntraVirtual[zeroIdx ][ 1 ] : 0	(8-15)

mvL1zeroCandm[ 0 ] = is_curr_picture_flag_L1 ? bvIntraVirtual[zeroIdx ][ 0 ] : 0	(8-16)

mvL1zeroCandm[ 1 ] = is_curr_picture_flag_L1 ? bvIntraVirtual[zeroIdx ][ 1 ] : 0	(8-17)

numCurrMergeCand = numCurrMergeCand + 1	(8-18)

- The variable zeroIdx is incremented by 1.

# References

[1] S. Liu, C. Pang, and J. Xu, “Description of Core Experiment 2 (CE2): Intra block copy relationship to inter coding”, JCTVC-S1102, Strasbourg, FR, October 2014.

[2] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding”, JCTVC-S1015, Strasbourg, FR, October 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).