| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0099 |
| --- | --- |

| Title: | Adaptive motion vector resolution for non-4:4:4 formats |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Adaptive MV resolution was adopted in the HEVC SCC Extensions draft specification, and MV can be signalled in the units of 1-pixel or ¼-pixel adaptively. In this contribution, it is proposed to round MV to integer-pel precision for chroma components in non-4:4:4 formats when the MV is signalled in the units of 1-pixel. Experimental results demonstrate that the proposed method leads to 0.0% and 0.1% coding performance degradation under RA and LD configurations, respectively.

# Introduction

At the 19th JCT-VC meeting, adaptive MV resolution was adopted in the HEVC SCC Extensions draft specification, and MV can be signalled in the units of 1 pixel or ¼-pixel adaptively. The integer-pel precision of MV not only saves signalling bits, but also reduces the complexity of both encoder and decoder as no interpolation filter is needed in motion compensation process. However, this only holds for 4:4:4 formats. For non-4:4:4 formats, the integer-pel precision MV may point to fractional-pel position for chroma components where interpolation filters are still necessary.

# Proposed method

In this contribution, it is proposed to round MV to integer-pel precision for chroma components in non-4:4:4 formats when the MV is signalled with integer-pel precision. Then, in this case, the interpolation filter can be avoided for both luma and chroma components.

# Experimental results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM3.0, and the common test conditions specified in Error: Reference source not found are followed. The summarized experimental results of 4:2:0 format sequences for lossy and lossless coding are shown in Table 1 and Table 2, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for lossy coding

|  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% | 0.2% | 0.4% | -0.5% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | -0.3% |
| Enc Time[%] | 96% |  |  | 95% |  |  |
| Dec Time[%] | 99% |  |  | 94% |  |  |

Table 2 Experimental results for lossless coding

|  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 99% |  |  |  | 96% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 88% |  |  |  |

# Draft Text Changes

The proposed text changes are based on the document JCTVC-S1005-v1. The changes are marked in yellow.

- 8.5.3.2.10 Derivation process for chroma motion vectors

This process is only invoked when ChromaArrayType is not equal to 0.

Input to this process is a luma motion vector mvLX.

Output of this process is a chroma motion vector mvCLX.

A chroma motion vector is derived from the corresponding luma motion vector.

For the derivation of the chroma motion vector mvCLX, the following applies:

mvCLX[ 0 ] = ((mvLX[ 0 ] >> use_integer_mv_flag) << ( 1 + use_integer_mv_flag)) / SubWidthC	(8-1)

mvCLX[ 1 ] = ((mvLX[ 1 ] >> use_integer_mv_flag) << ( 1 + use_integer_mv_flag)) / SubHeightC	(8-2)

# References

[1] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding”, JCTVC-S1015, Strasbourg, FR, October 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).