| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0101r3 |
| --- | --- |

| Title: | Content light level information SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg, Jim Helman MovieLabs 475 Sansome Street, Suite 740 San Francisco, CA 94111 Michael Smith, Michael Zink Warner Bros. 3400 W. Riverside Drive Burbank, CA 91505 | Email: | chadfogg@gmail.com jhelman@movielabs.com miksmith@gmail.com Michael.Zink@warnerbros.com |
| Source: | Motion Picture Laboratories Inc. and Warner Bros. |  |  |

_____________________________

# Abstract

This proposal requests a new AVC and HEVC static metadata SEI “Content light level information” (CLL) to indicate maximum intensities (in candelas per square metre) of video contained in the current layer of the coded video stream (CVS). The CLL variables Maximum Frame Average Light Level (MaxFALL) and Maximum Content Light Level (MaxCLL) are measurements or limits applied across the span of content during the authoring or encoding process as defined by Bluray Disc Association’s Blu-Ray 2.0 specification and CEA 861.3.

# Introduction

Examples of how to compute MaxFALL and MaxCLL are included in an accompanying slide presentation (JCTVC-T0101_slides.pdf) from Warner Bros. A liaison statement from Blu-Ray Disc Association (BDA) [1], which, with permission from the BDA video chair, is attached within this JCTVC-T0101-v4.zip as the file “BDA_TechInfo_UHDHDR_VideoProperty_Nov_2014.pdf”, describes an application use case of MaxFALL and MaxCLL defined by CEA 816.3  [2].

The model motivated by LCD filter white backlight absorption, applicable to the semantics definition below, translates real value content primary pixel levels (ER, EG, EB), as defined in HEVC and AVC Annex E (VUI), into CIE 1931 luminance by the following formula:

MaxE = Max( ER, EG, EB ).

Pixel intensity = RGBtoCIEY * [MaxE, MaxE, MaxE] * Uc

Where the 3x1 conversion matrix appropriate for colour_primaries==9 (Bt.2020):

RGBtoCIEY = [0.262700, 0.677998, 0.059302]

..the constant Uc (intensity at unity) taken from Table E-4 for transfer_characteristics = 16 (ST 2084) is:

Uc = 10000 candelas per square metre.

Note, when colour_primaries == 11 (XYZ), the above pixel intensity formula can be simplified as: Y*Uc

# Proposed syntax and semantics

| content_light_level_info ( payloadSize ) { | Descriptor |
| --- | --- |
| max_content_light_level | u(16) |
| max_frame_average_light_level | u(16) |
| } |  |

max_content_light_level indicates the maximum pixel intensity, in units of 1 candela per square metre, of all samples within the bitstream content. When not present, or the value of max_content_light_level is equal to 0, the value of max_content_light_level shall be regarded as unknown.

max_frame_average_light_level  indicates the maximum pixel intensity average of all frames, in units of 1 candela per square metre, of the bitstream content. When not present, or the value of max_frame_average_light_level is equal to 0, the value of max_frame_average_light_level shall be regarded as unknown.

pixel intensity is the equivalent luminance value computed from a single-pixel array of "linear domain” samples, where the array consists of one sample from each component at the same sample picture location, each equal to the maximum component, Emax, of the content at that sample location, where Emax = Max( ER, EG, EB ).

NOTE: Since the value of the pixel intensity at each picture location is computed from component samples all equal to Emax, it is possible that the true CIE Y Luminance value at that location is less than pixel intensity. This situation may occur, for example, when bright blue content samples (ER= 0,  EG = 0, EB >> 0) dominate the Emax calculation. Since the blue component EB contributes approximately 10% to the true CIE Y Luminance value, the true CIE Y Luminance value of the example blue pixel would therefore only be approximately 10% of pixel intensity.

NOTE: the frame-average computation used in estimating max_frame_average_light_level should only be performed on the active frame area of the video data, as for example conveyed by parameters such as black bar metadata or internal authoring or encoding knowledge.  If the coded video content is in “letterbox” format (e.g. where a 2.40:1 aspect ratio square pixel frame is fitted inside a 16:9 square pixel image container with coded black bar areas on the top and bottom within the conformance window of the frame), the black bar areas should not be part of the active image area and therefore are not included in the frame-average computation.  This permits max_frame_average_light_level to remain an upper bound on the maximum frame-average light level, even if image zooming or pan/scan is performed as a post-processing operation.

# References

[1] m36116, “Liaison from the Blu-Ray Disc Association”, 111th Meeting of MPEG, Geneva, Feb. 2015

[2] Consumer Electronics Association 861.3 (2015), HDR Static Metadata Extensions

# Patent rights declaration(s)

Motion Pictures Laboratories Inc. does not have any current or pending patent rights relating to the technology described in this contribution.

Warner Brothers does not have any current or pending patent rights relating to the technology described in this contribution.