| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0103_r3 |
| --- | --- |

| Title: | VUI proposal |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg, MovieLabs 475 Sansome Street, Suite 740 San Francisco, CA 94111 | Email: | chadfogg@gmail.com |
| Source: | Motion Picture Laboratories Inc.. |  |  |

_____________________________

# Abstract

This proposal points out that not all transfer functions described in video usability information (VUI) have stated the luminance intensity factors and therefore the intensities at zero (E′Y = 0.0) and unity (E′Y = 1.0) or points in between cannot be known from the information in Annex E alone. One of the design goals of VUI has been to serve as a stand-alone specification for implementers to unambiguously translate sample code levels output by the decoder into linear primary display intensities (optical output values), usually defined in candela per square meter multiplied by the real-value (L) of the EOTF output. Standards such as Bt.709 listed in Annex E tables E-3, E-4, and E-5 “Informative Remark” columns serve only as informative (non-normative) references, and therefore those references are not intended as required guides for that translation. Inputs to this JCT Geneva meeting from NHK (JCTVC-T0035) and others at previous meetings state that the luminance multiplication factor at unity (V=1.0 or E′Y = 0.0),  or identified by other points along the transfer function curve such as V=0.5, have been a variable in some applications for some number of years. The proposal from NHK and others propose a new transfer function that varies from previously identified transfer functions in VUI, mainly, if not solely, by the intensity factor of the transfer function. To complete the VUI description, this contribution proposes that a new SEI be defined to provide that intensity variable as, at least, non-normative information (a hint or clue), along with any other missing information in the VUI specification that has been historically assumed, undocumented, or only stated in the documents pointed to by the VUI table references, or references that those documents in turn reference.  This contribution also states that modern role of the transfer function is to render a video signal perceptually uniform (as possible) -- a preferred domain for encoders to operate upon.  Recent inputs to JCT have not provided that background, and that of the history of CRT television devices where some transfer functions have originated.

# Introduction

In section 2 of this proposal, information necessary to convey luminance intensity factor information for [1] is provided in a new SEI vui_info() proposed below.  The location of unity and zero on the integer code levels (or real signals just prior to integer quantization) varies by more than just  video_full_range_flag = 1 as stated in Annex E [2], but also the transfer function itself defined by other standards such as ST 2084, matrix_coeffs, and the application (such as file-based exchange or SDI transfer). To resolve this ambiguity, it is proposed to explicitly state the code level that corresponds to the zero and unity locations of the transfer function.

For an explanation of the origins of transfer functions, such as the relationship between voltage and current established by early 20th century Cathode Ray Tubes (CRT), see [3][4]. This, and other background information, are recommended for study in light of recent proposals [1].

# Proposal

The recommended level of explicit syntax for describing missing VUI parameters is described below:

## Syntax

| vui_info ( payloadSize ) { | Descriptor |
| --- | --- |
| vui_info_luminance_at_unity | u(16) |
| vui_info_luminance_at_zero | u(16) |
| vui_info_transfer_function_unity_code_level | u(16) |
| vui_info_transfer_function_zero_code_level | u(16) |
| } |  |

## Semantics

vui_info_luminance_at_unity indicates luminance in units of 1 candela per square meter of the content at the unity (E′Y = 1.0) of the transfer function indicated by transfer_characteristics in the video usability information parameters (Annex E).

vui_info_luminance_at_zero indicates luminance in units of 0.0001 candela per square meter of the content at the zero (E′Y = 0.0) of the transfer function indicated by transfer_characteristics in the video usability information parameters (Annex E).

vui_info_transfer_function_unity_code_level indicates the code level between 0 and 2^BitDepth_Y – 1 that corresponds to unity  (E′Y = 1.0) of the transfer function indicated by transfer_characteristics in the video usability information parameters (Annex E).

vui_info_transfer_function_zero_code_level indicates the code level between 0 and 2^BitDepth_Y – 1 that corresponds to zero  (E′Y = 0.0) of the transfer function indicated by transfer_characteristics in the video usability information parameters (Annex E).

## alternative syntax and semantics

A more compact alternative to the syntax table in section 2.1 above is provided below:

| vui_info ( payloadSize ) { | Descriptor |
| --- | --- |
| vui_info_luminance_at_unity | u(16) |
| vui_info_luminance_at_zero | u(16) |
| vui_info_range_type | u(8) |
| } |  |

vui_info_range_type indicates the range information according to the table below:

| vui_info_range_type |  |
| --- | --- |
| 0 | Y = Floor((219*D* N + 4*D + 0.5) C = Floor((224*D* N + 64*D + 0.5) |
| 1 | Floor((2BitDepth – 1) N + 0.5) |
| 2 | Floor((1015÷4)*D* N + D + 0.5) [not originally defined for 8-bit in ST 2084] |
| 3 | SMPTE RP-2077 protected full range (SMPTE ST 292) |
| 4-255 | reserved |

[ The interpretation of vui_info_range_type order is an extension of video_full_range_flag ]

where

N is the nonlinear signal (a.k.a. E′Y  or V ), nominally from zero (0.0) to unity (1.0)

D = 2BitDepth – 8

C = integer (non-linear / OETF output ) chroma (Cb´, Cr´ or Dz´,Dx´)

Y = integer (non-linear / OETF output) luma (Y´) or R´, G´, or B´.

For vui_info_range_type = 3 (SMPTE RP-2077 [5] protected full range), the protected code value CVP is computed as:

CVP = clamp( 2BitDepth-8,  2 BitDepth - 2BitDepth-8 - 1, CVFULL)

where

| clamp(a, b, x) = | x, if x  [a, b] |
| --- | --- |
|  | a if x < a |
|  | b if x > b |

CVFULL is the code value computed by vui_info_range_type 1 (video_full_range = 1)

## alternative with table and conditional zero and unity indicators

An alternative to the explicit syntax elements is suggested, provided that SEI messages cannot be refined in future editions without contradicting earlier definitions:

| vui_info ( payloadSize ) { | Descriptor |
| --- | --- |
| vui_info_luminance_at_unity | u(16) |
| vui_info_luminance_at_zero | u(16) |
| vui_info_range_type | u(8) |
| if( vui_info_range_type == 4 ){ |  |
| vui_info_transfer_function_unity_code_level | u(16) |
| vui_info_transfer_function_zero_code_level | u(16) |
| } |  |
| } |  |

| vui_info_range_type |  |
| --- | --- |
| 0 | Y = Floor((219*D* N + 4*D + 0.5) C = Floor((224*D* N + 512*D + 0.5) |
| 1 | Floor((2BitDepth – 1) N + 0.5) |
| 2 | Floor(((1015÷4)*D* N + D + 0.5) |
| 3 | SMPTE RP-2077 protected full range (SMPTE ST 292) |
| 4 | Specified by vui_info_transfer_function_zero_code_level and vui_info_transfer_function_unity_code_level |
| 5-255 | reserved |

Where:

N is the nonlinear signal (a.k.a. E′Y  or V ), nominally from zero (0.0) to unity (1.0)

D = 2BitDepth – 8

C = integer (non-linear / OETF output ) chroma (Cb´, Cr´ or Dz´,Dx´)

Y = integer (non-linear / OETF output) luma (Y´) or R´, G´, or B´.

## Explicit indicators alternative with protected range indicators

For the greatest future proof indications, no tables are relied upon in this syntax table

| vui_info ( payloadSize ) { | Descriptor |
| --- | --- |
| vui_info_luminance_at_unity | u(16) |
| vui_info_luminance_at_zero | u(16) |
| vui_info_range_type | u(8) |
| vui_info_transfer_function_unity_code_level | u(16) |
| vui_info_transfer_function_zero_code_level | u(16) |
| vui_info_protected_code_level_lower_range_start | u(16) |
| vui_info_protected_code_level_lower_range_end | u(16) |
| vui_info_protected_code_level_upper_range_start | u(16) |
| vui_info_protected_code_level_upper_range_end | u(16) |
| } |  |

vui_info_protected_code_level_lower_range_start indicates the first code level within the lower protected range.

vui_info_protected_code_level_lower_range_end indicates the last code level within the lower protected range.

vui_info_protected_code_level_upper_range_start indicates the first code level within the upper protected range.

vui_info_protected_code_level_upper_range_end indicates the last code level within the upper protected range.

| vui_info_range_type |  |
| --- | --- |
| 0 | Y = Floor((876*D* N + 4*D + 0.5) C = Floor((896*D* N + 512*D + 0.5) |
| 1 | Floor((2BitDepth – 1) N + 0.5) |
| 2 | Floor((1015*D* N + 4*D + 0.5) |
| 3 | SMPTE RP-2077 protected full range (SMPTE ST 292) |
| 4 | Specified by vui_info_transfer_function_zero_code_level and vui_info_transfer_function_unity_code_level |
| 5 | Specified by vui_info_transfer_function_zero_code_level , vui_info_transfer_function_unity_code_level, vui_info_protected_code_level_lower_range_start, vui_info_protected_code_level_lower_range_end, vui_info_protected_code_level_upper_range_start, vui_info_protected_code_level_upper_range_end |
| 6-255 | reserved |

# Examples

## Examples of zero and unity code level indicators

The formulae for computing the integer quantized code level (value) from real-value non-linear signal (output of OETF) are given in the table below, along and 10-bit zero and unity code level examples in the right two columns:

| Spec. | Real value non-linear N to integer map | BitDepth = 10 |  |
| --- | --- | --- | --- |
|  |  | Zero | Unity |
| ST 2084, RP-2077 | CVF = Floor((2BitDepth – 1) N + 0.5) | 0 | 1023 |
| RP-2077 protected range | CVF = Floor((2BitDepth – 1) N + 0.5) | 0 | 1023 |
| ST 2084 SDI (and BT.2020 full range?) | CVS = Floor((1015*D* N + 4*D + 0.5) | 4 | 1019 |
| BT.709, BT.2020 ST 2085 legal range | Y = Floor((876*D* N + 4*D + 0.5) | 64 | 940 |
|  | C = Floor((896*D* N + 512*D + 0.5) | N/A | N/A |

## Example of protected range indicators

For 10-bit protected range SMPTE 292 defined in SMPTE RP-2077:

| vui_info_protected_code_level_lower_range_start | 0 |
| --- | --- |
| vui_info_protected_code_level_lower_range_end | 3 |
| vui_info_protected_code_level_upper_range_start | 1020 |
| vui_info_protected_code_level_upper_range_end | 1023 |

Note that for 10-bit SMPTE RP-2077,  code values 0-3 are clamped to code value 4, and code levels 1020-1023 are clamped to code level 1019. The corresponding intensities that would map to those dark (E′Y <= ~3/1023)  and bright (E′Y > ~1019/1023) levels cannot be uniquely represented in RP-2077 protected mode (for example: vui_info_range_type = 3 above), despite the full range integer representation of the non-clamped code values in between.  However, the ST 2084 (and BT.2020?) definition of full range scales unity and zero to fit within the non-protected range, at the expense of slightly denser level steps (~1/1017) in the non-protected range.

## Examples of transfer function intensity

From table E-4 (Annex E) of the HEVC specification, only VUI transfer_characteristics 16 and 17 have intensity stated in the table definition formula column (not reproduced here). All other entries do not make an intensity statement.  The table below attempts to fill in the missing information:

| Value | Intensity at unity (candela /m2) | Intensity at zero (candela /m2) | Informative Remark |
| --- | --- | --- | --- |
| 1 | ~100 | ~0 | BT.709-5 |
| 4 | ~100 | ~0 | BT.470-6 (NTSC) BT.1700 |
| 5 | ~100 | ~0 | BT.470-6 |
| 6 | ~100 | ~0 | BT.601 SMPTE 170M |
| 7 | ~100 | ~0 | SMPTE 240M |
| 8 | ? | ? | Linear transfer characteristics |
| 9 | ? | ? | Logarithmic transfer characteristic (100:1 range) |
| 10 | ? | ? | Logarithmic transfer characteristic (100 * Sqrt( 10 ) : 1 range) |
| 11 | ? | ? | IEC 61966-2-4 (xvYCC) |
| 12 | 200? | ~0 | BT.1361 |
| 13 | ~100 | ~0 | IEC 61966-2-1 (sRGB or sYCC) |
| 14 | 100 | ~0 | Rec. ITU-R BT.2020 (10-bit) |
| 15 | 100 | ~0 | Rec. ITU-R BT.2020 (12-bit) |
| 16 | 10,000 | 0 | SMPTE ST 2084 (PQ) |
| 17 | 48 | 0 | SMPTE ST 428-1 (Digital Cinema) |

Intensity at zero is usually implicitly computed from intensity at unity, transfer function formula, and BitDepth.

# References

[1] JCTVC-T0035, “NHK’s proposal for an extended image dynamic range television (EIDRTV) system”, Jan. 2015

[2] JCTVC-R1013, “Edition 2 Draft Text of High Efficiency Video Coding (HEVC), Including Format Range (RExt), Scalability (SHVC), and Multi-View (MV-HEVC) Extensions”, (last version) Oct. 2014.

[3] http://www.johnhearfield.com/WSA/Valve.htm

[4] http://www.poynton.com/GammaFAQ.html

[5] SMPTE RP 2077, “Full-Range Image Mapping”

[6] SMPTE ST 2084, “High Dynamic Range Electro-Optical Transfer Function of Mastering Reference Displays”

[7] SMPTE ST 2085, “Y′D′ZD′X Color-Difference Computations for High Dynamic Range X′Y′Z′ Signals”

# Patent rights declaration(s)

Motion Pictures Laboratories Inc. does not have any current or pending patent rights relating to the technology described in this contribution.