| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0104 |
| --- | --- |

| Title: | HM reference encoder study |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chad Fogg, MovieLabs 475 Sansome Street, Suite 740 San Francisco, CA 94111 | Email: | chadfogg@gmail.com |
| Source: | Motion Picture Laboratories Inc.. |  |  |

_____________________________

# Abstract

This information document explores an alternative, non-normative approach to improving utilization of code levels and resulting coding efficiency, unlike the normative system changes (albeit no decoder reconstruction changes) to techniques such as the code level redistribution or luminance scaling described in JCTVC-T0102.  One of the key benefits of the methods in JCTVC-T0102 is to, in effect, assign fewer code levels to dark, usually noise-ridden areas of the transfer function, thus reducing entropy of the final integer signal in the container conversion steps that take video signal from linear light to the non-linear (perceptually uniform) quantized/integer signal which is passed through a codec chain.  A non-normative container or system solution would be to reduce the entropy while retaining detail as much as possible. No changes are required to either the signal container (relationship between integer sample code levels and display intensity) or decoder semantics. A motion-compensated noise reduction algorithm from isovideo is utilized in this study.

# Introduction

The processing steps are as follows:

10-bit Bt.2020 PQ YCbCr Non-constant luminance .yuv   noise reduction  HM 16.3 encoder

VUI settings reflecting the signal container are:

colour_primaries = 9, transfer_characteristics = 16, matrix_coeffs = 9, video_full_range=0

The encoder is HM HEVC Main10 Common conditions HE-RA with intra frame interval of 24 frames (1 second GOP).  Test clips are from StEM: http://www.dcimovies.com/2014_StEM_Access/

The accompanying .xls file provides statistics and plots.  The results show that film grain contributes significantly to PSNR energy calculations in the test content, and therefore visual subjective results should instead be evaluated as hinted by the Constant Rate Factor results in the .xls plots.