| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10 Feb. – 18 Feb. 2015 | Document: JCTVC-T0105 |
| --- | --- |

| Title: | CE3: Results of Test A on Intra Line Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, 1Chun-Chi Chen, 2Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw 2wpeng@cs.nctu.edu.tw |
| Source: | National Chiao Tung University (NCTU)/ITRI International |  |  |

_____________________________

# Abstract

In the 19th JCT-VC meeting, the Core Experiment 3 (JCTVC-S1103) was formed to study intra line copy (IntraLC) techniques. This contribution is based on JCTVC-S0136 to evaluate the coding performance of IntraLC under three configurations of search range: (A1) full frame for 4x8, 8x8, 16x16, and 32x32 IntraBC PUs, 2 CTUs for 4x4 and 8x4 IntraBC PUs, 2 CTUs for IntraLC PUs; (A2) full frame for 4x8, 8x8, 16x16, and 32x32 IntraBC PUs, 4 CTUs for 4x4 and 8x4 IntraBC PUs, 4 CTUs for IntraLC PUs; (A3) 4 CTUs for 4x4, 8x4, 4x8, 8x8, 16x16, and 32x32 IntraBC PUs, 4 CTUs for IntraLC PUs. Their coding results are reported respectively as follows:

- Test A1

  - RGB, text & graphics with motion, 1080p & 720p	: (AI) -2.0%,	(RA) -1.2%, 	(LB) -0.6%;

  - RGB, mixed content, 1440p & 1080p		: (AI) -1.6%, 	(RA) -1.1%, 	(LB) -0.6%;

  - YUV, text & graphics with motion, 1080p & 720p	: (AI) -1.8%,	(RA) -1.1%, 	(LB) -0.5%;

  - YUV, mixed content, 1440p & 1080p		: (AI) -1.5%, 	(RA) -1.1%, 	(LB) -0.5%;

  - Encoding time					: (AI) 115%, 	(RA) 107%, 	(LB) 102%;

  - Decoding time					: (AI) 101%, 	(RA) 100%, 	(LB) 102%.

- Test A2

  - RGB, text & graphics with motion, 1080p & 720p	: (AI) -2.7%,	(RA) -1.7%, 	(LB) -1.0%;

  - RGB, mixed content, 1440p & 1080p		: (AI) -2.0%, 	(RA) -1.3%, 	(LB) -0.5%;

  - YUV, text & graphics with motion, 1080p & 720p	: (AI) -2.4%,	(RA) -1.5%, 	(LB) -0.9%;

  - YUV, mixed content, 1440p & 1080p		: (AI) -1.9%, 	(RA) -1.4%, 	(LB) -0.7%;

  - Encoding time					: (AI) 116%, 	(RA) 107%, 	(LB) 102%;

  - Decoding time					: (AI) 101%, 	(RA) 101%, 	(LB) 103%.

- Test A3

  - RGB, text & graphics with motion, 1080p & 720p	: (AI) -3.2%,	(RA) -2.1%, 	(LB) -1.2%;

  - RGB, mixed content, 1440p & 1080p		: (AI) -2.5%, 	(RA) -1.7%, 	(LB) -0.9%;

  - YUV, text & graphics with motion, 1080p & 720p	: (AI) -2.9%,	(RA) -1.9%, 	(LB) -0.9%;

  - YUV, mixed content, 1440p & 1080p		: (AI) -2.4%, 	(RA) -1.7%, 	(LB) -0.9%;

  - Encoding time					: (AI) 113%, 	(RA) 102%, 	(LB) 102%;

  - Decoding time					: (AI) 101%, 	(RA) 99%, 	(LB) 102%.

# Introduction

The IntraLC [1] mode divides a prediction unit into multiple 1-pixel-wide rows or columns, termed as “lines”. Its operation is very similar to that of IntraBC, which is to perform the intra-copying operation within a given search range (e.g. 2 CTUs, 4 CTUs) inside the current frame and pixels within the current CU cannot be used as reference. The major difference between IntraBC and IntraLC is that the basic processing unit for the latter one is a line of pixels but not a block of pixels. Syntax-wise, a PU-level flag is introduced when intra_bc_flag is enabled to indicate whether the IntraLC mode is used or not. For each IntraLC PU, another flag is sent to indicate whether a PU is split row-wise or column-wise (Figure 1). For the 1st line in a PU, the BV prediction is the same as that of IntraBC mode (Figure 2); for the rest, the BV of (n-1)th line is used for the BV prediction of nth line, where n is larger than 1. The residual coding of IntraLC is the same as that for IntraBC except that RDPCM is disabled when IntraLC mode is enabled.

In order to speed up the IntraLC search, four fast algorithms as follows are applied:

- IntraLC is evaluated only at the smallest CU level.

- The NxN IntraLC mode will not be evaluated.

- Nx2N/2NxN IntraLC modes will not be evaluated if the encoder skips the evaluation for the Nx2N/2NxN IntraBC modes.

- The encoder firstly searches horizontally and vertically within the given search range and then searches the pixel positions indicated by using a hash-based technique [2] with the modification that only the most significant 4 bits of each sample are involved in the calculation of hash keys.

Figure 1. An example of IntraLC with PU-level splitting.

Figure 2. An example of IntraLC BVp.

# Experimental Results

The experiments were conducted using HM-16.2+SCM-3.0 software [3] and the screen content common test conditions [4] to collect the BD-rate and the encoding and decoding times of the proposed technique under the All-Intra, Random Access, and Low Delay B test conditions. Three different settings to the search range are listed in Table 1.

Table 1. Search range of each tests.

| PU Size | 4x4/8x4 IntraBC | 4x8/8x8/16x16/32x32 IntraBC | IntraLC |
| --- | --- | --- | --- |
| Test A1 | 1x2-CTU | Full-frame | 1x2-CTU |
| Test A2 | 1x4-CTU | Full-frame | 1x4-CTU |
| Test A3 | 1x4-CTU | 1x4-CTU | 1x4-CTU |

## Coding Performance

Table 2. BD-rate reduction and processing time ratio of Test A1 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.0% | -1.7% | -1.8% | -1.2% | -1.0% | -1.0% | -0.6% | -0.5% | -0.5% |
| RGB, mixed content, 1440p & 1080p | -1.6% | -1.5% | -1.5% | -1.1% | -0.9% | -1.0% | -0.6% | -0.2% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -1.8% | -1.6% | -1.6% | -1.1% | -0.9% | -1.0% | -0.5% | -0.4% | -0.5% |
| YUV, mixed content, 1440p & 1080p | -1.5% | -1.4% | -1.4% | -1.1% | -0.8% | -1.0% | -0.5% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.2% | -0.1% | 0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 115% |  |  | 107% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 102% |  |  |

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | -2.0% | -2.2% | -2.1% | -2.0% | -2.3% | -2.0% | -1.1% | -0.9% | -1.8% |
| Mixed content, 480p | -0.7% | -0.6% | -0.7% | -0.2% | -0.2% | -0.3% | 0.0% | -0.1% | 0.3% |
| Animation, 768p | -2.4% | -2.2% | -1.8% | -0.8% | -1.6% | -0.8% | -0.1% | -0.5% | -0.9% |
| Average of all sequences | -1.8% | -1.8% | -1.6% | -1.2% | -1.6% | -1.3% | -0.6% | -0.6% | -1.0% |
| Enc Time[%] | 118% |  |  | 101% |  |  | 102% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  | 102% |  |  |

Table 3. BD-rate reduction and processing time ratio of Test A1 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -1.6% | -1.9% | -6.5% | -0.2% | -1.0% | -1.3% | -4.6% | -0.1% | -1.0% | -1.0% | -3.3% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.5% | -0.6% | -0.8% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.9% | -2.1% | -6.1% | -0.3% | -1.2% | -1.4% | -4.5% | -0.2% | -1.2% | -1.2% | -3.4% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.7% | -0.7% | -0.9% | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 116% |  |  |  | 106% |  |  |  | 107% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 101% |  |  |  | 101% |  |  |  |

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| Text & graphics with motion, 720p | -1.4% | -1.4% | -2.5% | -0.2% | -0.3% | -0.7% | -1.3% | -0.1% | -0.1% | -0.2% | -0.3% | -0.1% |
| Mixed content, 480p | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | -0.6% | -0.6% | -0.6% | -0.6% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | -0.7% | -0.9% | -2.5% | -0.1% | 0.0% | -0.4% | -1.3% | 0.0% | 0.0% | -0.1% | -0.3% | 0.0% |
| Enc Time[%] | 125% |  |  |  | 105% |  |  |  | 103% |  |  |  |
| Dec Time[%] | 107% |  |  |  | 101% |  |  |  | 102% |  |  |  |

Table 4. BD-rate reduction and processing time ratio of Test A2 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.7% | -2.4% | -2.5% | -1.7% | -1.4% | -1.5% | -1.0% | -0.8% | -0.8% |
| RGB, mixed content, 1440p & 1080p | -2.0% | -1.8% | -1.9% | -1.3% | -1.2% | -1.2% | -0.5% | -0.3% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.4% | -2.2% | -2.4% | -1.5% | -1.3% | -1.4% | -0.9% | -0.7% | -0.9% |
| YUV, mixed content, 1440p & 1080p | -1.9% | -1.8% | -1.7% | -1.4% | -1.2% | -1.3% | -0.7% | -0.7% | -0.6% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 116% |  |  | 107% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 103% |  |  |

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | -2.2% | -2.5% | -2.4% | -2.2% | -2.3% | -2.4% | -1.2% | -1.8% | -0.7% |
| Mixed content, 480p | -0.8% | -0.8% | -0.8% | -0.2% | -0.4% | -0.8% | 0.0% | 0.3% | -0.1% |
| Animation, 768p | -2.5% | -2.3% | -1.9% | -0.8% | -1.5% | -0.8% | -0.3% | -0.6% | -1.0% |
| Average of all sequences | -1.9% | -2.0% | -1.9% | -1.4% | -1.6% | -1.6% | -0.7% | -1.0% | -0.6% |
| Enc Time[%] | 119% |  |  | 104% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 96% |  |  | 95% |  |  |

Table 5. BD-rate reduction and processing time ratio of Test A2 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -2.1% | -2.6% | -8.2% | -0.3% | -1.4% | -1.8% | -6.0% | -0.2% | -1.3% | -1.4% | -4.5% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.7% | -0.7% | -1.0% | -0.3% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.5% | -2.9% | -8.0% | -0.5% | -1.6% | -2.0% | -6.0% | -0.2% | -1.6% | -1.7% | -4.7% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.8% | -0.8% | -1.2% | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% |  |  |  | 107% |  |  |  | 107% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  | 101% |  |  |  |

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| Text & graphics with motion, 720p | -1.7% | -1.7% | -3.1% | -0.2% | -0.5% | -1.0% | -1.8% | -0.2% | -0.2% | -0.4% | -0.6% | -0.1% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | -0.6% | -0.6% | -0.6% | -0.6% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | -0.8% | -1.0% | -3.1% | 0.0% | -0.1% | -0.5% | -1.8% | 0.0% | 0.0% | -0.2% | -0.6% | 0.0% |
| Enc Time[%] | 143% |  |  |  | 107% |  |  |  | 105% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  | 102% |  |  |  |

Table 6. BD-rate reduction and processing time ratio of Test A3 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.2% | -2.8% | -3.0% | -2.1% | -1.7% | -1.9% | -1.2% | -0.9% | -0.9% |
| RGB, mixed content, 1440p & 1080p | -2.5% | -2.2% | -2.3% | -1.7% | -1.5% | -1.6% | -0.9% | -0.4% | -0.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.9% | -2.5% | -2.8% | -1.9% | -1.6% | -1.7% | -0.9% | -0.7% | -0.7% |
| YUV, mixed content, 1440p & 1080p | -2.4% | -2.2% | -2.2% | -1.7% | -1.6% | -1.5% | -0.9% | -0.6% | -1.0% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 113% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  | 102% |  |  |

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | -2.7% | -3.1% | -3.0% | -2.6% | -2.8% | -2.8% | -1.4% | -2.2% | -1.3% |
| Mixed content, 480p | -0.8% | -0.8% | -0.8% | -0.3% | -0.3% | -0.2% | -0.1% | 0.0% | -0.4% |
| Animation, 768p | -2.5% | -2.3% | -1.9% | -0.8% | -1.6% | -0.5% | -0.3% | -1.3% | -0.4% |
| Average of all sequences | -2.2% | -2.3% | -2.2% | -1.6% | -1.9% | -1.6% | -0.8% | -1.4% | -0.9% |
| Enc Time[%] | 120% |  |  | 104% |  |  | 101% |  |  |
| Dec Time[%] | 96% |  |  | 100% |  |  | 96% |  |  |

Table 7. BD-rate reduction and processing time ratio of Test A3 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -2.7% | -3.2% | -9.2% | -0.4% | -2.1% | -2.2% | -6.9% | -0.2% | -2.1% | -1.8% | -4.9% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.8% | -0.9% | -1.2% | -0.3% | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -3.2% | -3.6% | -9.3% | -0.5% | -2.4% | -2.6% | -7.0% | -0.2% | -2.3% | -2.0% | -5.1% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -1.0% | -1.1% | -1.4% | -0.4% | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 112% |  |  |  | 103% |  |  |  | 103% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  | 101% |  |  |  |

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| Text & graphics with motion, 720p | -2.3% | -2.2% | -4.1% | -0.3% | -0.6% | -1.1% | -2.0% | -0.2% | -0.2% | -0.4% | -0.6% | -0.1% |
| Mixed content, 480p | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | -0.6% | -0.6% | -0.6% | -0.6% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | -1.0% | -1.3% | -4.1% | -0.1% | -0.1% | -0.6% | -2.0% | 0.0% | 0.0% | -0.2% | -0.6% | 0.0% |
| Enc Time[%] | 121% |  |  |  | 107% |  |  |  | 104% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  | 100% |  |  |  |

## Memory Access Bandwidth

The worst-case memory bandwidth of IntraLC with different memory access patterns specified in JCTVC-S0308 [5] is reported as follows:

Table 8. p-value of inter, IntraBC, and IntraLC mode.

|  | Configurations |  |  |  |  |  |  |  | Per-pixel Memory Access Bandwidth (3 Component 4:4:4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Requested Block Size |  | Num. of Samples | IF Length (Y) | IF Length (U/V) | Bi Pred? | Memory Access Pattern | m= | 8 | 8 | 4 | 4 | 8 | 16 |
| Prediction Mode | M | N | MxN | L | L | Y or N |  | n= | 2 | 4 | 2 | 4 | 8 | 16 |
| 8x8, Bi-prediction | 8 | 8 | 64 | 8 | 6 | Y |  |  | 33 | 39 | 24 | 28.5 | 54 | 96 |
| 4x8, Inter | 4 | 8 | 32 | 8 | 6 | N |  |  | 26 | 31 | 18.5 | 22 | 42 | 96 |
| 8x4, Inter | 8 | 4 | 32 | 8 | 6 | N |  |  | 24 | 30 | 17.5 | 22 | 42 | 96 |
| 4x4, IntraBC | 4 | 4 | 16 | 1 | 1 | N |  |  | 18 | 24 | 9 | 12 | 48 | 192 |
| 1x4, IntraLC | 1 | 4 | 4 | 1 | 1 | N |  |  | 36 | 48 | 18 | 24 | 96 | 384 |
| 4x1, IntraLC | 4 | 1 | 4 | 1 | 1 | N |  |  | 24 | 48 | 12 | 24 | 96 | 384 |

## CABAC Throughput

The number of context-coded bins of IntraLC in the worse-case scenario is reported as follows:

Table 9. CABAC throughput of inter, IntraBC, and IntraLC mode.

| Prediction Mode (Note: CU Size = 8x8) | 4x8, Uni Prediction (SCM-3.0) | 4x4, IntraBC (SCM-3.0) | 1x4, IntraLC |
| --- | --- | --- | --- |
| Number of Context-coded Bins | 488 | 482 | 486 |
| Bins per Pixel | 7.63 | 7.56 | 7.63 |

# Conclusion

In this contribution, the coding performance of IntraLC under three different configurations of the search range is tested. Up to 3.2% BD-rate saving is observed in the All-Intra configuration in Test A3. According to the interesting gains, we would recommend the committee to adopt this coding mode in SCC draft standard.

# References

- C. C. Chen, Y. Chen, J. Z. Xu, T. Lin, and W. Wang, “Description of Core Experiment 3 (CE3): Intra Line Copy and Intra String Copy,” JCTVC-S1103, Strasbourg, Oct. 2014.

- B. Li, J. Xu, F Wu, X. Guo, and G. J. Sullivan, “Description of screen content coding technology proposal by Microsoft,” JCTVC-Q0035, Valencia, Mar. 2014.

- JCT-VC, “HM-16.2+SCM-3.0,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2+SCM-3.0

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-S1015, Strasbourg, Oct. 2014.

- Y. Chen, “BoG report on Intra String Copy (CE10),” JCTVC-S0308, Strasbourg, Oct. 2014.

# Appendix: Text

The yellow markers indicate the changes to the text of HEVC Screen Content Coding Draft Text 2.

7.3.8.6	Prediction unit syntax

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| … |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { /* Intra BC*/ |  |
| intra_lc_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_lc_flag[ x0 ][ y0 ] ) |  |
| row_splitting_flag[ x0 ][ y0 ] | ae(v) |
| for( i = 0; i < (intra_lc_flag[ x0 ][ y0 ] ? ( row_splitting_flag[ x0 ][ y0 ] ? nPbH : nPbW ) : 1 ); i++ ) { |  |
| bvd_coding( x0, y0, 2 ) |  |
| if ( i == 0) |  |
| bvp_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| … |  |

7.3.8.13	Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ¦ ¦ ( intra_bc_flag[ x0 ][ y0 ] & !intra_lc_flag[x0][y0] ) ) && explicit_rdpcm_enabled_flag && ( transform_skip_flag[ x0 ][ y0 ][ cIdx ] ¦ ¦ cu_transquant_bypass_flag ) ) { |  |
| explicit_rdpcm_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| … |  |

7.4.9.7	Prediction unit semantics

intra_lc_flag[ x0 ][ y0 ] equal to 1 specifies that the current prediction unit is coded in intra line copying mode. intra_lc_flag[ x0 ][ y0 ] equal to 0 specifies that the current prediction unit is coded in intra block copying mode. When not present, the value of intra_lc_flag is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

row_splitting_flag[ x0 ][ y0 ] equal to 1 specifies that the current prediction unit is row-wise splitting. row_splitting_flag[ x0 ][ y0 ] equal to 0 specifies that the current prediction unit is column-wise splitting. When not present, the value of row_splitting_flag is inferred to be equal to 0.

7.4.9.10	Motion vector difference semantics

The variable BvdIntra[ x0 ][ y0 ][ compIdx ] specifies the difference between a vector component to be used for the intra block copying prediction mode and its prediction. The value of BvdIntra[ x0 ][ y0 ][ compIdx ] shall be in the range of −128 to 128, inclusive [Ed. (RJ): Please check the range]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1. When the considered prediction block is coded in intra line copying mode, the array indices i, j specify the location (i, j) of the top-left luma sample of the considered block relative to the top-left luma sample of the prediction block. Otherwise, i and j are inferred to be equal to 0.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise, if refList is equal to 1, MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 2), BvdIntra[ x0 + i ][ y0 + j ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1 [Ed. (RJ): Please check this part.]

# Patent rights declaration(s)

NCTU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).