| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0111 |
| --- | --- |

| Title: | Results of CE3 Test B.4.1: Constrained run for Intra String Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou Ying Chen Vadim Seregin Marta Karczewicz 5775 Morehouse Drive San Diego, CA, 92122 | Tel: Email: | +1-858-845-1115 fzou@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal presents results by constraining the current Intra String Copy method to reduce worst case memory bandwidth access. The implementation is based on the common CE software of the current CE3. It has been discussed that it would be necessary to reduce the memory bandwidth and one way is to access at lease T pixels during each String Copy. The simulation results show that such a constraint (with T equal to 4) as implemented based on CE3 common software leads to 0.0% and 0.1% bitrate increase for text & graphics with motion, 1080p & 720p RGB and YUV respectively under CTC. In that case, the coding efficiency benefit of the Intra String Copy method drops to 0.3% for text & graphics with motion, 1080p & 720p RGB under CTC and 0.4% when 4 CTUs are constrained for both Intra string copy and Intra Block Copy.

# Introduction

In the 19th JCT-VC meeting in Strasbourg, CE3 [1] was established to study different Intra String Copy methods. In CE3, a common CE software is designed based on JCTVC-R0098 [2] and JCTVC-R0140 [3]. However, the matching string in the common software can be as short as 1 pixel, which requires much more memory access compared with Intra BC. JCTVC-S0175 [4] proposed the constrained run for Intra String Copy and is studied in CE3 Test B.4.1 to reduce the memory access.

# Simulation results

The proposed scheme is implemented on CE3 common software and tested using the common test condition defined in [5]. Table 1 demonstrates the coding performance under full frame Intra BC test conditions with 2CTU Intra String Copy when matching length constraint T=4 is applied.

Table 1. Results of constraint run (T = 4) under the common test condition (Test Condition 1 in CE3)

|  |  | CE3 SW |  |  | Test B.4.1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | All Intra |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.4% | -0.3% | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.4% | -0.3% | -0.2% | -0.2% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 133% |  |  | 134% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

Table 2 demonstrates the coding performance under the 4CTU Intra BC and 4CTU intra string copy test conditions when matching length constraint T=4 is applied.

Table 2. Results of constraint run (T = 4) under the 4CTU test condition (Test Condition 2 in CE3)

|  |  | CE3 SW |  |  | Test B.4.1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | All Intra |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.6% | -0.6% | -0.6% | -0.4% | -0.4% | -0.4% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% | -0.3% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 127% |  |  | 126% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

## Worst case memory access for intra string copy

In order to compare the Intra string copy methods with the current design, we make the following assumptions as discussed in BOG report JCTVC-S0308 [6]. Note that the following equation is considered for calculating the memory bandwidth for each component of the pixels within an MxN block, wherein L is the filter length and mxn is the memory access pattern.

However, the worst case memory access calculation of intra string copy with constraint run is slightly different. Figure 1 illustrates the worst case for an 8x8 CU block for an Intra String Copy block, where the matching string extends from one row to the following row. Correspondingly, the worst case memory access is calculated as follows

P = [6 * 3 * (m * n) *2 + 2 * 2 * (m * n) * 2]/(8 * 8)

Figure 1. Worst memory access for 8x8 CU for intra string copy

Memory pattern 8x2 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |
| --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 6 | 18 |
| Intra LC | 1 | 1 | 4 | 12 | 36 |
|  | 1 | 4 | 1 | 8 | 24 |
| Inter 8x8 | 8/6 | 8 | 8 | 12/10.5 | 33 |
| Intra String Copy | 1 | 1 | 1 | 16 | 48 |
| Intra String Copy (T=4) | 1 | 4 | 1 | 11 | 33 |

Memory pattern 8x4 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |
| --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 8 | 24 |
| Intra LC | 1 | 1 | 4 | 16 | 48 |
|  | 1 | 4 | 1 | 16 | 48 |
| Inter 8x8 | 8/6 | 8 | 8 | 16/12 | 39 |
| Intra String Copy | 1 | 1 | 1 | 32 | 96 |
| Intra String Copy (T=4) | 1 | 4 | 1 | 22 | 66 |

Memory pattern 4x2 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |
| --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 3 | 9 |
| Intra LC | 1 | 1 | 4 | 6 | 18 |
|  | 1 | 4 | 1 | 4 | 12 |
| Inter 8x8 | 8/6 | 8 | 8 | 10/7 | 24 |
| Intra String Copy | 1 | 1 | 1 | 8 | 24 |
| Intra String Copy (T=4) | 1 | 4 | 1 | 5.5 | 16.5 |

Memory pattern 4x4 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |
| --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 4 | 12 |
| Intra LC | 1 | 1 | 4 | 8 | 24 |
|  | 1 | 4 | 1 | 8 | 12 |
| Inter 8x8 | 8/6 | 8 | 8 | 12.5/8 | 28.5 |
| Intra String Copy | 1 | 1 | 1 | 16 | 48 |
| Intra String Copy (T=4) | 1 | 4 | 1 | 11 | 33 |

Memory pattern 8x8 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |
| --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 16 | 48 |
| Intra LC | 1 | 1 | 4 | 16 | 96 |
|  | 1 | 4 | 1 | 32 | 96 |
| Inter 8x8 | 8/6 | 8 | 8 | 18/18 | 54 |
| Intra String Copy | 1 | 1 | 1 | 64 | 192 |
| Intra String Copy (T=4) | 1 | 4 | 1 | 44 | 132 |

# Conclusion

The proposed constraint on minimum run (T =4) for intra string copy reduced the worst case memory access with negligible performance loss. Therefore, it is proposed to be included in the intra string copy methods for further evaluations.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-C. Chen, Y. Chen, J. Xu, T. Lin and W. Wang, “Description of Core Experiment 3 (CE3): Intra Line Copy and Intra String Copy”, JCT-VC Document, JCTVC-S1103, Strasbourg, France, Oct. 2014.

- B. Li and J. Xu, “SCCE4: Result of Test 3.1”, JCT-VC Document, JCTVC-R0098, Sapporo, Japan, Jun. 2014.

- F. Zou, Y. Chen, J. Sole and M. Karczewicz, “SCCE4: Test 3.2 Pixel based 1D dictionary coding”, JCT-VC Document, JCTVC-R0140, Sapporo, Japan, Jun. 2014.

- F. Zou, Y. Chen, V. Seregin and M. Karczewicz, “CE10: Test 7.1 Constrained run for Intra String Copy”, JCT-VC Document, JCTVC-S0175, Strasbourg, France, Oct. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu , “Common conditions for screen content coding tests”, JCTVC-S1015, Strasbourg, France, Oct. 2014.

- Y. Chen, “BoG report on Intra String Copy (CE10)”, JCT-VC Document, JCTVC-S0308, Strasbourg, France, Oct. 2014.

- Text Modifications

The text provided in this section is based on JCTVC-R1005_v2.

The text supports both 2D matching dictionary mode (supporting adaptive horizontal and vertical scanning) and 1D matching dictionary mode.

- 7.3.2.2.3	Sequence parameter set scc extensions syntax

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| dictionary_mode_enabled_flag | u(1) |
| } |  |

- 7.3.8.5	Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if(!intra_bc_flag[ x0 ][ y0 ] && dictionary_mode_enabled_flag ) |  |
| dictonary_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !dictionary_mode_flag[ x0 ][ y0 ]) |  |
| pred_mode_flag | ae(v) |
| if( palette_mode_enabled_flag && ChromaArrayType = = 3 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_mode_flag[ x0 ][ y0 ] ) |  |
| palette_coding( x0, y0, nCbS ) |  |
| else { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && !intra_bc_flag[ x0 ][ y0 ] ] && !dictionary_mode_flag[ x0 ][ y0 ] &&) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } else if (dictionary_mode_flag[ x0 ][ y0 ] ) { |  |
| dictionary_mode( x0, y0, log2CbSize) |  |
| if( !pcm_flag[ x0 ][ y0 ] && !dictionary_mode_flag[ x0 ][ y0 ]) { |  |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| if( residual_adaptive_colour_transform_enabled_flag && ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ intra_chroma_pred_mode[ x0 ][ y0 ] = = 4 ) ) |  |
| cu_residual_act_flag |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.8.X Dictionary mode syntax

| dictionary_mode( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| nCbS = ( 1 << log2CbSize ) |  |
| totalPixel = nCbS*nCbS |  |
| currPixel = 0 |  |
| currIndex = 0 |  |
| dictionary_type_flag[ x0 ][ y0 ] | ae(v) |
| dictionary_scan_flag[ x0 ][ y0 ] | ae(v) |
| while (currPixel < totalPixel ) { |  |
| dictionay_pred_mode_flag[ x0 ][ y0 ][ currIndex ] | ae(v) |
| if( dictionay_pred_mode_flag[ x0 ][ y0 ][ currIndex ] ) { |  |
| if( dictionary_type_flag[ x0 ][ y0 ] == 0 ) { |  |
| dictionay_pred_offset_minus1[ x0 ][ y0 ][ currIndex ] | ae(v) |
| } else { |  |
| dictionay_pred_offsetX[ x0 ][ y0 ][ currIndex ] | ae(v) |
| dictionay_pred_offsetY[ x0 ][ y0 ][ currIndex ] | ae(v) |
| } |  |
| dictionary_pred_length_minus4[ x0 ][ y0 ][ currIndex ] | ae(v) |
| currPixel += (dictionary_pred_length_minus1 [ x0 ][ y0 ][ currIndex ] + 1) |  |
| } else { |  |
| dictionary_direct_component[ x0 ][ y0 ][ currIndex ][ 0 ] | ae(v) |
| dictionary_direct_component[ x0 ][ y0 ][ currIndex ][ 1 ] | ae(v) |
| dictionary_direct_component[ x0 ][ y0 ][ currIndex ][ 2 ] | ae(v) |
| currPixel += 1 |  |
| } |  |
| currIndex += 1 |  |
| } |  |
| } |  |

- 7.4.3.2.3	Sequence parameter set scc extension

dictionary_mode_enabled_flag equal to 1 specifies that dictionary mode may be invoked in the decoding process for intra prediction. dictionary_mode_enabled_flag equal to 0 specifies that dictionary mode is not applied. When not present, the value of dictionary_mode_enabled_flag is inferred to be equal to 0.

7.4.9.5 Coding unit semantics

dictionary_mode_flag [ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in dictionary mode. dictionary_mode_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in dictionary mode. When palette_mode[ x0 ][ y0 ] is not present, it is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

7.4.9.X Dictionary mode semantics

dictionary_type_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded in dictionary mode and the prediction comes from the dictionary maintained by the decoder. dictionary_type_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in dictionary mode and the prediction comes from the previously reconstructed pixels belonging to the same picture. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionary_scan_flag[ x0 ][ y0 ] equal to 0 specifies that the horizontal scanning is applied to current coding unit. dictionary_scan_flag[ x0 ][ y0 ] equal to 1 specifies that the vertical scanning is applied to current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionay_pred_mode_flag[ x0 ][ y0 ][ i ] equal to 1 specifies the prediction mode is applied to the i-th decoding sub-string in the current CU. dictionay_pred_mode_flag[ x0 ][ y0 ][ i ] equal to 0 specifies the direct mode is applied to the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionay_pred_offset_minus1[ x0 ][ y0 ][ i ] plus 1 specifies the offset of the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionay_pred_offsetX[ x0 ][ y0 ][ i ] specifies the horizontal offset of the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionay_pred_offsetY[ x0 ][ y0 ][ i ] specifies the vertical offset of the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionary_pred_length_minus4[ x0 ][ y0 ][ i ] plus 4 specifies the length of the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionary_direct_component[ x0 ][ y0 ][ i ][ comp ] specifies the pixel value of the comp-th color component in the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

8.X Decoding process for dictionary mode

An array DictHor[] is used to maintain the virtual dictionary for normal dictionary mode in horizontal scanning. An array DictVer[] is used to maintain the virtual dictionary for normal dictionary mode in vertical scanning. Each element in the dictionary is a pixel including three components.

At the beginning of decoding one slice DictHor and DictVer are set to empty.

After decoding one CTU, if the size of DictHor is greater than (1<<18)*3/2, the oldest (1<<18)/2 elements are removed from the dictionary. After decoding one CTU, if the size of DictVer is greater than (1<<18)*3/2, the oldest (1<<18)/2 elements are removed from the dictionary.

If the current CU uses normal dictionary mode with horizontal scanning, 8.X.1 will be invoked.

If the current CU uses normal dictionary mode with vertical scanning, 8.X.2 will be invoked.

If the current CU uses reconstruction based dictionary mode with horizontal scanning, 8.X.3 will be invoked.

If the current CU uses reconstruction based dictionary mode with vertical scanning, 8.X.4 will be invoked.

After decoding one CU, if the current CU uses dictionary mode, the following applies.

If the current CU uses normal dictionary mode with horizontal scanning, the pixels in the current CU will also be added to DictVer using vertical scanning order.

Otherwise, if the current CU uses normal dictionary mode with vertical scanning, the pixels in the current CU will also be added to DictHor using horizontal scanning order.

Otherwise, (the current CU uses reconstruction based dictionary mode), the pixels in the current CU will added to both DictVer using vertical scanning order and DictHor using horizontal scanning order.

8.X.1 Decoding of CU in normal dictionary mode with horizontal scanning

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

–	arrays of dictInfoMatchFlag, dictInfoMatchOffset, dictInfoMatchOffset, dictInfoPixel containing the dictionary information.

Output of this process is a modified reconstructed picture before deblocking filtering.

decodedCount is set to 0.

currIdx is set to 0.

nCbS is set to 1<<log2CbSize.

totalPixel is set to nCbS*nCbS.

currentDictSize is set to the current size of DictHor.

While decodedCount is smaller than totalPixel, the following steps applies.

- If dictInfoMatchFlag[currIdx] is equal to 1,

  - for(i=0;i<dictInfoMatchLength[currIdx];i++)
DictHor[currentDictSize+decodedCount+i] = DictHor[currentDictSize+decodedCount+i-dictInfoMatchOffset[currIdx]]

  - decodedCount += dictInfoMatchLength[currIdx]

- Otherwise (dictInfoMatchFlag[currIdx] is equal to 0)

  - DictHor[currentDictSize+decodedCount]= dictInfoPixel[currIdx]

  - decodedCount += 1

- currIdx++

The pixels at (x, y), where x is in range of  xCb… xCb+nCbS, and y is in range of yCb … yCb+nCbS  will be set to DictHor[currentDictSize+(y-yCb)*nCbS+(x-xCb)]

8.X.2 Decoding of CU in normal dictionary mode with vertical scanning

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

–	arrays of dictInfoMatchFlag, dictInfoMatchOffset, dictInfoMatchOffset, dictInfoPixel containing the dictionary information.

Output of this process is a modified reconstructed picture before deblocking filtering.

decodedCount is set to 0.

currIdx is set to 0.

nCbS is set to 1<<log2CbSize.

totalPixel is set to nCbS*nCbS.

currentDictSize is set to the current size of DictVer.

While decodedCount is smaller than totalPixel, the following steps applies.

- If dictInfoMatchFlag[currIdx] is equal to 1,

  - for(i=0;i<dictInfoMatchLength[currIdx];i++)
DictVer[currentDictSize+decodedCount+i] = DictVer[currentDictSize+decodedCount+i-dictInfoMatchOffset[currIdx]]

  - decodedCount += dictInfoMatchLength[currIdx]

- Otherwise (dictInfoMatchFlag[currIdx] is equal to 0)

  - DictVer[currentDictSize+decodedCount]= dictInfoPixel[currIdx]

  - decodedCount += 1

- currIdx++

The pixels at (x, y), where x is in range of  xCb… xCb+nCbS, and y is in range of yCb … yCb+nCbS  will be set to DictVer[currentDictSize+(x-xCb)*nCbS+(y-yCb)]

8.X.3 Decoding of CU in reconstruction based dictionary mode with horizontal scanning

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

–	arrays of dictInfoMatchFlag, dictInfoMatchOffset, dictInfoMatchOffsetX, dictInfoMatchOffsetY, dictInfoPixel containing the dictionary information.

Output of this process is a modified reconstructed picture before deblocking filtering.

decodedCount is set to 0.

currIdx is set to 0.

nCbS is set to 1<<log2CbSize.

totalPixel is set to nCbS*nCbS.

While decodedCount is smaller than totalPixel, the following steps applies.

- If dictInfoMatchFlag[currIdx] is equal to 1,

  - offsetX = dictInfoMatchOffsetX[currIdx]

  - offsetY = dictInfoMatchOffsetY[currIdx]

  - for(i=0;i<dictInfoMatchLength[currIdx];i++, decodedCount++)
The pixel at (xCb+decodedCount%nCbS,yCb+decocedCount/nCbS) is set to equal to the pixel at (xCb-offsetX+decodedCount%nCbS, yCb-offsetY+decocedCount/nCbS)

- Otherwise (dictInfoMatchFlag[currIdx] is equal to 0)

  - The pixel at (xCb+decodedCount%nCbS,yCb+decocedCount/nCbS) is set to equal to the pixel of dictInfoPixel[currIdx]

  - decodedCount += 1

- currIdx++

8.X.4 Decoding of CU in reconstruction based dictionary mode with vertical scanning

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

–	arrays of dictInfoMatchFlag, dictInfoMatchOffset, dictInfoMatchOffsetX, dictInfoMatchOffsetY, dictInfoPixel containing the dictionary information.

Output of this process is a modified reconstructed picture before deblocking filtering.

decodedCount is set to 0.

currIdx is set to 0.

nCbS is set to 1<<log2CbSize.

totalPixel is set to nCbS*nCbS.

While decodedCount is smaller than totalPixel, the following steps applies.

- If dictInfoMatchFlag[currIdx] is equal to 1,

  - offsetX = dictInfoMatchOffsetX[currIdx]

  - offsetY = dictInfoMatchOffsetY[currIdx]

  - for(i=0;i<dictInfoMatchLength[currIdx];i++, decodedCount++)
The pixel at (xCb+decodedCount/nCbS,yCb+decocedCount%nCbS) is set to equal to the pixel at (xCb-offsetX+decodedCount/nCbS, yCb-offsetY+decocedCount%nCbS)

- Otherwise (dictInfoMatchFlag[currIdx] is equal to 0)

  - The pixel at (xCb+decodedCount/nCbS,yCb+decocedCount%nCbS) is set to equal to the pixel of dictInfoPixel[currIdx]

  - decodedCount += 1

- currIdx++

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| sao( ) | sao_merge_left_flag sao_merge_up_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sao_type_idx_luma sao_type_idx_chroma | Error: Reference source not found | 0 | 1 | 2 |
| coding_quadtree( ) | split_cu_flag[ ][ ] |  | 0..2 | 3..5 | 6..8 |
| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | dictionary_mode_flag | Table 9-X0 | 0 | 1 | 2 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode |  | 0 9..11 | 1..4 | 5..8 |
|  | prev_intra_luma_pred_flag[ ][ ] | Table 9- | 0 | 1 | 2 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | rqt_root_cbf | Error: Reference source not found |  | 0 | 1 |
| dictionary_mode( ) | dictionary_type_flag | Table 9-X1 | 0 | 3 | 6 |
|  | dictionary_scan_flag | Table 9-X1 | 1, 2 | 4, 5 | 7, 8 |
|  | dictionary_pred_mode_flag | Table 9-X2 | 0 | 1 | 2 |
|  | dictionary_pred_offset_minus1 | Table 9-X3 | 0..3 | 4..7 | 8..11 |
|  | dictionary_pred_offsetX dictionary_pred_offsetY | Table 9-X4 | 0 | 1 | 2 |
|  | dictionary_pred_length_minus1 | Table 9-X5 | 0..2 | 3..5 | 6..8 |

Table 9-X0 – Values of initValue for ctxIdx of dictionary_mode_flag

| Initialization variable | ctxIdx of dictionary_mode_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table 9-X1 – Values of initValue for ctxIdx of dictionary_type_flag and dictionary_scan_flag

| Initialization variable | ctxIdx of dictionary_type_flag and dictionary_scan_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

Table 9-X2 – Values of initValue for ctxIdx of dictionary_pred_mode_flag

| Initialization variable | ctxIdx of dictionary_pred_mode_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table 9-X3 – Values of initValue for ctxIdx of dictionary_pred_offset_minus1

| Initialization variable | ctxIdx of dictionary_pred_offset_minus1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

Table 9-X4 – Values of initValue for ctxIdx of dictionary_pred_offsetX and dictionary_pred_offsetY

| Initialization variable | ctxIdx of dictionary_pred_offsetX and dictionary_pred_offsetY |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table 9-X5 – Values of initValue for ctxIdx of dictionary_pred_length_minus1

| Initialization variable | ctxIdx of dictionary_pred_length_minus1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

| Table 9-2 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | intra_bc_flag | FL | cMax = 1 |
|  | dictionary_mode_flag | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | part_mode | Error: Reference source not found | ( xCb, yCb ) = ( x0, y0), log2CbSize |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | prev_intra_luma_pred_flag[ ][ ] | FL | cMax = 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | - |
|  | rqt_root_cbf | FL | cMax = 1 |
| dictionary_mode( ) | dictionary_type_flag | FL | cMax = 1 |
|  | dictionary_scan_flag | FL | cMax = 1 |
|  | dictionary_pred_mode_flag | FL | cMax = 1 |
|  | dictionary_pred_offset_minus1 | 9.3.3.X0 | - |
|  | dictionary_pred_offsetX | 9.3.3.X1 | - |
|  | dictionary_pred_offsetY | 9.3.3.X1 | - |
|  | dictionary_pred_length_minus1 | 9.3.3.X2 | totalPixel, currPixel |
|  | dictionary_direct_component[][][][compIdx] | FL | cMax = compIdx==0 ? (1<<BitDepthY)-1 : (1<<BitDepthC)-1 |

9.3.3.X0 Binarization process for dictionary_pred_offset_minus1

Inputs to this process is a request for a binarization for the syntax element dictionary_pred_offset_minus1.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element dictionary_pred_offset_minus1 is specified in Table 9-XX.

The suffix is binarized by FL with cMax = (1<<number_of_suffix_bins)-1.

Table 9-XX Binarization of dictionary_pred_offset_minus1

| Value | prefix | number_of_suffix_bins | suffix |
| --- | --- | --- | --- |
| [0, 3] | 1 | 2 | value-0 |
| [4, 19] | 01 | 4 | value-4 |
| [20, 275] | 001 | 8 | value-20 |
| [276, 65811] | 0001 | 16 | value-276 |
| Others | 0000 | 19 | value-65811 |

9.3.3.X1 Binarization process for dictionary_pred_offsetX and dictionary_pred_offsetY

Inputs to this process is a request for a binarization for the syntax element dictionary_pred_offsetX or dictionary_pred_offsetY.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element dictionary_pred_offsetX and dictionary_pred_offsetY is specified in Table 9-XX.

The suffix is binarized by EG1.

Table 9-XX Binarization of dictionary_pred_offsetX and dictionary_pred_offsetY

| Value | prefix | suffix |
| --- | --- | --- |
| 0 | 0 | na |
| >0 | 10 | value-1 |
| <0 | 11 | -value-1 |

9.3.3.X2 Binarization process for dictionary_pred_length_minus1

Inputs to this process is a request for a binarization for the syntax element dictionary_pred_length_minus1, number of total pixels in the current CU totalPixel, and the current pixel position currPixel.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element dictionary_pred_length_minus1 is specified in Table 9-XX.

The suffix is binarized by FL with cMax = (1<<number_of_suffix_bins)-1.

Table 9-XX Binarization of dictionary_pred_offset_minus1

| value | totalPixel-currPixel-1 | prefix | number_of_suffix_bins | suffix |
| --- | --- | --- | --- | --- |
| [0, 3] | <4 | na | 2 | value-0 |
|  | others | 1 |  |  |
| [4, 19] | <20 | 0 | 4 | value-4 |
|  | others | 01 |  |  |
| [20, 275] | <276 | 00 | 8 | value-20 |
|  | others | 001 |  |  |
| Others | All | 000 | 12 | value-276 |

| Table 9-3 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_transquant_bypass_flag | 0 | na | na | na | na | na |
| cu_skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| intra_bc_flag | 0 | na | na | na | na | na |
| dictionary_mode_flag | 0 | na | na | na | na | na |
| pred_mode_flag | 0 | na | na | na | na | na |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | na | na |
| part_mode log2CbSize > MinCbLog2SizeY | 0 | 1 | 3 | bypass | na | na |
| pcm_flag[ ][ ] | terminate | na | na | na | na | na |
| …… |  |  |  |  |  |  |
| dictionary_type_flag | 0 | na | na | na | na | na |
| dictionary_scan_flag | dictionary_type_flag | na | na | na | na | na |
| dictionary_pred_mode_flag | 0 | na | na | na | na | na |
| dictionary_pred_offset_minus1 (prefix) | 0 | 1 | 2 | 3 | na | na |
| dictionary_pred_offset_minus1 (suffix) | bypass | bypass | bypass | bypass | bypass | bypass |
| dictionary_pred_offsetX | 0 | bypass | bypass | bypass | bypass | bypass |
| dictionary_pred_offsetY | 0 | bypass | bypass | bypass | bypass | bypass |
| dictionary_pred_length_minus1 (prefix) | 0 | 1 | 2 | na | na | na |
| dictionary_pred_length_minus1 (suffix) | bypass | bypass | bypass | bypass | bypass | bypass |
| dictionary_direct_component | bypass | bypass | bypass | bypass | bypass | bypass |
| …… |  |  |  |  |  |  |