| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0112r1 |
| --- | --- |

| Title: | CE 1 Related: On escape pixel coding for palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou Vadim Seregin Rajan Joshi Marta Karczewicz Wei Pu 5775 Morehouse Drive San Diego, CA, 92122 | Tel: Email: | +1-858-845-1115 fzou@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The existing escape pixel in SCM3.0 and spec has a division problem in calculating the maximum possible quantized value. This proposal presents several solutions to solve this problem. Solution 1 utilizes a look up table of 6 elements to store six scaling factors (the same as the one used in SCM3.0 encoder) to derive the maximum possible quantized escape pixel value. Solution 2 utilizes a look up table of 52 elements storing the maximum value for each QP. Solution 3 utilizes qP and bitDepth to derive the number of bins for fixed length codeword for escape pixels. Solution 4 utilizes a codeword for quantized escape pixels which is independent on the maximum possible values. The proposed solutions are all implemented based on CE3 common software, and the simulation results demonstrate that they all have negligible RD difference under CTC. The existing escape reconstruction has undefined negative right shift operations. It is proposed to be aligned with the current coefficient dequantization to avoid undefined operations. And this has no effect on CTC.

# Introduction

In SCM3.0, truncated binary (TB) codeword is used to signal quantized escape pixels palette_escape_val. To use truncated binary codeword, the total number of possible value is needed. For example, if truncated binary codeword is used to signify a set of N possible values {0, 1, 2, 3, …, N-1}, N is used to construct a truncated binary codeword (where N-1 is the maximum value).

To get maximum quantized escape values, the following procedure is used in SCM3.0:

- The quantization parameter qP is derived as follows:

qP = ( cIdx  = =  0 ) ? Qp′Y : ( ( cIdx  = =  1 ) ? Qp′Cb : Qp′Cr )

- A quantization scaling factor quantScale, rightshift offset rightShiftOffset and rightshift parameter rightShiftParam are derived based on HEVC quantization lookup table as follows

quantScale = g_quantScales[qP%6], where g_quantScales = {26214, 23302, 20560, 18396, 16384, 14564 }

rightShiftParam = 14 +

rightShiftOffset = 1 << (rightShiftParam -1 )

- A quantization step size parameter qStep is derived as follows:

qStep = ( qP= =0 ) ? Round( 2(qP − 4) / 6 ) : 1

- A maximum possible quantized value maxValue is derived as follows:

maxValue = (( 1<<bitDepth ) − 1 ) / Qstep

- The number of bins numBins to represent maxValue is derived as follows

while( maxValue ) {
			maxValue = maxValue >> 1
			numBins++
		}

- The maximum parameter cMax for the truncated binary codeword is derived as follows

cMax = clip3(0, 2numBins-1, ((1<<bitDepth ) − 1 ) *quantScale +rightShiftOffset ) >> rightShiftParam)

Note that during cMax derivation, division operations are invoked which are not desirable at the decoder.

During quantization, the quantized value is constrained in the range of [0, cMax] inclusive

quantizedValue = clip3 (0, cMax, (escapeValue * quantScale +rightShiftOffset ) >> rightShiftParam))

At the decoder, dequantization is applied as follows:

deQuantEspValue = (quantizedValue * invQuantScale[qPrem] + addOffset )>>invQuantRightShift

To solve this problem, four solutions are presented in the Section 2.

- Proposed escape pixel coding

## Using a lookup table of 6 elements

It is proposed to use encoder parameters to derive maximum quantized escape value at the decoder. Thus encoder related parameters need to be stored in a lookup table at the decoder side.

- The quantization parameter qP is derived based on color component:

qP = ( cIdx  = =  0 ) ? Qp′Y : ( ( cIdx  = =  1 ) ? Qp′Cb : Qp′Cr )

- A quantization scaling factor quantScale, a right shift offset rightShiftOffset and a right shift parameter rightShiftParam are derived based on a lookup table stored at the decoder for instance:

quantScale = g_quantScales[qP%6], where g_quantScales = {26214, 23302, 20560, 18396, 16384, 14564 }

rightShiftParam = 14 +

rightShiftOffset = 1 << (rightShiftParam -1 )

- The maximum parameter cMax for the truncated binary codeword is derived as follows

cMax = ((1<<bitDepth ) − 1 ) quantScale +rightShiftOffset ) >> rightShiftParam

Note that in the case, the quantized escape pixel is constraint in the range of [0, cMax] inclusive.

## Using a lookup table of 52 elements

It is proposed to derive the maximum quantized escape pixel value using quantization parameter qP and color component bitDepth. Specifically, cMax can be obtained using a loop up table with qP and bitDepth. The following is an example of cMax derivation:

- The quantization parameter qP is derived based on color component. Different color component can have different quantization parameters. The following is an example to get Qp′Y, Qp′Cb or Qp′Cr for component Y, Cb and Cr respectively, which may or may not be different:

qP = ( cIdx  = =  0 ) ? Qp′Y : ( ( cIdx  = =  1 ) ? Qp′Cb : Qp′Cr )

- A base quantization parameter baseQp is derived with given bitDepth:

baseQp = qP – 6*(bitDepth - 8)

- If baseQp is equal or larger than 0, cMax can be found through lookup table given baseQp as input entry

- Otherwise, (baseQp is negative), an cMax can be derived as follows

cMax = Table[qP]>>(bitDepth - 8)

Table 1. cMax lookup table

| qP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cMax | 255 | 255 | 255 | 255 | 255 | 227 | 202 | 180 | 161 | 143 | 128 | 114 | 101 | 90 | 80 |

| qP | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cMax | 72 | 64 | 57 | 51 | 45 | 40 | 36 | 32 | 28 | 25 | 23 | 20 | 18 | 16 | 14 |

| qP | 30 | 31 | 32 | 32 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cMax | 13 | 11 | 10 | 9 | 8 | 7 | 6 | 6 | 5 | 4 | 4 | 4 | 3 | 3 | 3 |

| qP | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cMax | 2 | 2 | 2 | 2 | 1 | 1 | 1 |

Note that in the case, the quantized escape pixel is constraint in the range of [0, cMax] inclusive.

## Using a fixed length codeword

It is proposed to use qP and bitDepth to derive the number of bits needed to code quantized escape pixels and then usewhixed length codewordcoding. Specifihe following procedure is applied:

- The quantization parameter qP is derived based on color component:

qP = ( cIdx  = =  0 ) ? Qp′Y : ( ( cIdx  = =  1 ) ? Qp′Cb : Qp′Cr )

- A quantization bit reduction factor quantBits is derived using an equation.

quantBits =  (qP  < 4) ? 0: ( qP – 4 ) / 6

- A maximum bit parameter maxBit is derived based on bitdepth and quantBits,:

maxBit =  bitDepth - quantBits

Note that in this case, the quantized escape pixel is constrained in the range of [ 0, 2maxBit-1] inclusive. And then fixed length codeword is used to code quantized escape pixels. During dequantization, the escape pixel is reconstructed by left shifting quantBits:

recSamples[ xCb + x ][ yCb + y ] =

Clip3( 0, (1<<bitDepth)-1,   palette_escape_val[ cIdx ][ xCb + x ][ yCb + y] << quantBits)

## Using a codeword independent of maxValue

It is proposed to use the remaining coefficient coding for quantized escape pixels with Golomb rice parameter being equal to 3.

# Fix of undefined dequantization of escape pixel

- The existing escape reconstruction has undefined negative right shift operations. It is proposed to be aligned with the current coefficient dequantization to avoid undefined operations. And this has no effect on CTC. Specifically, the following procedure is proposed:

- A right shift parameter invQuantRightShift and an offset parameter addOffset derived as follows:

invQuantRightShift = 6 – qPper

- addOffset = invQuantRightShift <= ? 0: 1<< (invQuantRightShift - 1)

- If invQuantRightShift is larger than 0, the following procedure is applied

deQuantEspValue = ( EspValue * invQuantScale[qPrem] + addOffset )>>invQuantRightShift

Otherwise, (invQuantRightShift is 0 or negative), the following procedure is applied

invQuantLeftShift = - invQuantRightShift

deQuantEspValue = (EspValue * invQuantScale[qPrem]) << invQuantLeftShift

# Simulation results

The proposed scheme is implemented on CE3 common software and tested using the common test condition defined in [1]. Table 1 demonstrates the coding performance under full frame Intra BC test conditions.

Table 1. Results of Section 2.1 using a lookup table of 6 elements under the common test condition

|  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V |  |  |  |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |  |  |  |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |  |  |  |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |  |  |  |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |  |  |  |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 97% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  |  |  |  |

Table 2. Results of Section 2.2 using a lookup table of 52 elements under the common test condition

|  |  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | G/Y | B/U |  | R/V |  |  |
| RGB, text & graphics with motion, 1080p & 720p |  | 0.0% | 0.0% |  | 0.0% |  |  |
| RGB, mixed content, 1440p & 1080p |  | 0.0% | 0.0% |  | 0.0% |  |  |
| RGB, Animation, 720p |  | 0.0% | 0.0% |  | 0.0% |  |  |
| RGB, camera captured, 1080p |  | 0.0% | 0.0% |  | 0.0% |  |  |
| YUV, text & graphics with motion, 1080p & 720p |  | 0.0% | 0.0% |  | 0.0% |  |  |
| YUV, mixed content, 1440p & 1080p |  | 0.0% | 0.0% |  | 0.0% |  |  |
| YUV, Animation, 720p |  | 0.0% | 0.0% |  | 0.0% |  |  |
| YUV, camera captured, 1080p |  | 0.0% | 0.0% |  | 0.0% |  |  |
| Enc Time[%] |  | 98% |  |  |  |  |  |
| Dec Time[%] |  | 99% |  |  |  |  |  |

Table 3. Results of Section 2.3 using fixed length codeword under the common test condition

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G/Y |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.1% |  | 0.1% |
| RGB, mixed content, 1440p & 1080p | -0.2% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] | 92% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |

Table 4. Results of Section 2.4 using coefficient coding under the common test condition

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.2% | 0.2% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |

# Conclusion

The proposed three solutions remove the division operations during palette escape pixel coding in the current SCM3.0 and spec. And the proposed methods have unnoticeable RD performance change. It is proposed to be included in the SCM4.0 and spec.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- H. Yu, R. Cohen, K. Rapaka, J. Xu , “Common conditions for screen content coding tests”, JCTVC-S1015, Strasbourg, France, Oct. 2014.

- Text Modifications (based on JCTVC-S1005)

## Using a lookup table of 6 elements

Same as Section 2.1

## Using a lookup table of 52 elements

Same as Section 2.2

## Using a fixed length codeword

9.3.3.13 Binarization process for palette_escape_val

Input to this process is a request for a binarization for the syntax element palette_escape_val, cu_transquant_bypass_flag and colour component index cIdx.

Output of this process is the binarization of palette_escape_val.

The variable bitDepth is derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC

The binarization of palette_escape_val is derived as follows:

–	If cu_transquant_bypass_flag is true, the binarization of palette_escape_val is derived by invoking the FL binarization process specified in subclause 9.3.3.5 with the input parameter set to (1<<bitdepth) − 1.

–	Otherwise (cu_transquant_bypass_flag is false) the following ordered steps apply

- The quantization parameter qP is derived based on color component:

qP = ( cIdx  = =  0 ) ? Qp′Y : ( ( cIdx  = =  1 ) ? Qp′Cb : Qp′Cr )

- A quantization bit reduction factor quantBits is derived using an equation.

- quantBits =  (qP  < 4) ? 0: ( qP – 4 ) / 6

- A bit parameter numBins is derived based on bitdepth and quantBits,:

numBins  =  bitDepth - quantBits

- The maximum parameter cMax for the fixed length binarization is derived as follows

cMax = ( 1  <<  numBins ) − 1

The binarization for the palette_escape_value is derived by invoking the FL binarization process specified in subclause 9.3.3.5 with cMax.

8.4.5.2.8 Decoding process for palette mode

…..

If paletteSampleMode[ xCb + x ][ yCb + y ] is not equal to ESCAPE_MODE, the following applies:

recSample[ cIdx ][ xCb + x ][ yCb + y ] = currentPaletteEntries[ cIdx ][ paletteIndexMap[ xCb + x ][ yCb + y ] ],

–	Otherwise, if cu_transquant_bypass_flag is true, the following applies:

recSample[ cIdx ][ xCb + x ][ yCb + y ] = paletteEscapeVal[ cIdx ][ xCb + x ][ yCb + y ],

–	Otherwise (paletteSampleMode[ xCb + x ][ yCb + y ] is equal to ESCAPE_MODE and cu_transquant_bypass_flag is false), the following ordered steps apply: [Ed. (GJS): Indentation problem.] [Ed. (RJ): fixed]

- The derivation process for quantization parameters as specified in subclause Error: Reference source not found is invoked with the location ( xCb, yCb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture.

- The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = max( 0, Qp′Y )		(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = max( 0, Qp′Cb )		(8-2)

–	Otherwise (cIdx is equal to 2),

qP = max(0, Qp′Cr )		(8-3)

- A quantization bit reduction factor quantBits is derived using an equation.

- quantBits =  (qP  < =4) ? 0: (qP – 4 ) / 6

- The reconstructed sample value recSamples[ xCb + x ][ yCb + y ] is reconstructed

recSamples[ xCb + x ][ yCb + y ] =

- Clip3( 0, (1<<bitDepth)-1,   palette_escape_val[ cIdx ][ xCb + x ][ yCb + y] << quantBits)

- The variables bitDepth and bdShift1 are derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC		(8-4)

bdShift1 = Max( 20 − bitDepth, extended_precision_processing_flag ? 11 : 0 )	(8-5)

- The variables log2TransformRange, bdShift2, coeffMin and coeffMax are derived as follows:

–	If cIdx is equal to 0,

log2TransformRange = extended_precision_processing_flag ? Max( 15, BitDepthY + 6 ) : 15	(8-6)

bdShift2 = BitDepthY + Log2( nTbS ) + 10 − log2TransformRange	(8-7)

coeffMin = CoeffMinY		(8-8)

coeffMax = CoeffMaxY		(8-9)

–	Otherwise,

log2TransformRange = extended_precision_processing_flag ? Max( 15, BitDepthC + 6 ) : 15	(8-10)

bdShift2 = BitDepthC + Log2( nTbS ) + 10 − log2TransformRange	(8-11)

coeffMin = CoeffMinC		(8-12)

coeffMax = CoeffMaxC		(8-13)

[Ed. (GJS): Correct equation numbering.]

- The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

- The following applies

recSamples[ xCb + x ][ yCb + y ] =
Clip3( coeffMin, coeffMax, ( ( palette_escape_val[ cIdx ][ xCb + x ][ yCb + y ]* 16 * 
			levelScale[ qP%6 ]  <<  (qP / 6 ) ) + ( 1  <<  ( bdShift2 − 1 ) ) )  >>  bdShift2 )	(8-14)

[Ed. (GJS): Indentation problem.]

[Ed. (GJS): Reformulate as sentences or add equation numbers.]

[Ed. (RJ): Added equation numbers.]

- The reconstructed sample value recSamples[ xCb + x ][ yCb + y ] is modified as follows:

recSamples[ xCb + x ][ yCb + y ] = ( recSamples[ xCb + x ][ yCb + y ] 
	+ ( 1  <<  ( bdShift2 − 1 ) ) )  >>  bdShift2

## Using a codeword independent of maxValue

      - Binarization process for palette_escape_val

Input to this process is a request for a binarization for the syntax element palette_escape_val, cu_transquant_bypass_flag and colour component index cIdx.

Output of this process is the binarization of palette_escape_val.

The variable bitDepth is derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC

The binarization of palette_escape_val is derived as follows:

- If cu_transquant_bypass_flag is true, the binarization of palette_escape_val is derived by invoking the FL binarization process specified in subclause Error: Reference source not found with the input parameter set to (1<<bitdepth) − 1.

- Otherwise (cu_transquant_bypass_flag is false) the following ordered steps apply

- The quantization parameter qP is derived as follows:

qP = ( cIdx  = =  0 ) ? Qp′Y : ( ( cIdx  = =  1 ) Qp′Cb ? Qp′Cr ) [Ed. (GJS): Equation formatting problem.]

- A quantization step size parameter qStep is derived as follows:

qStep = ( qP= =0 ) ? Round( 2(qP − 4) / 6 ) : 1

- A maximum possible quantized value maxValue is derived as follows:

maxValue = ( ( 1<<bitDepth ) − 1 ) / Qstep

- The number of bins numBins of the fixed length binarization codeword is derived as follows

while( maxValue ) {
			maxValue = maxValue >> 1
			numBins++
		}

- The maximum parameter cMax for the fixed length binarization is derived as follows

cMax = ( 1  <<  numBins ) − 1

The binarization for the palette_escape_value is derived by invoking the coeff_abs_level_remaining binarization process specified in subclause 9.3.3.12 with cRiceParam = 3 and cIdx.

## Fix of undefined dequantization of escape pixel

8.4.5.2.8 Decoding process for palette mode

- The derivation process for quantization parameters as specified in subclause Error: Reference source not found is invoked with the location ( xCb, yCb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture.

- The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = max( 0, Qp′Y )		            							(8-15)

–	Otherwise, if cIdx is equal to 1,

qP = max( 0, Qp′Cb )								(8-16)

–	Otherwise (cIdx is equal to 2),

qP = max(0, Qp′Cr )									(8-17)

- The variables bitDepth and bdShift1 are is derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC

- A quantization ratio qPper and a quantization remainder qPrem are derived as follows

qPper = qP /6

qPrem = qP%6

5.	A right shift parameter invQuantRightShift and an offset parameter addOffset derived as follows:

invQuantRightShift = 6 – qPper

addOffset = invQuantRightShift <= 0 ? 0 : 1<< (invQuantRightShift - 1)

6.	If invQuantRightShift is larger than 0, the following procedure is applied

recSamples[ xCb + x ][ yCb + y ] = (palette_escape_val[ cIdx ][ xCb + x ][ yCb + y ]*	levelScale [qPrem] + addOffset )>>invQuantRightShift

Otherwise, (invQuantRightShift is 0 or negative), the following procedure is applied

invQuantLeftShift = - invQuantRightShift

recSamples[ xCb + x ][ yCb + y ] = (palette_escape_val[ cIdx ][ xCb + x ][ yCb + y ]							* levelScale [qPrem]) << invQuantLeftShift

7.	recSamples[ xCb + x ][ yCb + y ] is further modified to be in the depth range as follows

- recSamples[ xCb + x ][ yCb + y ] = clip3 (0, 2bitDepth-1, recSamples[ xCb + x ][ yCb + y ])