| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0112 |
| --- | --- |

| Title: | CE 1 Related: On escape pixel coding for palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou Vadim Seregin Rajan Joshi Marta Karczewicz Wei Pu 5775 Morehouse Drive San Diego, CA, 92122 | Tel: Email: | +1-858-845-1115 fzou@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The existing escape pixel in SCM3.0 and spec has a division problem in calculating the maximum possible quantized value. This proposal presents three solutions to solve this problem. Solution 1 utilizes a look up table of 6 elements to store six scaling factors (the same as the one used in SCM3.0 encoder) to derive the maximum possible quantized escape pixel value. Solution 2 utilizes a look up table of 52 elements storing the maximum value for each QP. Solution 3 utilizes a codeword for quantized escape pixels which is independent on the maximum possible values. The proposed solutions are all implemented based on CE3 common software, and the simulation results demonstrate that they all lead to 0.0% and 0.0% bitrate increase for text & graphics with motion, 1080p & 720p RGB and YUV respectively under CTC. The existing escape reconstruction has undefined negative right shift operations. It is proposed to be aligned with the current coefficient dequantization to avoid underfined operations. And this has no effect on CTC.

# Introduction

In SCM3.0, truncated binary (TB) codeword is used to signal quantized escape pixels palette_escape_val. To use truncated binary codeword, the total number of possible value is needed. For example, if truncated binary codeword is used to signify a set of N possible values {0, 1, 2, 3, …, N-1}, N is used to construct a truncated binary codeword (where N-1 is the maximum value).

To get maximum quantized escape values, the following procedure is used in SCM3.0:

- The quantization parameter qP is derived as follows:

qP = ( cIdx  = =  0 ) ? Qp′Y : ( ( cIdx  = =  1 ) ? Qp′Cb : Qp′Cr )

- A quantization scaling factor quantScale, rightshift offset rightShiftOffset and rightshift parameter rightShiftParam are derived based on HEVC quantization lookup table as follows

quantScale = g_quantScales[qP%6], where g_quantScales = {26214, 23302, 20560, 18396, 16384, 14564 }

rightShiftParam = 14 +

rightShiftOffset = 1 << (rightShiftParam -1 )

- A quantization step size parameter qStep is derived as follows:

qStep = ( qP= =0 ) ? Round( 2(qP − 4) / 6 ) : 1

- A maximum possible quantized value maxValue is derived as follows:

maxValue = (( 1<<bitDepth ) − 1 ) / Qstep

- The number of bins numBins to represent maxValue is derived as follows

while( maxValue ) {
			maxValue = maxValue >> 1
			numBins++
		}

- The maximum parameter cMax for the truncated binary codeword is derived as follows

cMax = clip3(0,   , ((1<<bitDepth ) − 1 )  quantScale +rightShiftOffset ) >> rightShiftParam)

Note that during cMax derivation, division operations are invoked which are not desirable at the decoder. To solve this problem, three solutions are presented in the following section.

# Proposed escape pixel coding

## Using a lookup table of 6 elements

It is proposed to use encoder parameters to derive maximum quantized escape value at the decoder. Thus encoder related parameters need to be stored in a lookup table at the decoder side.

- The quantization parameter qP is derived based on color component.:

qP = ( cIdx  = =  0 ) ? Qp′Y : ( ( cIdx  = =  1 ) ? Qp′Cb : Qp′Cr )

- A quantization scaling factor quantScale, a right shift offset rightShiftOffset and a right shift parameter rightShiftParam are derived based on a lookup table stored at the decoder for instance:

quantScale = g_quantScales[qP%6], where g_quantScales = {26214, 23302, 20560, 18396, 16384, 14564 }

rightShiftParam = 14 +

rightShiftOffset = 1 << (rightShiftParam -1 )

- The maximum parameter cMax for the truncated binary codeword is derived as follows

cMax = ((1<<bitDepth ) − 1 ) quantScale +rightShiftOffset ) >> rightShiftParam

Note that in the case, the quantized escape pixel is constraint in the range of [0, cMax] inclusive.

## Using a lookup table of 52 elements

It is proposed to derive the maximum quantized escape pixel value using quantization parameter qP and color component bitDepth. Specifically, cMax can be obtained using a loop up table with qP and bitDepth. The following is an example of cMax derivation

- The quantization parameter qP is derived based on color component. Different color component can have different quantization parameters. The following is an example to get Qp′Y, Qp′Cb or Qp′Cr for component Y, Cb and Cr respectively, which may or may not be different:

qP = ( cIdx  = =  0 ) ? Qp′Y : ( ( cIdx  = =  1 ) ? Qp′Cb : Qp′Cr )

- A base quantization parameter baseQp is derived with given bitDepth:

baseQp = qP – 6*(bitDepth - 8)

- If baseQp is equal or larger than 0, cMax can be found through lookup table given baseQp as input entry

- Otherwise, (baseQp is negative), an cMax can be derived as follows

cMax = Table[qP]>>(bitDepth - 8)

Table 1. cMax lookup table

| qP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cMax | 255 | 255 | 255 | 255 | 255 | 227 | 202 | 180 | 161 | 143 | 128 | 114 | 101 | 90 | 80 |

| qP | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cMax | 72 | 64 | 57 | 51 | 45 | 40 | 36 | 32 | 28 | 25 | 23 | 20 | 18 | 16 | 14 |

| qP | 30 | 31 | 32 | 32 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cMax | 13 | 11 | 10 | 9 | 8 | 7 | 6 | 6 | 5 | 4 | 4 | 4 | 3 | 3 | 3 |

| qP | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cMax | 2 | 2 | 2 | 2 | 1 | 1 | 1 |

Note that in the case, the quantized escape pixel is constraint in the range of [0, cMax] inclusive.

## Using a codeword independent of maxValue

It is proposed to use the remaining coefficient coding for quantized escape pixels with Golomb rice parameter being equal to 3.

# Fix of undefined dequantization of escape pixel

The existing escape reconstruction has undefined negative right shift operations. It is proposed to be aligned with the current coefficient dequantization to avoid undefined operations. And this has no effect on CTC.

# Simulation results

The proposed scheme is implemented on CE3 common software and tested using the common test condition defined in [5]. Table 1 demonstrates the coding performance under full frame Intra BC test conditions with 2CTU Intra String Copy when matching length constraint T=4 is applied.

Table 1. Results of Section 2.1 using a lookup table of 6 elements under the common test condition

|  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V |  |  |  |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |  |  |  |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |  |  |  |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |  |  |  |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |  |  |  |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 101% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |  |

Table 2. Results of Section 2.2 using a lookup table of 52 elements under the common test condition

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | #VALUE! | #VALUE! | #VALUE! |
| RGB, mixed content, 1440p & 1080p | #VALUE! | #VALUE! | #VALUE! |
| RGB, Animation, 720p | #VALUE! | #VALUE! | #VALUE! |
| RGB, camera captured, 1080p | #VALUE! | #VALUE! | #VALUE! |
| YUV, text & graphics with motion, 1080p & 720p | #VALUE! | #VALUE! | #VALUE! |
| YUV, mixed content, 1440p & 1080p | #VALUE! | #VALUE! | #VALUE! |
| YUV, Animation, 720p | #VALUE! | #VALUE! | #VALUE! |
| YUV, camera captured, 1080p | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #VALUE! |  |  |
| Dec Time[%] | #VALUE! |  |  |

Table 3. Results of Section 2.3 using a lookup table of 52 elements under the common test condition

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | #VALUE! | #VALUE! | #VALUE! |
| RGB, mixed content, 1440p & 1080p | #VALUE! | #VALUE! | #VALUE! |
| RGB, Animation, 720p | #VALUE! | #VALUE! | #VALUE! |
| RGB, camera captured, 1080p | #VALUE! | #VALUE! | #VALUE! |
| YUV, text & graphics with motion, 1080p & 720p | #VALUE! | #VALUE! | #VALUE! |
| YUV, mixed content, 1440p & 1080p | #VALUE! | #VALUE! | #VALUE! |
| YUV, Animation, 720p | #VALUE! | #VALUE! | #VALUE! |
| YUV, camera captured, 1080p | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #VALUE! |  |  |
| Dec Time[%] | #VALUE! |  |  |

# Conclusion

The proposed three solutions remove the division operations during palette escape pixel coding in the current SCM3.0 and spec. And the proposed methods have unnoticeable RD performance change.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- H. Yu, R. Cohen, K. Rapaka, J. Xu , “Common conditions for screen content coding tests”, JCTVC-S1015, Strasbourg, France, Oct. 2014.

- Text Modifications