| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0113 |
| --- | --- |

| Title: | CE 3 Related: MV and Run coding for Intra String Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou Vadim Seregin Ying Chen Wei Pu Marta Karczewicz 5775 Morehouse Drive San Diego, CA, 92122 | Tel: Email: | +1-858-845-1115 fzou@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal presents results of improved binarization methods for motion vector coding and run coding for CE3 Intra String Copy. Specifically, a combination of “greater than 0”, “greater than 1” and Exponential Golomb codeword with parameter 5 (EG5) is proposed for motion vector coding. And a combination of “greater than 0” and EG is proposed for run coding with up to 4 context coded bins in EG prefix. The implementation is based on the common CE software of the current CE3. The simulation results show that the proposed motion vector coding leads to XX% and XX% bitrate reduction for text & graphics with motion, 1080p & 720p RGB and YUV respectively under CTC. And the proposed run coding leads to XX% and XX% bitrate reduction for text & graphics with motion, 1080p & 720p RGB and YUV respectively under CTC. The combination of these two methods can lead to XX% and XX% bitrate reduction for text & graphics with motion, 1080p & 720p RGB and YUV respectively under Intra String Copy full frame search.

# Introduction

In the 19th JCT-VC meeting in Strasbourg, CE3 [1] was established to study different Intra String Copy methods. In CE3, a common CE software is designed based on JCTVC-R0098 [2] and JCTVC-R0140 [3].To further improve the coding efficiency of Intra String Copy, two methods of binarizations are proposed for motion vector coding and run coding respectively.

# Proposed MV coding for Intra String Copy

In the CE3 common software (developed based on SCM3.0), a motion vector is used to locate the reference matching string in the current picture if a matching string is found. And a combination of “greater than 0” and EG1 are used to code the absolute motion vector value, followed by a sign when the absolute motion vector is nonzero.

However, it is found that a relative large percentage of absolute values being equal to 1. And the distribution of remaining values are quite different from the one for EG1. Thus a novel binarization method is proposed to further improve the MV coding efficiency for Intra String Copy.

Specifically, a combination of “greater than 0”, “greater than 1” and Exponential Golomb codeword with parameter 5 (EG5) is proposed for motion vector coding, followed by sign coding when the absolute value is none zero.

# Proposed Run coding for Intra String Copy

In the CE3 common software (developed based on SCM3.0), a concatenation of several fixed length based run coding is used. It requires certain calculation before parsing the bitstream, and thus reduces the parsing throughput.

To solve this problem and improve the run coding efficiency, a combination of “greater than 0” and EG is used for run coding for Intra String Copy. And up to 5 context coded bins are allowed.

# Simulation results

The proposed scheme is implemented on CE3 common software and tested using the common test condition defined in [5]. Table 1 demonstrates the coding performance under full frame Intra BC test conditions with 2CTU Intra String Copy.

## Simulation results of MV Coding

Table 1. Results of proposed MV coding under the common test condition (Test Condition 1 in CE3)

|  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V |  |  |  |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.4% | -0.3% |  |  |  |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |  |  |  |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.4% | -0.3% |  |  |  |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% |  |  |  |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% |  |  |  |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 133% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |  |

Table 2 demonstrates the coding performance under the 4CTU Intra BC and 4CTU intra string copy test conditions.

Table 2. Results of proposed MV coding under the 4CTU test condition (Test Condition 2 in CE3)

|  |  |  |  |  | Test B.4.1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  |  |  |
|  | G/Y | B/U | R/V |  |  |  |
| RGB, text & graphics with motion, 1080p & 720p | -0.6% | -0.6% | -0.6% |  |  |  |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |  |  |  |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.5% |  |  |  |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |  |  |  |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |  |  |  |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 127% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |

Table 3 Results of proposed MV coding under the full frame Intra BC and full frame intra string copy (with vertical processing enabled) test conditions.

## Simulation result of run coding

Table 4. Results of proposed MV coding under the common test condition (Test Condition 1 in CE3)

Table 5. Results of proposed MV coding under the 4CTU test condition (Test Condition 2 in CE3)

Table 6. Results of proposed MV coding under the full frame Intra BC and full frame intra string copy (with vertical processing enabled) test conditions.

# Conclusion

The proposed binarization methods for mv coding and run coding significantly improve the coding efficiency of Intra String Copy. Therefore, it is proposed to be included in the intra string copy methods for further evaluations.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-C. Chen, Y. Chen, J. Xu, T. Lin and W. Wang, “Description of Core Experiment 3 (CE3): Intra Line Copy and Intra String Copy”, JCT-VC Document, JCTVC-S1103, Strasbourg, France, Oct. 2014.

- B. Li and J. Xu, “SCCE4: Result of Test 3.1”, JCT-VC Document, JCTVC-R0098, Sapporo, Japan, Jun. 2014.

- F. Zou, Y. Chen, J. Sole and M. Karczewicz, “SCCE4: Test 3.2 Pixel based 1D dictionary coding”, JCT-VC Document, JCTVC-R0140, Sapporo, Japan, Jun. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu , “Common conditions for screen content coding tests”, JCTVC-S1015, Strasbourg, France, Oct. 2014.

- Text Modifications

The text provided in this section is based on JCTVC-R1005_v2.

The text supports both 2D matching dictionary mode (supporting adaptive horizontal and vertical scanning) and 1D matching dictionary mode.

- 7.3.2.2.3	Sequence parameter set scc extensions syntax

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| dictionary_mode_enabled_flag | u(1) |
| } |  |

- 7.3.8.5	Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if(!intra_bc_flag[ x0 ][ y0 ] && dictionary_mode_enabled_flag ) |  |
| dictonary_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !dictionary_mode_flag[ x0 ][ y0 ]) |  |
| pred_mode_flag | ae(v) |
| if( palette_mode_enabled_flag && ChromaArrayType = = 3 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_mode_flag[ x0 ][ y0 ] ) |  |
| palette_coding( x0, y0, nCbS ) |  |
| else { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && !intra_bc_flag[ x0 ][ y0 ] ] && !dictionary_mode_flag[ x0 ][ y0 ] &&) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } else if (dictionary_mode_flag[ x0 ][ y0 ] ) { |  |
| dictionary_mode( x0, y0, log2CbSize) |  |
| if( !pcm_flag[ x0 ][ y0 ] && !dictionary_mode_flag[ x0 ][ y0 ]) { |  |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| if( residual_adaptive_colour_transform_enabled_flag && ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ intra_chroma_pred_mode[ x0 ][ y0 ] = = 4 ) ) |  |
| cu_residual_act_flag |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.8.X Dictionary mode syntax

| dictionary_mode( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| nCbS = ( 1 << log2CbSize ) |  |
| totalPixel = nCbS*nCbS |  |
| currPixel = 0 |  |
| currIndex = 0 |  |
| dictionary_type_flag[ x0 ][ y0 ] | ae(v) |
| dictionary_scan_flag[ x0 ][ y0 ] | ae(v) |
| while (currPixel < totalPixel ) { |  |
| dictionay_pred_mode_flag[ x0 ][ y0 ][ currIndex ] | ae(v) |
| if( dictionay_pred_mode_flag[ x0 ][ y0 ][ currIndex ] ) { |  |
| if( dictionary_type_flag[ x0 ][ y0 ] == 0 ) { |  |
| dictionay_pred_offset_minus1[ x0 ][ y0 ][ currIndex ] | ae(v) |
| } else { |  |
| dictionay_pred_offsetX[ x0 ][ y0 ][ currIndex ] | ae(v) |
| dictionay_pred_offsetY[ x0 ][ y0 ][ currIndex ] | ae(v) |
| } |  |
| dictionary_pred_length_minus1[ x0 ][ y0 ][ currIndex ] | ae(v) |
| currPixel += (dictionary_pred_length_minus1 [ x0 ][ y0 ][ currIndex ] + 1) |  |
| } else { |  |
| dictionary_direct_component[ x0 ][ y0 ][ currIndex ][ 0 ] | ae(v) |
| dictionary_direct_component[ x0 ][ y0 ][ currIndex ][ 1 ] | ae(v) |
| dictionary_direct_component[ x0 ][ y0 ][ currIndex ][ 2 ] | ae(v) |
| currPixel += 1 |  |
| } |  |
| currIndex += 1 |  |
| } |  |
| } |  |

- 7.4.3.2.3	Sequence parameter set scc extension

dictionary_mode_enabled_flag equal to 1 specifies that dictionary mode may be invoked in the decoding process for intra prediction. dictionary_mode_enabled_flag equal to 0 specifies that dictionary mode is not applied. When not present, the value of dictionary_mode_enabled_flag is inferred to be equal to 0.

7.4.9.5 Coding unit semantics

dictionary_mode_flag [ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in dictionary mode. dictionary_mode_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in dictionary mode. When palette_mode[ x0 ][ y0 ] is not present, it is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

7.4.9.X Dictionary mode semantics

dictionary_type_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded in dictionary mode and the prediction comes from the dictionary maintained by the decoder. dictionary_type_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in dictionary mode and the prediction comes from the previously reconstructed pixels belonging to the same picture. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionary_scan_flag[ x0 ][ y0 ] equal to 0 specifies that the horizontal scanning is applied to current coding unit. dictionary_scan_flag[ x0 ][ y0 ] equal to 1 specifies that the vertical scanning is applied to current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionay_pred_mode_flag[ x0 ][ y0 ][ i ] equal to 1 specifies the prediction mode is applied to the i-th decoding sub-string in the current CU. dictionay_pred_mode_flag[ x0 ][ y0 ][ i ] equal to 0 specifies the direct mode is applied to the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionay_pred_offset_minus1[ x0 ][ y0 ][ i ] plus 1 specifies the offset of the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionay_pred_offsetX[ x0 ][ y0 ][ i ] specifies the horizontal offset of the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionay_pred_offsetY[ x0 ][ y0 ][ i ] specifies the vertical offset of the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionary_pred_length_minus1[ x0 ][ y0 ][ i ] plus 1 specifies the length of the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionary_direct_component[ x0 ][ y0 ][ i ][ comp ] specifies the pixel value of the comp-th color component in the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

8.X Decoding process for dictionary mode

An array DictHor[] is used to maintain the virtual dictionary for normal dictionary mode in horizontal scanning. An array DictVer[] is used to maintain the virtual dictionary for normal dictionary mode in vertical scanning. Each element in the dictionary is a pixel including three components.

At the beginning of decoding one slice DictHor and DictVer are set to empty.

After decoding one CTU, if the size of DictHor is greater than (1<<18)*3/2, the oldest (1<<18)/2 elements are removed from the dictionary. After decoding one CTU, if the size of DictVer is greater than (1<<18)*3/2, the oldest (1<<18)/2 elements are removed from the dictionary.

If the current CU uses normal dictionary mode with horizontal scanning, 8.X.1 will be invoked.

If the current CU uses normal dictionary mode with vertical scanning, 8.X.2 will be invoked.

If the current CU uses reconstruction based dictionary mode with horizontal scanning, 8.X.3 will be invoked.

If the current CU uses reconstruction based dictionary mode with vertical scanning, 8.X.4 will be invoked.

After decoding one CU, if the current CU uses dictionary mode, the following applies.

If the current CU uses normal dictionary mode with horizontal scanning, the pixels in the current CU will also be added to DictVer using vertical scanning order.

Otherwise, if the current CU uses normal dictionary mode with vertical scanning, the pixels in the current CU will also be added to DictHor using horizontal scanning order.

Otherwise, (the current CU uses reconstruction based dictionary mode), the pixels in the current CU will added to both DictVer using vertical scanning order and DictHor using horizontal scanning order.

8.X.1 Decoding of CU in normal dictionary mode with horizontal scanning

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

–	arrays of dictInfoMatchFlag, dictInfoMatchOffset, dictInfoMatchOffset, dictInfoPixel containing the dictionary information.

Output of this process is a modified reconstructed picture before deblocking filtering.

decodedCount is set to 0.

currIdx is set to 0.

nCbS is set to 1<<log2CbSize.

totalPixel is set to nCbS*nCbS.

currentDictSize is set to the current size of DictHor.

While decodedCount is smaller than totalPixel, the following steps applies.

- If dictInfoMatchFlag[currIdx] is equal to 1,

  - for(i=0;i<dictInfoMatchLength[currIdx];i++)
DictHor[currentDictSize+decodedCount+i] = DictHor[currentDictSize+decodedCount+i-dictInfoMatchOffset[currIdx]]

  - decodedCount += dictInfoMatchLength[currIdx]

- Otherwise (dictInfoMatchFlag[currIdx] is equal to 0)

  - DictHor[currentDictSize+decodedCount]= dictInfoPixel[currIdx]

  - decodedCount += 1

- currIdx++

The pixels at (x, y), where x is in range of  xCb… xCb+nCbS, and y is in range of yCb … yCb+nCbS  will be set to DictHor[currentDictSize+(y-yCb)*nCbS+(x-xCb)]

8.X.2 Decoding of CU in normal dictionary mode with vertical scanning

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

–	arrays of dictInfoMatchFlag, dictInfoMatchOffset, dictInfoMatchOffset, dictInfoPixel containing the dictionary information.

Output of this process is a modified reconstructed picture before deblocking filtering.

decodedCount is set to 0.

currIdx is set to 0.

nCbS is set to 1<<log2CbSize.

totalPixel is set to nCbS*nCbS.

currentDictSize is set to the current size of DictVer.

While decodedCount is smaller than totalPixel, the following steps applies.

- If dictInfoMatchFlag[currIdx] is equal to 1,

  - for(i=0;i<dictInfoMatchLength[currIdx];i++)
DictVer[currentDictSize+decodedCount+i] = DictVer[currentDictSize+decodedCount+i-dictInfoMatchOffset[currIdx]]

  - decodedCount += dictInfoMatchLength[currIdx]

- Otherwise (dictInfoMatchFlag[currIdx] is equal to 0)

  - DictVer[currentDictSize+decodedCount]= dictInfoPixel[currIdx]

  - decodedCount += 1

- currIdx++

The pixels at (x, y), where x is in range of  xCb… xCb+nCbS, and y is in range of yCb … yCb+nCbS  will be set to DictVer[currentDictSize+(x-xCb)*nCbS+(y-yCb)]

8.X.3 Decoding of CU in reconstruction based dictionary mode with horizontal scanning

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

–	arrays of dictInfoMatchFlag, dictInfoMatchOffset, dictInfoMatchOffsetX, dictInfoMatchOffsetY, dictInfoPixel containing the dictionary information.

Output of this process is a modified reconstructed picture before deblocking filtering.

decodedCount is set to 0.

currIdx is set to 0.

nCbS is set to 1<<log2CbSize.

totalPixel is set to nCbS*nCbS.

While decodedCount is smaller than totalPixel, the following steps applies.

- If dictInfoMatchFlag[currIdx] is equal to 1,

  - offsetX = dictInfoMatchOffsetX[currIdx]

  - offsetY = dictInfoMatchOffsetY[currIdx]

  - for(i=0;i<dictInfoMatchLength[currIdx];i++, decodedCount++)
The pixel at (xCb+decodedCount%nCbS,yCb+decocedCount/nCbS) is set to equal to the pixel at (xCb-offsetX+decodedCount%nCbS, yCb-offsetY+decocedCount/nCbS)

- Otherwise (dictInfoMatchFlag[currIdx] is equal to 0)

  - The pixel at (xCb+decodedCount%nCbS,yCb+decocedCount/nCbS) is set to equal to the pixel of dictInfoPixel[currIdx]

  - decodedCount += 1

- currIdx++

8.X.4 Decoding of CU in reconstruction based dictionary mode with vertical scanning

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

–	arrays of dictInfoMatchFlag, dictInfoMatchOffset, dictInfoMatchOffsetX, dictInfoMatchOffsetY, dictInfoPixel containing the dictionary information.

Output of this process is a modified reconstructed picture before deblocking filtering.

decodedCount is set to 0.

currIdx is set to 0.

nCbS is set to 1<<log2CbSize.

totalPixel is set to nCbS*nCbS.

While decodedCount is smaller than totalPixel, the following steps applies.

- If dictInfoMatchFlag[currIdx] is equal to 1,

  - offsetX = dictInfoMatchOffsetX[currIdx]

  - offsetY = dictInfoMatchOffsetY[currIdx]

  - for(i=0;i<dictInfoMatchLength[currIdx];i++, decodedCount++)
The pixel at (xCb+decodedCount/nCbS,yCb+decocedCount%nCbS) is set to equal to the pixel at (xCb-offsetX+decodedCount/nCbS, yCb-offsetY+decocedCount%nCbS)

- Otherwise (dictInfoMatchFlag[currIdx] is equal to 0)

  - The pixel at (xCb+decodedCount/nCbS,yCb+decocedCount%nCbS) is set to equal to the pixel of dictInfoPixel[currIdx]

  - decodedCount += 1

- currIdx++

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| sao( ) | sao_merge_left_flag sao_merge_up_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sao_type_idx_luma sao_type_idx_chroma | Error: Reference source not found | 0 | 1 | 2 |
| coding_quadtree( ) | split_cu_flag[ ][ ] |  | 0..2 | 3..5 | 6..8 |
| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | dictionary_mode_flag | Table 9-X0 | 0 | 1 | 2 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode |  | 0 9..11 | 1..4 | 5..8 |
|  | prev_intra_luma_pred_flag[ ][ ] | Table 9- | 0 | 1 | 2 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | rqt_root_cbf | Error: Reference source not found |  | 0 | 1 |
| dictionary_mode( ) | dictionary_type_flag | Table 9-X1 | 0 | 3 | 6 |
|  | dictionary_scan_flag | Table 9-X1 | 1, 2 | 4, 5 | 7, 8 |
|  | dictionary_pred_mode_flag | Table 9-X2 | 0 | 1 | 2 |
|  | dictionary_pred_offset_minus1 | Table 9-X3 | 0..3 | 4..7 | 8..11 |
|  | dictionary_pred_offsetX dictionary_pred_offsetY | Table 9-X4 | 0 | 1 | 2 |
|  | dictionary_pred_length_minus1 | Table 9-X5 | 0..2 | 3..5 | 6..8 |

Table 9-X0 – Values of initValue for ctxIdx of dictionary_mode_flag

| Initialization variable | ctxIdx of dictionary_mode_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table 9-X1 – Values of initValue for ctxIdx of dictionary_type_flag and dictionary_scan_flag

| Initialization variable | ctxIdx of dictionary_type_flag and dictionary_scan_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

Table 9-X2 – Values of initValue for ctxIdx of dictionary_pred_mode_flag

| Initialization variable | ctxIdx of dictionary_pred_mode_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table 9-X3 – Values of initValue for ctxIdx of dictionary_pred_offset_minus1

| Initialization variable | ctxIdx of dictionary_pred_offset_minus1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

Table 9-X4 – Values of initValue for ctxIdx of dictionary_pred_offsetX and dictionary_pred_offsetY

| Initialization variable | ctxIdx of dictionary_pred_offsetX and dictionary_pred_offsetY |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table 9-X5 – Values of initValue for ctxIdx of dictionary_pred_length_minus1

| Initialization variable | ctxIdx of dictionary_pred_length_minus1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

| Table 9-2 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | intra_bc_flag | FL | cMax = 1 |
|  | dictionary_mode_flag | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | part_mode | Error: Reference source not found | ( xCb, yCb ) = ( x0, y0), log2CbSize |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | prev_intra_luma_pred_flag[ ][ ] | FL | cMax = 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | - |
|  | rqt_root_cbf | FL | cMax = 1 |
| dictionary_mode( ) | dictionary_type_flag | FL | cMax = 1 |
|  | dictionary_scan_flag | FL | cMax = 1 |
|  | dictionary_pred_mode_flag | FL | cMax = 1 |
|  | dictionary_pred_offset_minus1 | 9.3.3.X0 | - |
|  | dictionary_pred_offsetX | 9.3.3.X1 | - |
|  | dictionary_pred_offsetY | 9.3.3.X1 | - |
|  | dictionary_pred_length_minus1 | 9.3.3.X2 | totalPixel, currPixel |
|  | dictionary_direct_component[][][][compIdx] | FL | cMax = compIdx==0 ? (1<<BitDepthY)-1 : (1<<BitDepthC)-1 |

9.3.3.X0 Binarization process for dictionary_pred_offset_minus1

Inputs to this process is a request for a binarization for the syntax element dictionary_pred_offset_minus1.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element dictionary_pred_offset_minus1 is specified in Table 9-XX.

The suffix is binarized by FL with cMax = (1<<number_of_suffix_bins)-1.

Table 9-XX Binarization of dictionary_pred_offset_minus1

| Value | prefix | number_of_suffix_bins | suffix |
| --- | --- | --- | --- |
| [0, 3] | 1 | 2 | value-0 |
| [4, 19] | 01 | 4 | value-4 |
| [20, 275] | 001 | 8 | value-20 |
| [276, 65811] | 0001 | 16 | value-276 |
| Others | 0000 | 19 | value-65811 |

9.3.3.X1 Binarization process for dictionary_pred_offsetX and dictionary_pred_offsetY

Inputs to this process is a request for a binarization for the syntax element dictionary_pred_offsetX or dictionary_pred_offsetY.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element dictionary_pred_offsetX and dictionary_pred_offsetY is specified in Table 9-XX.

The suffix is binarized by EG1.

Table 9-XX Binarization of dictionary_pred_offsetX and dictionary_pred_offsetY

| AbsValue | Greater than 0 flag | Greater than 1 flag | Prefix of EG5 | suffix |
| --- | --- | --- | --- | --- |
| 0 | 0 | N/A | N/A | N/A |
| 1 | 1 | 0 | N/A | N/A |
| 2 | 1 | 1 | 0 | 00000 |
| 3 | 1 | 1 | 0 | 00001 |
| 4 | 1 | 1 | 0 | 00010 |
| … | 1 | 1 | … | … |

9.3.3.X2 Binarization process for dictionary_pred_length_minus1

Inputs to this process is a request for a binarization for the syntax element dictionary_pred_length_minus1, number of total pixels in the current CU totalPixel, and the current pixel position currPixel.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element dictionary_pred_length_minus1 is specified in Table 9-XX.

The suffix is binarized by FL with cMax = (1<<number_of_suffix_bins)-1.

Table 9-XX Binarization of dictionary_pred_offset_minus1

| value | totalPixel-currPixel-1 | prefix | number_of_suffix_bins | suffix |
| --- | --- | --- | --- | --- |
| [0, 3] | <4 | na | 2 | value-0 |
|  | others | 1 |  |  |
| [4, 19] | <20 | 0 | 4 | value-4 |
|  | others | 01 |  |  |
| [20, 275] | <276 | 00 | 8 | value-20 |
|  | others | 001 |  |  |
| Others | All | 000 | 12 | value-276 |

| Table 9-3 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_transquant_bypass_flag | 0 | na | na | na | na | na |
| cu_skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| intra_bc_flag | 0 | na | na | na | na | na |
| dictionary_mode_flag | 0 | na | na | na | na | na |
| pred_mode_flag | 0 | na | na | na | na | na |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | na | na |
| part_mode log2CbSize > MinCbLog2SizeY | 0 | 1 | 3 | bypass | na | na |
| pcm_flag[ ][ ] | terminate | na | na | na | na | na |
| …… |  |  |  |  |  |  |
| dictionary_type_flag | 0 | na | na | na | na | na |
| dictionary_scan_flag | dictionary_type_flag | na | na | na | na | na |
| dictionary_pred_mode_flag | 0 | na | na | na | na | na |
| dictionary_pred_offset_minus1 (prefix) | 0 | 1 | 2 | 3 | na | na |
| dictionary_pred_offset_minus1 (suffix) | bypass | bypass | bypass | bypass | bypass | bypass |
| dictionary_pred_offsetX | 0 | bypass | bypass | bypass | bypass | bypass |
| dictionary_pred_offsetY | 0 | bypass | bypass | bypass | bypass | bypass |
| dictionary_pred_length_minus1 (prefix) | 0 | 1 | 2 | na | na | na |
| dictionary_pred_length_minus1 (suffix) | bypass | bypass | bypass | bypass | bypass | bypass |
| dictionary_direct_component | bypass | bypass | bypass | bypass | bypass | bypass |
| …… |  |  |  |  |  |  |