| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0114 |
| --- | --- |

| Title: | Non-CE2: Adding a context for significance flag in BVD |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Andrew Segall 5750 NW Pacific Rim Blvd, Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution proposes an additional context to improve significance flag coding for BVD. Specifically significance flag for vertical component is coded with two contexts depends on whether significance flag for horizontal component is significant or not. Average BD rate difference of YUV Text & graphics with motion 1080P for the proposed approach is:  0.4%, 0.2%, 0.1% for AI, RA, LD in lossy case and 0.1%, 0.1%, 0.0% for AI, RA, LD in lossless case.

# Introduction

In the current HEVC screen content coding draft [1], block vector difference (BVD) is coded with the following syntax elements.

bvd_coding ( x0, y0, refList ) {

abs_bvd_greater0_flag[ 0 ]

abs_bvd_greater0_flag[ 1 ]

if( abs_bvd_greater0_flag[ 0 ] ) {

abs_bvd_minus1[ 0 ]

bvd_sign_flag[ 0 ]

}

if( abs_bvd_greater0_flag[ 1 ] ) {

abs_bvd_minus1[ 1 ]

bvd_sign_flag[ 1 ]

}

}

Here, abs_bvd_greater0_flag [0] is entropy coded using a single context model and abs_bvd_greater0_flag [1] is entropy coded using a single context model that is independent of the context model for abs_bvd_greater0_flag [0].  Coding each of the horizationtal and vertical abs_bvd_greater0_flag syntax elements in such a manner is less than ideal.  In this contribution more efficient coding method for abs_bvd_greater0_flag syntax elements is introduced by adding one context for significance flag in BVD.

Specifically, a first context model may be selected for abs_bvd_greater0_flag[1] when abs_bvd_greater0_flag[0] indicates that the absolute value of the BVD for the horiztional component is greater than zero (e.g., abs_bvd_greater0_flag[0] is equal to one) and the second context model may be selected for abs_bvd_greater0_flag[1] when abs_bvd_greater0_flag[0] indicates that the absolute value of the BVD for the horiztional component is not greater than zero (e.g., abs_bvd_greater0_flag[0] is equal to zero).  The corresponding technique is shown in Figure 1.

Figure 1 Proposed context assignment for significance flag in BVD

# Experimental Results

The compression performance is measured using BD rate compared with SCM-3.0 anchor.

Lossy:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.2% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -0.3% | -0.3% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.2% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | -0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.1% | -0.2% |
| RGB, Animation, 720p | -0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.2% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |

Lossless:

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.2% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.3% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.2% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.2% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | -0.2% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.2% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |

# Conclusion

This contribution proposes an additional context for significance flag in BVD to further improve coding performance. Average BD rate difference of YUV Text & graphics with motion 1080P for the proposed approach is:  0.4%, 0.2%, 0.1% for AI, RA, LD in lossy case and 0.1%, 0.1%, 0.0% for AI, RA, LD in lossless case.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Rajan Joshi, Jizheng Xu, “High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 2,” 19th meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Strasbourg, FR, 17 Oct. – 24 Oct. 2014