| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0121_r2 |
| --- | --- |

| Title: | On lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yan Ye, Yuwen He 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# In this update, the experimental results on 4:2:0 sequences are added.

In the second update, one typo and bugs in slice level lossless signaling are fixed.

# Abstract

In this contribution, two design modifications for lossless coding are proposed. In the first design modification, it is proposed to add a high level signaling to indicate lossless coding at the picture level.  Two options are proposed for adding the high level signaling: a flag in either PPS extension or the slice segment header. In the second design modification, it is proposed to add split_transform_flag signaling constraint when cu_transquant_bypass_flag is enabled. Experimental results show that compared to the SCM-3.0 anchor the first modification alone provides 10% decoding complexity reduction with no impact on the coding performance; and the combination of the two modifications reduces the encoding and decoding complexity by 15% and 10% on average with the average bit-rate increase of 0%, 0.1% and 0.1% for AI, RA and LB, respectively.

# Introduction

The current design of lossless coding in HEVC screen content coding specification draft 2 [1] inherits the HEVC version 1 design, which uses the flag cu_transquant_bypass_flag to enable lossless coding at the CU level. When cu_transquant_bypass_flag is equal to 1, inverse transform and inverse quantization, as well as in-loop filters including deblocking and SAO of the CU are all bypassed. The presence of this CU level flag is conditioned upon a PPS flag, transquant_bypass_enabled_flag. When transquant_bypass_enabled_flag is equal to 0, the CU level flag cu_transquant_bypass_flag is not present, and is inferred to be 0. As such, lossy coding can be signaled at the picture level, but lossless coding can only be signaled at the block level.

Compared to other video applications, the lossless coding mode is an especially important operation mode for screen content coding [2]. In a typical lossless mode of screen content coding, lossless coding is applied to the entire video picture or the entire video sequence. Using the current design, all CUs in the coded video picture or video sequence need to signal the cu_transquant_bypass_flag with value equal to 1. Although CABAC coding ensures that the signaling overhead is minimal, the decoder is still required to parse this flag for all CUs. Therefore, in a first design modification of this contribution, it is proposed to add high level signaling of lossless coding. Having high level lossless signaling not only allows the lossless video applications to avoid signaling the CU level flag, but also allows the lossless decoder to bypass all unused decoding modules (inverse transform, inverse quantization, deblocking, and SAO) from higher level. In this contribution, two options for high level signaling of lossless coding are proposed: option 1 adds the lossless flag in PPS screen content coding (SCC) extension, and option 2 adds the lossless flag in slice segment header. Additionally, bitstream conformance constraints for either option are proposed to avoid signaling of unnecessary syntax elements that are not used in lossless coding.

When a CU is coded in lossless mode (cu_transquant_bypass_flag = 1), inverse transform is not performed. Thus, whether to perform TU quadtree splitting may not significantly impact coding performance. In this contribution, a second design modification is proposed to add split_transform_flag signaling conditions by disallowing TU quadtree splitting for some CUs. This modification leads to significant encoder speedup with minimal performance loss.

# Proposed high level lossless signaling

## Option 1: lossless coding signaling in PPS SCC extension

In this option, it is proposed to add a flag in PPS SCC extension to signal that all CUs in all slices attached to this PPS are coded in lossless mode.

Table 1 Modified PPS SCC extension for option 1

| pps_scc_extensions( ) { | Descriptor |
| --- | --- |
| if( !transquant_bypass_enabled_flag ) |  |
| transquant_bypass_default_flag |  |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| } |  |

transquant_bypass_default_flag specifies the inferred value of cu_transquant_bypass_flag when transquant_bypass_enabled_flag is equal to 0. When transquant_bypass_default_flag is not present, it is inferred to be equal to 0.

There are a number of existing syntax elements in PPS that are related to the transform, quantization, and in-loop filtering processes. It is proposed to add the following bitstream conformance constraints when lossless coding is set to 1 (transquant_bypass_default_flag) in PPS extension:

- In the PPS, the following syntax elements shall be equal to 0 when transquant_bypass_default_flag is equal to 1: transform_skip_enabled_flag, cu_qp_delta _enabled_flag, pps_cb_qp_offset, pps_cr_qp_offset, loop_filter_across_tiles_enabled_flag, pps_loop_filter_across_slices_enabled_flag, deblocking_filter_override_enabled_flag, pps_ scaling_list_data_present_flag, chroma_qp_offset_list_enabled_flag, log2_sao_offset_scale_ luma, log2_sao_offset_scale_chroma;

- In the PPS, the following syntax elements shall be equal to 1 when transquant_bypass_default_flag is equal to 1: deblocking_filter_control_present_flag, pps_deblocking_filter_disabled_flag, pps_extension_present_flag, pps_scc_extensions_flag.

Table 2 Modified slice segment header for option 1

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( sample_adaptive_offset_enabled_flag && ! transquant_bypass_enabled_flag) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| …… |  |
| if(! transquant_bypass_enabled_flag) |  |
| slice_qp_delta | se(v) |
| …… |  |
| } |  |

slice_qp_delta specifies the initial value of QpY to be used for the coding blocks in the slice until modified by the value of CuQpDeltaVal in the coding unit layer. When slice_qp_delta is not present, it is inferred to be equal to 0. The initial value of the QpY quantization parameter for the slice, SliceQpY, is derived as follows:

SliceQpY = 26 + init_qp_minus26 + slice_qp_delta	(7-1)

The value of SliceQpY shall be in the range of −QpBdOffsetY to +51, inclusive.

The existing CU level cu_transquant_bypass_flag is modified as follow:

cu_transquant_bypass_flag equal to 1 specifies that the scaling and transform process as specified in subclause Error: Reference source not found and the in-loop filter process as specified in subclause Error: Reference source not found are bypassed. When cu_transquant_bypass_flag is not present, it is inferred to be equal to 0transquant_bypass_default_flag.

## Option 2: lossless coding signaling in slice segment header

In this option, it is proposed to add a flag in slice segment header to signal that all CUs in the slice are coded in lossless mode.

Table 3 Modified slice segment header for option 2

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if(!transquant_bypass_enabled_flag) |  |
| transquant_bypass_default_flag |  |
| …… |  |
| if( sample_adaptive_offset_enabled_flag && !transquant_bypass_default_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| …… |  |
| if(!transquant_bypass_default_flag) |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag && !transquant_bypass_default_flag) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( chroma_qp_offset_list_enabled_flag && !transquant_bypass_default_flag) |  |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag && !transquant_bypass_default_flag) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| …… |  |
| } |  |

transquant_bypass_default_flag specifies the inferred value of cu_transquant_bypass_flag for the coding units in the current slice when cu_transquant_bypass_flag is not present. When transquant_bypass_default_flag is not present, it is inferred to be equal to 0.

Modified coding_unit() for option 2

cu_transquant_bypass_flag equal to 1 specifies that the scaling and transform process as specified in subclause Error: Reference source not found and the in-loop filter process as specified in subclause Error: Reference source not found are bypassed. When cu_transquant_bypass_flag is not present, it is inferred to be equal to 0transquant_bypass_default_flag.

# Proposed simplified transform quadtree splitting in lossless coding

The transform_tree() syntax structure signals the quadtree splitting flag split_transform_flag of TUs, the cbf_luma, cbf_cb and cbf_cr flags, followed by the coefficients. For losslessly coded CUs, because transform is not applied, whether to perform TU quadtree splitting does not significantly impact the coding performance, especially for CUs with large block size and/or intra coding mode. On the other hand, encoding speed can be significantly improved because the RDO process related to TU splitting can be skipped. Therefore, when cu_transquant_bypass_flag is equal to 1, the following is proposed to conditionally avoid the signaling of split_transform_flag.

- For all the intra-coded CUs, and all the inter-coded and IBC-coded CUs with block size from 64x64 to 32x32, the split_transform_flag are not signaled; instead, it will be inferred to the corresponding default value  which is defined in the same way of HEVC SCC specification draft 2 [1].

- For 8x8 and 16x16 CUs coded with inter mode or IBC mode, transform quadtree splitting is allowed.

Table 5 Modified transform_tree() with the constrained split_transform_flag signaling

| transform_tree( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if((!cu_transquant_bypass_flag¦¦ (CuPredMode[ x0 ][ y0 ] != MODE_INTRA && log2TrafoSize <= 4) ) && log2TrafoSize <= MaxTbLog2SizeY && log2TrafoSize > MinTbLog2SizeY && trafoDepth < MaxTrafoDepth && !( IntraSplitFlag && ( trafoDepth = = 0 ) ) ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( ( log2TrafoSize > 2 && ChromaArrayType != 0 ) ¦ ¦ ChromaArrayType = = 3 ) { |  |
| if( trafoDepth = = 0 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( ChromaArrayType = = 2 && ( !split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ log2TrafoSize = = 3 ) ) |  |
| cbf_cb[ x0 ][ y0 + ( 1 << ( log2TrafoSize − 1 ) ) ][ trafoDepth ] | ae(v) |
| } |  |
| if( trafoDepth = = 0 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( ChromaArrayType = = 2 && ( !split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ log2TrafoSize = = 3 ) ) |  |
| cbf_cr[ x0 ][ y0 + ( 1 << ( log2TrafoSize − 1 ) ) ][ trafoDepth ] | ae(v) |
| } |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( 1 << ( log2TrafoSize − 1 ) ) |  |
| y1 = y0 + ( 1 << ( log2TrafoSize − 1 ) ) |  |
| transform_tree( x0, y0, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y0, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x0, y1, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x1, y1, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] != 1 ) ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ ( ChromaArrayType = = 2 && ( cbf_cb[ x0 ][ y0 + ( 1 << ( log2TrafoSize − 1 ) ) ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 + ( 1 << ( log2TrafoSize − 1 ) ) ][ trafoDepth ] ) ) ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| transform_unit( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx ) |  |
| } |  |
| } |  |

On the other hand, the above conditional signaling method of split_transform_flag could be also implemented as one encoder-only method. In this case, signaling of split_transform_flag is unchanged; the encoder will only test the transform quadtree splitting cases according to the signaling conditions of split_transform_flag in Table 5.

# Simulation results

The proposed methods are implemented based on SCM-3.0, and tested following the current common test condition [3]. Three sets of simulation results are evaluated below using the following settings:

- Setting one: applying only the first proposed modification, i.e., high level lossless signaling. The simulation results are collected based on option 1, adding the flag in PPS SCC extension. But it is expected that adding the flag in slice segment header will produce similar RD performance with the difference of a few header bits.

- Setting two: applying both proposed modifications, i.e., high level signaling in PPS SCC extension and the proposed split_transform_flag signaling constraint in Table 5.

- Setting three: applying both proposed modifications, but with the second modification implemented as an encoder only change.

The encoding and decoding time reported in this section are measured using a cluster system with homogeneous cores and thus reliable.

## Coding performance and complexity evaluation of Setting One

As can be seen from Table 6, the proposed high level signaling has minimal impact on the coding performance and the encoding time. However, Table 6 shows that the decoding time can be reduced by about 10% compared to the anchor.

Table 6 Performance comparison between the anchor and Setting one

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 89% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.3% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.3% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 91% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.6% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.2% | -0.5% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 91% |  |  |  |

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 88% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 97% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 96% |  |  |  |

## Coding performance and complexity evaluation of Setting Two

As shown in Table 7, together the two proposed modifications (high level lossless signal and  split_transform_flag signaling constraint) significantly reduce the encoding and decoding time by 15% and 10% on average for all 3 settings. Impact on coding performance is negligible with the average bit-rates increases of 0%, 0.1% and 0.1% for AI, RA and LB, respectively. Comparing these results with those from Setting One, it can be observed that the significant encoding time reduction is mainly due to the proposed split_transform_flag signaling constraint. Thus, it is asserted that the proposed split_transform_flag signaling constraint provides a good trade-off between coding efficiency and complexity.

Table 7 Performance comparison between the anchor and Setting two

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | -0.1% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 80% |  |  |  |
| Dec Time[%] | 85% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | -0.2% | 0.4% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.0% | -0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 85% |  |  |  |
| Dec Time[%] | 94% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.0% | -0.4% | 0.5% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.3% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 89% |  |  |  |
| Dec Time[%] | 93% |  |  |  |

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.2% | 0.2% | 0.0% | 0.3% |
| Mixed content, 480p | 0.2% | 0.2% | 0.2% | 0.2% |
| Animation, 768p | 0.2% | 0.2% | 0.2% | 0.2% |
| Average of all sequences | 0.2% | 0.2% | 0.0% | 0.3% |
| Enc Time[%] | 94% |  |  |  |
| Dec Time[%] | 83% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.1% | 0.1% | 0.1% | 0.2% |
| Mixed content, 480p | 0.1% | 0.1% | 0.1% | 0.1% |
| Animation, 768p | 0.1% | 0.1% | 0.1% | 0.1% |
| Average of all sequences | 0.1% | 0.1% | 0.1% | 0.2% |
| Enc Time[%] | 94% |  |  |  |
| Dec Time[%] | 84% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.0% | 0.0% | 0.0% | 0.1% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.1% | 0.1% | 0.1% | 0.1% |
| Average of all sequences | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 97% |  |  |  |
| Dec Time[%] | 88% |  |  |  |

## Coding performance and complexity evaluation of Setting Three

As shown in Table 8, when the second modification is implemented as an encoder only method, the proposed constraint on whether to test split_transform_flag can provide encoding time saving of 12%. And, the corresponding average bit-rate increases are 0.1%, 0.1% and 0.1% for AI, RA and LB respectively.

Table 8 Performance comparison between the anchor and Setting three

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.2% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 85% |  |  |  |
| Dec Time[%] | 85% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | -0.1% | 0.5% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | -0.1% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 89% |  |  |  |
| Dec Time[%] | 94% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | -0.3% | 0.5% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.0% | -0.2% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 91% |  |  |  |
| Dec Time[%] | 95% |  |  |  |

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.2% | 0.2% | 0.1% | 0.4% |
| Mixed content, 480p | 0.2% | 0.2% | 0.2% | 0.2% |
| Animation, 768p | 0.2% | 0.2% | 0.2% | 0.2% |
| Average of all sequences | 0.2% | 0.2% | 0.1% | 0.4% |
| Enc Time[%] | 95% |  |  |  |
| Dec Time[%] | 85% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.2% | 0.1% | 0.0% | 0.2% |
| Mixed content, 480p | 0.1% | 0.1% | 0.1% | 0.1% |
| Animation, 768p | 0.1% | 0.1% | 0.1% | 0.1% |
| Average of all sequences | 0.1% | 0.1% | 0.0% | 0.2% |
| Enc Time[%] | 95% |  |  |  |
| Dec Time[%] | 88% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| Text & graphics with motion, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| Mixed content, 480p | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | 0.1% | 0.1% | 0.1% | 0.1% |
| Average of all sequences | 0.1% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 94% |  |  |  |
| Dec Time[%] | 84% |  |  |  |

# References

- R. Joshi, J. Xu, HEVC Screen Content Coding Draft Text 2, JCTVC-S1005, Oct. 2014, Strasbourg, France.

- Requirements for an extension of HEVC for coding of screen content, ISO/IEC JTC1/SC29/WG11 N14174, Jan. 2014, San Jose, USA

- H. Yu, R. Cohen, K. Rapaka, J. Xu, Common Test Conditions for Screen Content Coding, JCTVC-S1015, Oct. 2014, Strasbourg, France.

# Patent rights declaration(s)

InterDigital Communications LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).