| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0122_r1 |
| --- | --- |

| Title: | Implicit transform quadtree partition for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

In this update, the lossless results are added.

# Abstract

In HEVC screen content coding specification draft 2, when the syntax element max_transform_hierarchy_depth_inter in SPS is equal to 0, one implicit transform quadtree partition method is applied to inter CUs, such that the transform quadtrees of all inter CUs with non-square PU partitions are forced to split once (i.e., split_transform_flag is always inferred to 1 in this case). In this contribution, it is proposed to extend this implicit transform quadtree splitting rule to intra block copy mode. The proposed method is tested against the SCM-3.0 anchor when explicit signaling of transform quadtree partition is disabled (i.e., max_transform_hierarchy_depth_inter = 0) in SPS. Experimental results show that the proposed method provides the average {G/Y, B/Cb, R/Cr} BD-rate savings of {2.8%, 2.0%, 1.9%}, {2.0%, 1.4%, 1.3%} and {1.1%, 0.4%, 0.3%} for AI, RA and LB configurations respectively.

# Introduction

According to the current specification draft of HEVC screen content coding (SCC) extension [1], when transform quadtree splitting is disabled for inter mode by setting the SPS syntax element max_transform_hierarchy_depth_inter to 0, implicit transform quadtree is applied to force all inter-coded CUs with non-square partitions to automatically split once for transform coding. Currently, this implicit transform quadtree splitting is only applied to inter-coded CUs.

# Proposed method

In this contribution, it is proposed to also apply the implicit transform quadtree splitting to CUs coded in intra block copy mode. Specifically, based on the proposed method, the derivation process of split_transform_flag is specified as follows:

split_transform_flag[ x0 ][ y0 ][ trafoDepth ] specifies whether a block is split into four blocks with half horizontal and half vertical size for the purpose of transform coding. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

The variable interSplitFlag is derived as follows:

- If max_transform_hierarchy_depth_inter is equal to 0 and CuPredMode[ x0 ][ y0 ] is equal to MODE_INTER or intra_bc_flag[ x0 ][ y0 ] is equal to 1 and PartMode is not equal to PART_2Nx2N and trafoDepth is equal to 0, interSplitFlag is set equal to 1.

- Otherwise, interSplitFlag is set equal to 0.

When split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred as follows:

- If one or more of the following conditions are true, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 1:

- log2TrafoSize is greater than MaxTbLog2SizeY.

- IntraSplitFlag is equal to 1 and trafoDepth is equal to 0.

- interSplitFlag is equal to 1.

Otherwise, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

# Simulation results

The proposed method is implemented based on SCM-3.0 and tested under the common test condition (CTC) as specified in [2], except the following two parameters that are set to be different from their CTC values: QuadtreeTUMaxDepthIntra = 1, QuadtreeTUMaxDepthInter = 1.

The simulation results are collected from a homogeneous cluster system, thus timing is reliable.

As it can be seen in Table 1, for lossy coding, the proposed method reportedly provides the average {G/Y, B/Cb, R/Cr} BD-rate reductions of {2.8%, 2.0%, 1.9%}, {2.0%, 1.4%, 1.3%} and {1.1%, 0.4%, 0.3%} for AI, RA and LB all the categories of test sequences, respectively, without introducing noticeable encoding and decoding difference. As shown Table 2, for lossless coding, the corresponding bit-rate savings of the proposed method are 0.2%, 0.1% and 0.1% for AI, RA and LB respectively.

Table 1 BD-rate performance of the proposed method, compared to SCM-3.0 anchor

|  |  | All Intra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | G/Y |  | B/U |  | R/V |  |  |
| RGB, text & graphics with motion, 1080p & 720p |  | -5.4% |  | -4.1% |  | -4.2% |  |  |
| RGB, mixed content, 1440p & 1080p |  | -6.7% |  | -4.9% |  | -4.9% |  |  |
| RGB, Animation, 720p |  | -0.1% |  | -0.1% |  | -0.1% |  |  |
| RGB, camera captured, 1080p |  | -0.1% |  | 0.0% |  | 0.0% |  |  |
| YUV, text & graphics with motion, 1080p & 720p |  | -4.3% |  | -3.0% |  | -2.6% |  |  |
| YUV, mixed content, 1440p & 1080p |  | -5.4% |  | -3.2% |  | -3.0% |  |  |
| YUV, Animation, 720p |  | -0.1% |  | -0.3% |  | 0.0% |  |  |
| YUV, camera captured, 1080p |  | -0.1% |  | -0.1% |  | -0.1% |  |  |
| Enc Time[%] |  | 101% |  |  |  |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  |  |  |
|  |  | Random Access |  |  |  |  |  |  |
|  |  | G/Y |  | B/U |  | R/V |  |  |
| RGB, text & graphics with motion, 1080p & 720p |  | -3.3% |  | -2.4% |  | -2.5% |  |  |
| RGB, mixed content, 1440p & 1080p |  | -5.8% |  | -4.0% |  | -3.9% |  |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, camera captured, 1080p |  | 0.0% |  | -0.1% |  | 0.0% |  |  |
| YUV, text & graphics with motion, 1080p & 720p |  | -2.5% |  | -1.7% |  | -1.5% |  |  |
| YUV, mixed content, 1440p & 1080p |  | -4.3% |  | -2.7% |  | -2.6% |  |  |
| YUV, Animation, 720p |  | 0.0% |  | -0.1% |  | -0.1% |  |  |
| YUV, camera captured, 1080p |  | -0.1% |  | 0.0% |  | -0.1% |  |  |
| Enc Time[%] |  | 100% |  |  |  |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  |  |  |
|  |  | Low delay B |  |  |  |  |  |  |
|  |  | G/Y |  | B/U |  | R/V |  |  |
| RGB, text & graphics with motion, 1080p & 720p |  | -1.7% |  | -1.1% |  | -1.1% |  |  |
| RGB, mixed content, 1440p & 1080p |  | -3.4% |  | -1.6% |  | -1.6% |  |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, text & graphics with motion, 1080p & 720p |  | -1.3% |  | -0.5% |  | -0.1% |  |  |
| YUV, mixed content, 1440p & 1080p |  | -2.1% |  | -0.7% |  | 0.4% |  |  |
| YUV, Animation, 720p |  | 0.1% |  | 0.4% |  | 0.0% |  |  |
| YUV, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.1% |  |  |
| Enc Time[%] |  | 100% |  |  |  |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  |  |  |

Table 2 Bit-rate performance of the proposed method, compared to SCM-3.0 anchor

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.4% | -0.4% | -1.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -1.4% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.4% | -0.5% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.3% | -1.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.2% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.4% | -1.1% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.3% | -1.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.2% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.3% | -1.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 104% |  |  |  |

# References

- R. Joshi, J. Xu, HEVC Screen Content Coding Draft Text 2, JCTVC-S1005, Oct. 2014, Strasbourg, France.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, Common Test Conditions for Screen Content Coding, JCTVC-S1015, Oct. 2014, Strasbourg, France.

# Patent rights declaration(s)

InterDigital Communications LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).