| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0123 |
| --- | --- |

| Title: | Non-CE1: Mapping of reconstructed pixels to palette indices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Wei Pu, Marta Karczewicz, Rajan Joshi, Ted Hsieh 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is related to the tests performed in CE1 A category, where outside of coding unit row or column is used in the palette copy above mode, and presents methods of mapping the reconstructed outside pixels into the palette entries. In the first method, the sum of absolute differences calculation is replaced with bitwise “exclusive or” operation. Reportedly, it provides 1.4%, 0.3%, 0.0%, 0.0%, 1.6%, 0.5%, 0.0%, 0.0% luma BD-rate reduction among sequence classes used in the common test conditions for the lossy configuration. In the second method, the look up table is derived and used for mapping. Reportedly, it provides 1.2%, 0.2%, 0.0%, 0.0%, 1.4%, 0.4%, 0.0%, 0.0% luma BD-rate reduction among sequence classes used in the common test conditions for the lossy configuration.

# Introduction

In CE1 category A tests, pixels located outside of the current coding unit are used in the palette copy above mode. In the CE tests to map reconstructed pixel, sum of absolute differences (SAD) between the pixel to be mapped and palette entry is calculated, and the index of the palette entry having smallest SAD is assigned to the reconstructed pixel. In the worst case, all colour components and all palette entries are checked per reconstructed pixel.

Since this operation is performed at decoder side as well, it is desired to have it as simple as possible. In the contribution two methods are proposed.

## Method 1. Replacing SAD with “exclusive or”

It is proposed to replace SAD calculation with “exclusive or” (XOR) in the mapping process, where the error is calculated as follows.

dist(P0,P1)=(Y0Y1)+(U0U1)+(V0V1)

This method was implemented on top of CE1 A1.1 test and the simulation results are summarised in the next table.

Table 1. CE1 test A1.1 (left) and A1.1 with method 1 (right)

## Method 2. Mapping with look up table

To avoid a loop over palette entries, it is proposed to generate a look up table (LUT) first based on the palette entries, and as a second step for each reconstructed pixel look up for the index.

LUT is derived by selecting N most significant bits (MSB) for every colour sample of the palette table. This N bits are used as an index in the look table per colour component.

LUT[y][u][v] is initialised with zero. Let’s consider a palette entryP=Y,U,V with index idx. The LUT indices are calculated as follows:

idxY=Y>>(bitdepth−N),idxU=U>>(bitdepth−N),idxV=V>>(bitdepth−N),

where bitdepth is the sample bitdepth. The palette index is set to the look up table

LUT[idxY][idxU][idxV]=idx,

and this process is repeated for the palette entries involved in the mapping process.

In the second step, for the reconstructed pixels, LUT index is calculated as it is done for palette entries and palette index is obtained by looking in the table.

LUT derivation can be done at parsing stage and palette index look up can be performed during reconstruction.

This method was implemented on top of CE1 A1.1 test and the simulation results are summarised in the next table, where N is set equal to 3.

Table 2. CE1 test A1.1 (left) and A1.1 with method 2 (right)

Tests were performed on heterogeneous Linux cluster, so running time can be inaccurate. The full set of the results can be found in the accompanying Excel tables.

# Conclusion

Two methods, simplifying the mapping process of the reconstructed pixels located outside of the coding unit used in the palette copy above mode, are presented. Simulation results reportedly show 0.0-0.1% luma BD rate loss comparing to CE1 A1.1 test for method 1, and 0.0-0.3% luma BD rate loss for the method 2. It is proposed to consider proposed simplifications for CE1 tests in category A.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).