| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0126-r1 |
| --- | --- |

| Title: | CE3 Test B.3: 2-D Intra String Copy in HEVC SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Wang, Meng Xu, Zhan Ma, H. Yu 2330 Central Expy Santa Clara, CA 95050 USA | Tel: Email: | +1 408-330-5142 wei.wang.sc@huawei.com |
| Source: | Huawei R&D USA |  |  |

_____________________________

# Abstract

This contribution presents a hybrid 1-D and 2-D string copy method designed for improving the overall coding performance of HEVC SCC WD and SCM 3.0. The performance reported in this contribution was evaluated under common test conditions with various search range configurations.

# Proposed Method

Basically, for any given location, in addition to the RUN based 1D string match, a 2D string copy with flexible width and height is also performed. If an area within the current CU covered by the 2D-match has more pixels than the total pixels represented by a single 1D RUN length, the 2D string copy mode is selected and 4 parameters, i.e. mvx, mvy, width, height, are encoded into bit-stream. Otherwise, a 1D string copy mode, similar to the current color index map coding in SCM-3.0, is selected and its associated syntax elements are encoded into bit-stream.

Technical Details:

In this proposal, prior to the string match search, the original pixels in the current CU as well as the reconstructed pixels in the search area are pre-quantized. To improve the compression quality, we use the pixels referred by the match information as prediction to the current CU and then generate prediction residuals accordingly. These prediction residuals are encoded by the HEVC residual coding

A one-bit context coded “2D_flag” is added to the syntax to indicate if the 2D-string copy mode is selected.

- If 2D_flag = 1:

  - a one-bit context coded mvx0_flag is added to indicate whether its motion vector mvx is equal to zero; and a one-bit context coded mvy0_flag is added to indicate whether its motion vector mvy is equal to 0.

    - When a 2D block is a single color block, both mvx and mvy are set to 0 and its pixel values are encoded using the frequent pixel table method described below.

  - a one-bit context is used to encode the sign of non-zero mvx, followed by the absolute value of mvx, which is encoded using truncated binary coding (TBC) method. The max value of TBC input parameter can be obtained from the width of search range.

  - a one-bit context is used to encode the sign of non-zero mvy, followed by the absolute value of mvy, which is encoded using truncated binary coding (TBC) method. The max value of TBC input parameter can be obtained from the height of search range.

  - both width and height are encoded using TBC method. The max value of TBC input parameter can be obtained from the current location, reference location and width/height of search range.

- If 2D_flag = 0, a modified version of the current index map coding method is applied to the quantized pixels where the COPY_ABOVE flag is encoded using only one context.

- A frequent pixel table is created and used in this proposal. When coding pixels in non-copy modes, instead of writing its RGB value to bit-stream, its index in the frequent table is coded in the bit-stream. In addition, color_found_flag is also defined and added in the syntax.

  - color_found_flag is set to 0 to indicate that the pixel value is not found in the frequent pixel table. This flag along with the RGB value are written to bit-stream, and this RGB value is added to the frequent pixel table.

  - color_found_flag is set to 1 to indicate that the pixel value is found in the frequent pixel table. This flag and the frequent pixel table index are written to bit-stream.

# Simulations

Experiments are performed on top of the SCM 3.0 with the common test condition [2]. In addition, we also performed tests using various restricted search areas (e.g. 1x4 CTUs) for intra block copy (IBC). Results for lossy and lossless are summarized as follows.

Table 1 : Lossy coding efficiency (Anchor SCM3.0 IBC 1x4)

|  | IBC 1x4/2D 1x4 |  |  | IBC 2x4/2D 2x4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | All Intra |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.7% | -0.8% | -0.7% | -6.8% | -6.9% | -6.8% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.2% | -3.3% | -3.3% | -3.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.6% | -0.7% | -0.6% | -7.4% | -7.3% | -7.2% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.5% | -0.3% | -3.4% | -3.3% | -3.3% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 162.0% |  |  | 194% |  |  |
| Dec Time[%] | 115.0% |  |  | 110% |  |  |
|  | Random Access |  |  | Random Access |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.5% | -4.4% | -4.4% | -4.4% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.1% | -2.1% | -2.1% | -2.0% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -0.6% | -0.7% | -4.5% | -4.6% | -4.5% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.5% | -0.4% | -2.2% | -2.2% | -2.3% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.0% | 0.1% | -0.1% | 0.1% |
| Enc Time[%] | 117.0% |  |  | 125% |  |  |
| Dec Time[%] | 105.0% |  |  | 104% |  |  |
|  | Low delay B |  |  | Low delay B |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.2% | -0.2% | -2.8% | -2.6% | -2.5% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% | -0.9% | -0.8% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | 0.1% | -2.6% | -2.5% | -2.2% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.8% | -0.6% | -1.2% | -1.5% | -1.3% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | -0.1% | -0.1% | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 115.0% |  |  | 120% |  |  |
| Dec Time[%] | 109.0% |  |  | 108% |  |  |

Table 2 : Lossy coding efficiency (Anchor SCM3.0 IBC 1x4)

|  | IBC 3x5/2D 3x5 |  |  | IBC FF/2D 1x4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | All Intra |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -9.0% | -9.0% | -8.9% | -11.6% | -11.5% | -11.5% |
| RGB, mixed content, 1440p & 1080p | -4.6% | -4.5% | -4.4% | -6.8% | -6.6% | -6.7% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -9.7% | -9.5% | -9.4% | -12.4% | -12.2% | -12.1% |
| YUV, mixed content, 1440p & 1080p | -4.8% | -4.6% | -4.6% | -6.8% | -6.7% | -6.8% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 248% |  |  | 143% |  |  |
| Dec Time[%] | 106% |  |  | 112% |  |  |
|  | Random Access |  |  | Random Access |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -6.0% | -5.9% | -5.9% | -7.1% | -6.9% | -6.9% |
| RGB, mixed content, 1440p & 1080p | -2.9% | -2.7% | -2.7% | -4.2% | -3.9% | -4.0% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -6.1% | -6.0% | -5.9% | -7.3% | -7.1% | -7.1% |
| YUV, mixed content, 1440p & 1080p | -3.1% | -3.1% | -3.0% | -4.4% | -4.5% | -4.6% |
| YUV, Animation, 720p | 0.1% | 0.2% | 0.0% | 0.1% | 0.2% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 135% |  |  | 116% |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |
|  | Low delay B |  |  | Low delay B |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.9% | -3.7% | -3.6% | -4.0% | -3.7% | -3.8% |
| RGB, mixed content, 1440p & 1080p | -1.1% | -0.8% | -0.8% | -1.9% | -1.6% | -1.7% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -3.6% | -3.5% | -3.5% | -3.8% | -3.8% | -3.4% |
| YUV, mixed content, 1440p & 1080p | -1.4% | -1.1% | -1.2% | -2.3% | -2.4% | -2.3% |
| YUV, Animation, 720p | 0.1% | 0.2% | 0.3% | 0.2% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 128% |  |  | 113% |  |  |
| Dec Time[%] | 107% |  |  | 107% |  |  |

Table 3 : Lossless coding efficiency (Anchor SCM3.0 IBC 1x4)

|  | IBC 1x4/2D 1x4 |  |  |  | IBC 2x4/2D 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  | All Intra |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change | Bit-rate change (Max) |
|  |  |  | (Min) |  |  |  | (Min) |  |
| RGB, text & graphics with motion, 1080p & 720p | -3.3% | -3.4% | -8.1% | -0.1% | -6.0% | -6.7% | -14.2% | -0.5% |
| RGB, mixed content, 1440p & 1080p | -0.5% | -0.5% | -0.8% | -0.2% | -1.7% | -1.7% | -2.0% | -1.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -3.1% | -3.2% | -8.3% | -0.1% | -6.3% | -6.8% | -14.6% | -0.6% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.5% | -0.8% | -0.1% | -1.7% | -1.8% | -2.0% | -1.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 167.0% |  |  |  | 209% |  |  |  |
| Dec Time[%] | 104.0% |  |  |  | 103% |  |  |  |
|  | Random Access |  |  |  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change | Bit-rate change (Max) |
|  |  |  | (Min) |  |  |  | (Min) |  |
| RGB, text & graphics with motion, 1080p & 720p | -1.6% | -1.9% | -5.1% | -0.1% | -5.2% | -4.5% | -11.1% | -0.3% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | -0.1% | -0.3% | -0.3% | -0.5% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.4% | -1.8% | -5.1% | -0.1% | -5.2% | -4.5% | -11.2% | -0.3% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% | -0.3% | -0.3% | -0.5% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 116.0% |  |  |  | 128% |  |  |  |
| Dec Time[%] | 105.0% |  |  |  | 104% |  |  |  |
|  | Low Delay B |  |  |  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change | Bit-rate change (Max) |
|  |  |  | (Min) |  |  |  | (Min) |  |
| RGB, text & graphics with motion, 1080p & 720p | -1.6% | -1.3% | -3.6% | 0.0% | -5.3% | -3.7% | -10.8% | -0.2% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | -0.2% | -0.4% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.3% | -1.2% | -3.2% | -0.1% | -5.2% | -3.6% | -10.1% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.4% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 112.0% |  |  |  | 121% |  |  |  |
| Dec Time[%] | 105.0% |  |  |  | 104% |  |  |  |

Table 4 : Lossless coding efficiency (Anchor SCM3.0 IBC 1x4)

|  | IBC 3x5/2D 3x5 |  |  |  | IBC FF/2D 1x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  | All Intra |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change | Bit-rate change (Max) |
|  |  |  | (Min) |  |  |  | (Min) |  |
| RGB, text & graphics with motion, 1080p & 720p | -7.0% | -7.8% | -16.6% | -0.7% | -8.2% | -9.4% | -25.5% | -0.6% |
| RGB, mixed content, 1440p & 1080p | -2.1% | -2.1% | -2.6% | -1.6% | -3.2% | -3.2% | -3.3% | -3.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -7.3% | -8.0% | -17.0% | -0.8% | -8.6% | -9.6% | -25.4% | -0.8% |
| YUV, mixed content, 1440p & 1080p | -2.3% | -2.3% | -2.7% | -1.6% | -3.4% | -3.4% | -3.6% | -3.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 271% |  |  |  | 145% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |
|  | Random Access |  |  |  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -6.3% | -5.5% | -13.4% | -0.3% | -7.1% | -6.3% | -17.1% | -0.3% |
| RGB, mixed content, 1440p & 1080p | -0.4% | -0.4% | -0.6% | -0.2% | -0.6% | -0.6% | -1.0% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -6.3% | -5.6% | -13.5% | -0.4% | -7.2% | -6.4% | -16.9% | -0.4% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.4% | -0.6% | -0.2% | -0.6% | -0.6% | -1.0% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 142% |  |  |  | 112% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 101% |  |  |  |
|  | Low Delay B |  |  |  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change | Bit-rate change (Max) | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change | Bit-rate change (Max) |
|  |  |  | (Min) |  |  |  | (Min) |  |
| RGB, text & graphics with motion, 1080p & 720p | -6.4% | -4.6% | -13.1% | -0.2% | -7.2% | -4.7% | -15.1% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.5% | -0.1% | -0.3% | -0.3% | -0.7% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -6.3% | -4.5% | -12.4% | -0.3% | -7.2% | -4.7% | -14.4% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.5% | -0.1% | -0.3% | -0.3% | -0.7% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 132% |  |  |  | 108% |  |  |  |
| Dec Time[%] | 103% |  |  |  | 101% |  |  |  |

The lossy result of removing residual coding is also attached.

Table 5 : Lossy coding efficiency (anchor IBC 1x4, left: residual coding, right: without residual coding)

|  | All Intra |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.7% | -0.8% | -0.8% | -0.7% | -0.8% | -0.8% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.3% | -0.2% | -0.2% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.7% | -0.8% | -0.9% | -0.6% | -0.8% | -0.7% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.5% | -0.2% | -0.2% | -0.5% | -0.3% |
| YUV, Animation, 720p | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 184% |  |  | 190% |  |  |
| Dec Time[%] | 107% |  |  | 106% |  |  |
|  | Random Access |  |  | Random Access |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.6% | -0.6% | -0.8% | -0.6% | -0.8% | -0.7% |
| RGB, mixed content, 1440p & 1080p | -0.4% | -0.5% | -0.5% | -0.5% | -0.3% | -0.5% |
| RGB, Animation, 720p | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.1% | 0.2% | 0.3% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -0.8% | -0.7% | -0.5% | -0.7% | -0.8% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.6% | -0.4% | 0.0% | -0.5% | -0.5% |
| YUV, Animation, 720p | 0.2% | 0.1% | -0.2% | 0.1% | -0.1% | -0.6% |
| YUV, camera captured, 1080p | 0.2% | 0.0% | 0.4% | 0.1% | -0.1% | 0.3% |
| Enc Time[%] | 121% |  |  | 120% |  |  |
| Dec Time[%] | 106% |  |  | 100% |  |  |
|  | Low delay B |  |  | Low delay B |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.4% | -0.2% | -0.4% | -0.4% | -0.3% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.2% | 0.2% | -0.2% | 0.3% | 0.3% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -0.5% | -0.4% | -0.5% | -0.5% | -0.6% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.3% | -0.4% | -0.6% | -0.5% |
| YUV, Animation, 720p | 0.2% | 0.8% | 0.1% | 0.2% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.1% | 0.1% | 0.3% | 0.1% |
| Enc Time[%] | 117% |  |  | 116% |  |  |
| Dec Time[%] | 110% |  |  | 102% |  |  |

Table 6 : Lossy coding efficiency (anchor IBC FF, left: residual coding, right: without residual coding)

|  | All Intra |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.4% | -0.6% | -0.5% | -0.3% | -0.6% | -0.4% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.1% | -0.2% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.6% | -0.4% | -0.6% | -0.8% | -0.5% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.4% | -0.3% | -0.2% | -0.4% | -0.3% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 201% |  |  | 192% |  |  |
| Dec Time[%] | 111% |  |  | 107% |  |  |
|  | Random Access |  |  | Random Access |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | #VALUE! | #VALUE! | #VALUE! | -0.4% | -0.6% | -0.6% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.2% | -0.2% | 0.1% | -0.2% | -0.3% |
| RGB, Animation, 720p | 0.1% | 0.5% | 0.1% | 0.1% | 0.4% | 0.1% |
| RGB, camera captured, 1080p | 0.2% | 0.1% | 0.2% | 0.1% | 0.3% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.5% | -0.6% | -0.4% | -0.6% | -0.5% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.4% | -0.5% | -0.5% | -0.8% | -0.6% |
| YUV, Animation, 720p | 0.2% | -0.7% | 0.1% | 0.0% | -0.8% | 0.2% |
| YUV, camera captured, 1080p | 0.1% | -0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| Enc Time[%] | #VALUE! |  |  | 129% |  |  |
| Dec Time[%] | #VALUE! |  |  | 104% |  |  |
|  | Low delay B |  |  | Low delay B |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | 0.1% | -0.2% | -0.2% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.2% | 0.1% | -0.2% | 0.3% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% | 0.0% | 0.3% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | #VALUE! | #VALUE! | #VALUE! | -0.5% | -0.7% | -0.6% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.9% | 0.0% | -0.2% | -0.9% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.4% | -0.1% | 0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.2% | 0.0% | 0.4% | -0.2% |
| Enc Time[%] | #VALUE! |  |  | #VALUE! |  |  |
| Dec Time[%] | #VALUE! |  |  | #VALUE! |  |  |

# Recommendation

It is recommended to further study the hybrid 1-D and 2-D intra string copy in potential CE.

# Reference

[1] W. Wang, Z. Ma, M. Xu, H. Yu, “Non-CE6: 2-D Index Map Coding of Palette Mode in HEVC SCC”, Doc. JCT-VC S0151, Strasbourg, FR, Oct. 2014.

[2] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”, Doc. JCT-VC S1015, Strasbourg, FR, Oct. 2014.

# Patent rights declaration(s)

Huawei R&D USA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).